Abstract

Converting Tissue “Engineering” into an Engineering Discipline with 3D-Printable Bioreactors, Arduino-Based Sensors, and 3D Printers with Submicron Precision

Adrien Hosking, Jonah Melegrito, Princess Akande, Nicole Bueno, Guochang (Nick) Ye, Timofey Broslav, Thomas Ward, Kristen Brenner, Soha Patil, Alexis Hopkins, and Dr. Jim Brenner, Florida Tech, Biological and Chemical Engineering and Sciences, Nanotechnology Minor Program Chair, jbrenner@fit.edu, 321-749-3437

Tissue “engineering” has not developed quickly for the following reasons:  a) a lack of automation, meaning that students focus mostly on keeping cells alive rather than growing fully developed tissues capable of being transplanted; b) the time from a) prevents students from making enough samples to get statistically significant trends; c) taking measurements traditionally has meant that one takes the tissue out of incubation to see it, by which times cells are dying; d) a lack of "patient monitors" to assess pH, glucose concentration, lactate concentration, dissolved oxygen concentration, temperature, liquid level, conductivity (salinity), cell mass, and in vivo imaging inside an incubated volume; e) insufficient mathematical modeling and experimental validation of bioreactors; and f) the fiber diameter in 3D-printed objects is > 30 m, whereas the fibers inside the body are typically 1-3 mm and cells range from 5-30 m.  This inconsistency in fiber diameter affects cellular adhesion biomechanics, which affects the cells’ ability to form tight junctions to form a leak-tight tissue, which in turn affects fluid transport and ultimately results in necrosis due to lack of oxygen and/or nutrients.

We are constructing a tissue engineering test bed with an array of 36 bioreactors in parallel, with all of the sensors listed in d) being Arduino-based and most being purchased from Open Aquarium and Atlas Scientific.  Some of the concentration sensors are electrochemically-based and compatible with a CheapStat potentiostat.  The primary competition is a group at Vanderbilt running eight bioreactors using LabSmith microfluidics hardware toward a simulation of entire organs-on-a-chip with automated feeding and with in vivo imaging, but without all of the necessary instrumentation to do proper mass and energy balance accounting and with some side effects, often toxicity, from the dyes used for cell staining.  In parallel with this project, a 3D printer is being developed to systematically dose suspensions of collagen, Bioglass, crosslinking agents, and cell-laden hydrogels in controlled ratios.  After mounting the resulting cell-laden scaffolds onto glass slides inside a custom-designed 3D-printable bioreactor, the cell-laden scaffolds will be dosed with glucose, dyes, etc. for maintenance, accounting, and imaging of tissues of the scaffolds.  Not only will we have the ability to mass produce candidate tissues at a fast enough rate to obtain statistically valid trends, by automating the in vivo imaging and also developing a set of less harmful, reversible dyes, we will minimize the mundane, manual aspects of tissue culturing.  Data acquisition and control will use a combination of Arduinos and National Instruments (NI) hardware that are ultimately displayed in NI LabView.

Table of Contents

Abstract	1

Table of Contents	3

List of Figures	xx

List of Tables	xx

Notation	xx

Chapter 1 – Motivation/Introduction	26

1.1 - Microfluidic Approaches to Tissue Engineering Test Beds	26

1.2 - Vanderbilt/LabSmith Inspiration	32

1.3 - Improvements to Existing Ted Bed Designs	xx

1.4 - Arduinos Lowering Sensor Cost	xx

1.5 - Literature Analysis	xx

1.6 - Problem Identification	xx

1.7 - Big Picture Task Plan	xx

Chapter 2 – Tissue Engineering Test Bed Design	xx

2.1 - Questions and Issues Sheet Summary	xx

2.2 - Bioreactor Design Considerations	xx

2.3 - Imaging Design Considerations	xx

2.4 - Mass and Energy Balances	xx

2.5 - Instrumentation Specifications	xx

2.5.1 - Within Bioreactors	xx

2.5.1.1 - Temperature	xx

2.5.1.2 - Liquid Level	xx

2.5.1.3 - pH	xx

2.5.1.4 - Electrical Conductivity	xx

2.5.1.5 - Dissolved Oxygen	xx

2.5.1.6 - Oxidation-Reduction Potential	xx

2.5.1.7 - Glucose Concentration	xx

2.5.1.8 - Lactate Concentration	xx

2.5.1.9 - Fiber Optic LED Microsocope	xx

2.5.1.10 - Impeller Rotation Rate	xx

2.5.1.11 - Cell/Scaffold Mass	xx

2.5.2 - External to Bioreactors	xx

2.5.2.1 - Flow Totalizers	xx

2.5.2.2 - Differential Pressure Transducers	xx

2.5.2.3 - Turbidity Sensors	xx

2.5.2.4 - Capsule Filters	xx

2.5.2.4.1 - Filtration Theory and Design Considerations	xx

2.5.2.4.2 - Capsule Filter Specifications	xx

2.5.2.5 - Diaphragm Pumps	xx

2.5.2.6 - Diatomaceous Earth	xx

2.5.2.7 - Glucose	xx

2.5.3 - For Feed Tanks	xx

2.5.3.1 - Diaphragm Pumps and Peristaltic Pumps	xx

2.5.3.2 - Syringe Pumps	xx

2.5.3.3 - Feed Tanks	xx

2.5.4 - Aquarium Level	xx

2.5.4.1 - Colorimetric Imaging of Unwanted Biofilm Growth	xx

2.5.4.2 - UV LED Sterilization	xx

2.5.4.3 - Heaters	xx

2.6 - Bioreactor Process Flowsheet	xx

2.7 - Piping and Instrumentation Diagram	xx

2.8 – Hazardous Operability (HAZOP) Analysis	xx

2.9 - National Instruments LabView Code	xx

2.10 - National Instruments Data Acquisition System	xx

2.11 - Wireless Data Transmission from Sensors	xx

2.12 - Other Arduino Code	xx

2.13 - Budget	xx

Chapter 3 – Subsystems Fabrication and Validation	xx

3.1 - Valve Actuation System Assembly and Validation	xx

3.2 - Syringe Pump System Assembly and Validation	xx

3.3 - Sensor Subsystem Assembly	xx

3.4 - Individual Sensor Validation	xx

3.4.1 - Temperature	xx

3.4.2 - Liquid Level	xx

3.4.3 - pH	xx

3.4.4 - Electrical Conductivity	xx

3.4.5 - Dissolved Oxygen	xx

3.4.6 - Oxidation-Reduction Potential	xx

3.4.7 - Glucose Concentration	xx

3.4.8 - Lactate Concentration	xx

3.4.9 - Impeller Rotation Rate	xx

3.4.10 - Flow Totalizers	xx

3.4.11 - Syringe Pumps	xx

3.4.12 - Colorimetric Imaging of Unwanted Biofilm Growth	xx

3.4.13 - Cell/Scaffold Mass	xx

3.5 - Imaging	xx

3.6 - Mass Balance Closure	xx

3.7 - Energy Balance Closure	xx

3.8 - Validation of Safety Responses to Expected Step Disturbances	xx

3.9 - Validation of Safety Responses to Emergency Situations	xx

Chapter 4 – Evaluation of Tissue Engineering Test Bed	xx

4.1 - Dye Tracer Testing With “Blank” Reactor	xx

4.2 - Glucose Tracer Testing With “Blank” Reactor 	xx

4.3 - Dye Tracer Testing of Bioreactor Only	xx

4.4 - Glucose Tracer Testing of Bioreactor Only	xx

4.5 - Dye Tracer Testing of Full System	xx

4.6 - Glucose Tracer Testing of Full System	xx

Chapter 5 – Conclusions and Future Work	xx

Appendixes

Appendix 2.1 -  Questions and Issues Sheet	xx

Appendix 2.4 – Complete List of Mass and Energy Balances	xx

Appendix A2.5 – Instrumentation Specifications	xx

Appendix A2.5.1 - Within Bioreactors 	xx

Appendix A2.5.1.1 - Temperature sensor for inside reactors 	xx

Appendix A2.5.1.2 - Liquid level measured with 5 kg load cell 	xx

Appendix A2.5.1.3 - pH 	xx

Appendix A2.5.1.3.1 - pH Sensor and Shield 	xx

Appendix A2.5.1.3.2 - pH/ORP Electrode Storage Solution 	xx

Appendix A2.5.1.3.3 - pH = 4, 7, and 10 Calibration Solutions 	xx

Appendix A2.5.1.4 - Electrical Conductivity	xx

Appendix A2.5.1.4.1 - Generic Electrical Conductivity Sensor 	xx

Appendix A2.5.1.4.2 - Vernier Electrical Conductivity Probe 	xx

Appendix A2.5.1.4.3 - 10x Phosphate Buffered Saline 	xx

Appendix A2.5.1.4.4 - Electrical Conductivity Standards Pack #1 	xx

Appendix A2.5.1.4.5 - Electrical Conductivity Standards Pack #2 	xx

Appendix A2.5.1.4.6 - Vernier LabQuest 2 	xx

Appendix A2.5.1.5 - Dissolved Oxygen Sensors 	xx

Appendix A2.5.1.6 - ORP 	xx

Appendix A2.5.1.7 - Glucose concentration & potentiostat	xx

Appendix A2.5.1.7.1 - Glucose concentration 	xx

Appendix A2.5.1.7.2 - IORodeo Multiplexed RodeoStat Potentiostat 	xx

Appendix A2.5.1.7.3 - Ten pin card edge connector for three electrode electrochemical sensors 	xx

Appendix A2.5.1.7.4 - Twenty pin card edge connector 	xx

Appendix A2.5.1.7.5 - Twenty pin ribbon cable sets 	xx

Appendix A2.5.1.7.6 - Potassium Ferricyanide for Potentiostat Calibration 	xx

Appendix A2.5.1.8 - Lactate concentration 	xx

Appendix A2.5.1.9 - LED microscope camera + holder 	xx

Appendix A2.5.1.9.1 - Generic LED IP68 Endoscope 	xx

Appendix A2.5.1.9.2 - Andonstar microscope 	xx

Appendix A2.5.1.9.3 - Vernier SpectroVis Plus Spectrophotometer 	xx

Appendix A2.5.1.9.4 - Plastic Holder for Two Glass Slides 	xx

Appendix A2.5.1.9.5 - Microscope Stand	xx

Appendix A2.5.1.10 - Overhead mixer for reactor and impeller speed control 	xx

Appendix A2.5.1.10.1 - Overhead mixer 	xx

Appendix A2.5.1.10.2 - Rotating Teflon shaft impeller for each bioreactor 	xx

Appendix A2.5.1.10.3 - Chuck Collet for Stirrers 	xx

Appendix A2.5.1.10.4 - Tachometer for stirrers 	xx

Appendix A2.5.1.10.5 - Teflon adapters for rotating shafts 	xx

Appendix A2.5.1.11.1 - 1 kg load cell for measuring cell mass + scaffold 	xx

Appendix A2.5.1.11.2 - Vibration Isolation 	xx

Appendix A2.5.1.11.3 - Vibration Sensor 	xx

Appendix A2.5.1.12 - Miscellaneous Items for Bioreactor 	xx

Appendix A2.5.1.12.1 - Cereal container used for bioreactor 	xx

Appendix A2.5.1.12.2 - 10 K Ohm Potentiometers 	xx

Appendix A2.5.1.12.3 - Waterproof USB Bulkhead Connector 	xx

Appendix A2.5.1.12.4 - BNC adapters to breadboard 	xx

Appendix A2.5.2 - Bioreactor External Recirculation Loop 	xx

Appendix A2.5.2.1 - Flow Totalizers	xx

Appendix A2.5.2.1.1 - 1/2" NPT Hall Effect Flow Totalizer 	xx

Appendix A2.5.2.1.2 - 1/4" NPT Hall Sensor-Based Flowmeter 	xx

Appendix A2.5.2.2 – Differential Pressure Transducer 	xx

Appendix A2.5.2.3 - Turbidity Sensor 	xx

Appendix A2.5.2.4.2 - Five Micron Capsule Filter 	xx

Appendix A2.5.2.5 - Diaphragm Pumps 	xx

Appendix A2.5.2.6 - Diatomaceous Earth 	xx

Appendix A2.5.2.7 - Glucose 	xx

Appendix A2.5.2.8 - Polyethylene Glycol (PEG) 	xx

Appendix A2.5.2.9 - Protein Adhesion 	xx

Appendix A2.5.3 - Feed Tanks 	xx

Appendix A2.5.3.1 - Diaphragm Pumps 	xx

Appendix A2.5.3.1.1 - Better Diaphragm Pump 	xx

Appendix A2.5.3.1.2 - Alternate Diaphragm Pump 	xx

Appendix A2.5.3.1.3 - Peristaltic pump 	xx

Appendix A2.5.3.2 - Syringe Pumps 	xx

Appendix A2.5.3.2.1 - Eight-Channel Kloehn Syringe Pumps 	xx

Appendix A2.5.3.2.2 - Newer Kloehn Syringe Pump With Multi-Channel Input 	xx

Appendix A2.5.3.3 - Feed Tanks 	xx

Appendix A2.5.3.4 - 1/16" PEEK Fittings 	xx

Appendix A2.5.3.5 - 1/16" PEEK Tubing	xx

Appendix A2.5.3.6 - Manually Operated Valves for Shutoff Around Feed Tanks xx

Appendix A2.5.4 - Aquarium Level 	xx

Appendix A2.5.4.1 - Colorimetric Imaging of Unwanted Biofilm Growth 	xx

Appendix A2.5.4.2 - UV LED's and UV LED Detectors 	xx

Appendix A2.5.4.2.1 - 470-475 nm LED Strip 	xx

Appendix A2.5.4.2.2 - 265 nm LED dimensions 	xx

Appendix A2.5.4.2.3 - Aluminum Printed Circuit Board for UVC LED's 	xx

Appendix A2.5.4.2.4 - 365-370 and 400-410 nm LED's 	xx

Appendix A2.5.4.2.5 - 270-285 nm UVC LED assembly diagram 	xx

Appendix A2.5.4.2.6 - 370 nm UV LED 	xx

Appendix A2.5.4.2.7 - 475 nm LED's 	xx

Appendix A2.5.4.2.8 - 253.7 nm UV Lamp 	xx

Appendix A2.5.4.2.9 - 365-370 nm UV LED + PCB Board 	xx

Appendix A2.5.4.2.10 - 253.7 nm AlGaN UV Detector 	xx

Appendix A2.5.4.2.11 - 365-370 nm UV LED + PCB Board 	xx

Appendix A2.5.4.2.12 - 440 nm LED strip 	xx

Appendix A2.5.4.2.13 - 440-450 nm LED 	xx

Appendix A2.5.4.2.14 - Power Supplies for UV LED's 	xx

Appendix A2.5.4.2.15 - 200-370 nm UV Detector 	xx

Appendix A2.5.4.3 - Temperature and Heat-Related Sensors/Controls 	xx

Appendix A2.5.4.3.1 - Aquarium Heaters 	xx

Appendix A2.5.4.3.2 - Temperature Sensor for Entire Aquarium 	xx

Appendix A2.5.4.3.3 - Temperature and Humidity of Vapor Space  	xx

Appendix A2.5.4.4 - Load cell for entire aquarium 	xx

Appendix A2.5.4.5 - Non-Temperature Environmental Monitoring 	xx

Appendix A2.5.4.5.1 - CO2 concentration for entire headspace 	xx

Appendix A2.5.4.5.2 - OpenDrop for Concentration Monitoring Comparison 	xx

Appendix A2.5.4.5.3 - Gas phase particle size analysis 	xx

Appendix A2.5.4.5.4 - Barometric Pressure Sensors 	xx

Appendix A2.5.4.6 - Flow totalizers 	xx

Appendix A2.5.4.6.1 - Flow totalizer for entire aquarium 	xx

Appendix A2.5.4.6.2 - 0.15 - 1.50 L/min Flowmeter 	xx

Appendix A2.5.4.6.3 - Stirrer for entire aquarium 	xx

Appendix A2.5.4.6.4 - Tachometer for stirrers	xx

Appendix A2.5.4.7 - Arduino Data Acquisition Equipment 	xx

Appendix A2.5.4.7.1 - Arduino Uno 	xx

Appendix A2.5.4.7.2 - Arduino ESP8266 WiFi Card 	xx

Appendix A2.5.4.7.3 - Multiplexers 	xx

Appendix A2.5.4.7.4 - Arduino Unos with Breadboards 	xx

Appendix A2.5.4.7.5 - Electrical signal isolators for breadboards 	xx

Appendix A2.5.4.7.6 - BNC adapters to breadboard 	xx

Appendix A2.5.4.7.7 - Female USB to Breadboard Adapter for Endoscopes 	xx

Appendix A2.5.4.7.8 - Solderless Breadboard: 1560 Tie Point, 1 Bus Strip 	xx

Appendix A2.5.4.7.9 - Arduino Kit 	xx

Appendix A2.5.4.7.10 - Serial to USB Panel Mount Connector 	xx

Appendix A2.5.4.7.11 - 12 V, 2 A DC adapters for Arduino Uno 	xx

Appendix A2.5.4.8 - Electrical Feedthroughs and Enclosures 	xx

Appendix A2.5.4.8.1 - Waterproof Cable Gland Connectors 	xx

Appendix A2.5.4.8.2 - Waterproof USB Bulkhead Connector 	xx

Appendix A2.5.4.8.3 - LeMoTech Project Box Enclosure 	xx

Appendix A2.5.4.9 - Valves and Aeration Supplies 	xx

Appendix A2.5.4.9.1 - Electrically Actuated Shutoff Valve 	xx

Appendix A2.5.4.9.2 - Manually Operated Valves Inside Aquarium 	xx

Appendix A2.5.4.9.3 - Large Shutoff Valves for Aquarium 	xx

Appendix A2.5.4.9.4 - Manifolds Inside Aquarium 	xx

Appendix A2.5.4.9.5 - Bubble Disk Aerator 	xx

Appendix A2.5.4.9.6 - Suction clips for mounting instruments and plumbing 	xx

Appendix A2.5.4.10 - Assorted Hose Barb Fittings	xx

Appendix A2.5.4.11 - Tubing and O-Rings	xx

Appendix A2.5.4.11.1 - Silicone Tubing 	xx

Appendix A2.5.4.11.2 - Tygon Tubing	xx

Appendix A2.5.4.11.3 - O-Rings	xx

Appendix A2.7 - Piping & Instrumentation Diagrams	xx

Appendix A2.7.3 - Feed Distribution System	xx

Appendix A2.7.4 - Waste Collection System	xx

Appendix A2.8 - Hazard and Operability Analysis	xx

Appendix A2.9 - LabView VI's	xx

Appendix A2.9.1 – LabView VI’s from Prior Brenner Projects	xx

Appendix A2.9.2 – VI’s for Tissue Engineering Test Bed	xx

Appendix A2.11 – Equipment List & Budget		xx

Appendix A3.1.1 - Instrumentation for 10-Way Valve Automation 	xx

## Appendix A3.1.2 - National Instruments SC-2054	xx

Appendix A3.1.3 - National Instruments R1005050 Ribbon Cable 182762-02	xx

Appendix A3.1.4 - 26 Wire Rainbow Ribbon Cable	xx

Appendix A3.1.5 - National Instruments PCI-DIO-96 96 signal digital input/output card 		xx

Appendix A3.2.1 - Valco VICI HPLC 8 Position Valve Controller	xx

Appendix A3.2.2 - Stainless Steel Ferrules for Multiposition Valves	xx

List of Figures	11

Figure 1.2.1: LabSmith basic microfluidic kit 	33

Figure 1.2.2: Vanderbilt microscope setup  	34

Figure 1.2.3: Vanderbilt process flow diagram 	35

Figure 1.2.4: Vanderbilt valve and pump schematic 	36

Figure 1.3.1: VICI 10-way valves	38

Figure 1.3.2: Kloehn eight channel syringe pumps	39

Figure 1.4.1: Libelium’s Open Aquarium inspiration for making inexpensive, fully instrumented bioreactors includes Arduino-based control of temperature and liquid level, as well as sensing of pH and of solution electrical conductivity (i.e. salinity) 		39

Figure 2.2.1:  Bioreactor instrumentation and controls............................................	39

Figure 2.5.2.1.1: Hall effect water flow sensor used for flowrate monitoring	39

Figure 2.5.2.2.1: Differential pressure transducer 	39

Figure 2.5.2.3.1 – Turbidity sensor used for turbidity monitoring 	39

Figure 2.5.2.4.2.1 – 5-micron capsule filter 	39

Figure 2.5.2.5.1  – Diaphragm pump 	39

Figure 2.6.1 – Bioreactor process flow diagram	39

Figure 2.7.2.1: P&ID of bioreactor external recirculation loop	76

Figure 2.7.3.1: Oxygen add-in system P&ID	80

Figure 2.7.3.2: Flowsheet of feedstocks via syringe pump reservoirs	82

Figure 2.7.3.3: P&ID connecting feedstocks to common output	82

Figure 2.7.4.1: Flowsheet of output streams flowing through common filtration system	86

Figure 2.7.4.2: P&ID of purification system.	86

Figure 3.1.1: Wiring diagram for control of 10-way valves responsible for feed distribution to bioreactors 1-36 	xx

Figure 3.1.2: Complete wiring connection from 10-way valve to NI MAX software 	xx

Figure 3.1.3: Complete wiring connection from 10-way valve to NI MAX software showing communication issue between NI SC-2054 and VICI 10-way valve

Figure 3.3.1 – Experimental setup for filtration testing	127

Figure 3.3.1.1 – Turbidity vs. particle concentration curve (polynomial)	129

Figure 3.3.1.2 – Turbidity vs. particle concentration curve (log-log)	130

Figure 3.3.1.3 – Turbidity validation test	131

Figure 3.3.1.4 – Differential pressure validation test	131

Figure 3.3.1.5 – Flowmeter validation test	132

Figure 3.3.1.6 – Differential pressure and flow rate test	132

Figure 3.3.2.1 – Filter reusability test with turbidity	133

Figure 3.3.2.2 – Filter reusability test with differential pressure	134

Figure 3.3.2.3 – Upper control limits (UCL’s)	135

Figure 3.3.2.4 – Differential pressure change based on flow rate	136

Figure 3.3.2.5 – Total resistance change based on flow rate	136

Figure 3.3.3.1 – Experimental data vs. theoretical model	137

Figure 3.3.4.1 – Moving average of flow rate validation test (run 1), n = 25	139

Figure 4.1: Pulse injection tracer study schematic 	143

Figure 4.2: Theoretical expectation of tracer study result for a non-ideal reactor 	145

Figure 4.3: E(t) vs. t integration schematic showing bounds from 0-3 minutes 	146

Figure 4.4: Cumulative time distribution schematic 	148

Figure 4.5: Effects of dispersion on effluent tracer concentration 	149

Figure 4.1.1: Bioreactor tracer testing apparatus	153

Figure 4.1.2: Power connection of bioreactor testing apparatus	154

Figure 4.1.3:  Molecular structure of PEG	157

Figure 4.1.4: Ct curves for tracer dye studies, trials 1-5, conducted prior to coating the reactor system with PEG	160

Figure 4.1.5: Et tracer dye studies, trials 1-5, conducted prior to coating the reactor system with PEG	161

Figure 4.1.6: Ft curves for tracer dye studies, trials 1-5, conducted prior to coating the reactor system with PEG	162

Figure 4.1.7: Ct curves for tracer glucose studies, trials 1-5, conducted prior to coating the reactor system with PEG	164

Figure 4.1.8: Et curves for tracer glucose studies trials 1-5 conducted prior to coating the reactor system with PEG	165

Figure 4.1.9: Ft curves for tracer glucose studies, trials 1-5, conducted prior to coating the reactor system with PEG	166

Figure 4.1.10: Ct curves for tracer dye studies, trials 1-5, conducted after coating the reactor system with PEG 	168

Figure 4.1.11: Et for tracer dye studies trials, 1-5, conducted after coating the reactor system with PEG	169

Figure 4.1.12: Ft curves for tracer dye studies, trials 1-5, conducted after coating the reactor system with PEG	170

Figure 4.1.13: Ct curves for tracer glucose studies, trials 1-5, conducted after coating the reactor system with PEG 	172

Figure 4.1.14: Et curves for tracer glucose studies trials, 1-5, conducted after coating the reactor system with PEG	173

Figure 4.1.15: Ft curves for tracer glucose studies trials, 1-5, conducted after coating the reactor system with PEG	174

Appendixes

Figure A2.5.1.1.1 - Temperature sensor	xx

Figure A2.5.1.1.2 - Wiring diagram for temperature sensor	xx

Figure A2.5.1.2.1 - Load cell sensor used to measure reactor mass	xx

Figure A2.5.1.2.2 - Wiring diagram for load cell sensor 	xx

Figure A2.5.1.2.3 - Load cell wiring	xx

Figure A2.5.1.2.4 - Dimensions of load cell sensor	xx

Figure A2.5.1.3.1.1 - pH sensor and shield	xx

Figure A2.5.1.3.2.1 - pH/ORP electrode storage solution	xx

Figure A2.5.1.3.3.1 - pH = 4, 7, and 10 calibration solutions	xx

Figure A2.5.1.4.1.1 - Electrical conductivity sensor and shield	xx

Figure A2.5.1.4.1.2 - Wiring diagram for electrical conductivity sensor 	xx

Figure A2.5.1.4.2.1 - Vernier electrical conductivity probe	xx

Figure A2.5.1.4.3.1 - 10x phosphate buffered saline (PBS)	xx

Figure A2.5.1.4.4.1 - Electrical conductivity standards pack #1	xx

Figure A2.5.1.4.5.1 - Electrical conductivity standards pack #2	xx

Figure A2.5.1.4.6.1 - Vernier LabQuest 2	xx

Figure A2.5.1.4.6.2 - Vernier LabQuest 2 front panel	xx

Figure A2.5.1.4.6.3 - Vernier LabQuest 2 ports I	xx

Figure A2.5.1.4.6.4 - Vernier LabQuest 2 ports II	xx

Figure A2.5.1.5a.1 - Dissolved oxygen sensor and shield 	xx

Figure A2.5.1.5b.1 - Vernier dissolved oxygen probe	xx

Figure A2.5.1.6.1 - Oxidation reduction potential (ORP) sensor	xx

Figure A2.5.1.6.2 - Atlas Scientific shield for ORP sensor	xx

Figure A2.5.1.7.1.1 - Pine Instruments three electrode electrochemical sensor	xx

Figure A2.5.1.7.2.1 - Multiplexed RodeoStat potentiostat	xx

Figure A2.5.1.7.3.1 - Ten pin card edge connector 	xx

Figure A2.5.1.7.4.1 - Twenty pin card edge connector	xx

Figure A2.5.1.7.5.1 - Twenty pin ribbon cable sets	xx

Figure A2.5.1.7.6.1 - Potassium ferricyanide for potentiostat calibration	xx

Figure A2.5.1.8.1.1 - Pine Instruments three electrode electrochemical sensor	xx

Figure A2.5.1.9.1.1 - Microscope	xx

Figure A2.5.1.9.2.1 - Andonstar microscope	xx

Figure A2.5.1.9.2.2 - Fiber optic LED sensor used for cellular imaging	xx

Figure A2.5.1.9.3.1 - Vernier SpectroVis Plus Spectrophotometer	xx

Figure A2.5.1.9.3.2 - Vernier SpectroVis Plus spectrophotometer wiring 	xx

Figure A2.5.1.9.4.1 - Glass microscope slide holders	xx

Figure A2.5.1.9.5.1 - Endoscope Stand	xx

Figure A2.5.1.10.1.1 - Impeller speed controller	xx

Figure A2.5.1.10.2.1 - Rotating Teflon shaft impeller	xx

Figure A2.5.1.10.3.1 - Chuck collet for stirrers	xx

Figure A2.5.1.10.4.1 - Tachometer for stirrers	xx

Figure A2.5.1.10.5.1 - Teflon adapters for rotating shafts	xx

Figure A2.5.1.11.1.1 - Load cell sensor used to measure cell/scaffold mass	xx

Figure A2.5.1.11.1.2 - Wiring diagram for load cell sensor 	xx

Figure A2.5.1.11.1.3 - Load cell wiring	xx

Figure A2.5.1.11.1.4 - Dimensions of load cell sensor	xx

Figure A2.5.1.11.3.1 - Vibration sensor	xx

Figure A2.5.1.11.3.2 - Vibration Sensor and Shield Dimensions	xx

Figure A2.5.1.12.1.1 - Cereal container used for bioreactor	xx

Figure A2.5.1.12.1.2 - Cereal container used for bioreactor	xx

Figure A2.5.1.12.2.1 - 10K ohm potentiometers	xx

Figure A2.5.1.12.3.1 - Waterproof USB bulkhead connector	xx

Figure A2.5.1.12.4.1 - BNC adapters to breadboard	xx

Figure A2.5.2.1.1.1 - 1/2" NPT Hall Sensor-Based Flowmeter	xx

Figure A2.5.2.1.2.1 - 1/4" NPT Hall Sensor-Based Flowmeter	xx

Figure A2.5.2.1.2.2 – Flowmeter Schematics 	xx

Figure A2.5.2.2.1 - Differential Pressure Transducer	xx

Figure A2.5.2.2.2 – Pressure transducer operating characteristics	xx

Figure A2.5.2.2.3 – Voltage output vs. differential pressure	xx

Figure A2.5.2.2.4 – Pressure transducer schematics	xx

Figure A2.5.2.3.1 - Turbidity Transducer + Shield	xx

Figure A2.5.2.3.2 - Turbidity sensor schematics	xx

Figure A2.5.2.4.2.1 - Five Micron Capsule Filter	xx

Figure A2.5.2.5.1: Diaphragm pump 	xx

Figure A2.5.2.5.2: Diaphragm pump schematic (mm scale) 	xx

Figure A2.5.2.5.3: Diaphragm pump schematic for inlet and outlet 	xx

Figure A2.5.2.5.4: Wiring diagram for diaphragm pump. 	xx

Figure A2.5.3.1.1.1: Diaphragm pump (Diaphragm Pump for Arduino Projects 2019)	xx

Figure A2.5.3.1.1.2: Diaphragm pump schematic (mm scale) (Diaphragm Pump for Arduino Projects 2019)	xx

Figure A2.5.3.1.1.3: Diaphragm pump schematic for inlet and outlet (mm scale) (Diaphragm Pump for Arduino Projects 2019)	xx

Figure A2.5.3.1.1.4: Wiring diagram for diaphragm pump. Alligator clips connect a 12 V power supply to the terminals on the diaphragm pump.	xx

Figure A2.5.3.1.2.1 - Alternate Diaphragm Pump	xx

Figure A2.5.3.1.3.1 - Peristaltic Pump	xx

Figure A2.5.3.2.1.1 - Kloehn V8 Multi-Channel Syringe Pump	xx

Figure A2.5.3.2.1.2 - Kloehn V8 Multi-Channel Syringe Pump Pinout Diagram	xx

Figure A2.5.3.2.2.1 - Newer Kloehn Syringe Pump with Multi-Channel Input	xx

Figure A2.5.3.2.2.2 - Kloehn Syringe Pump with Multi-Channel Input Pinout	xx

Figure A2.5.3.3.1 - Feed Tanks = Whatman Zapcap CR 0.2 m Filters with Shutoff Valves	xx

Figure A2.5.3.4.1 - PEEK Fittings	xx

Figure A2.5.3.6.1 - Manually Operated Valves For Shutoff Around Feed Tanks	xx

Figure A2.5.4.2.1.1 - 470-475 nm LED Strip	xx

Figure A2.5.4.2.2.1 - 265 nm LED	xx

Figure A2.5.4.2.2.2 - 265 nm LED, 0.2-0.3 mW @ 40 mA	xx

Figure A2.5.4.2.2.3 - 265 nm LED dimensions	xx

Figure A2.5.4.2.3.1 - Aluminum Printed Circuit Board for UVC LED's 	xx

Figure A2.5.4.2.4.1 - 365-370 and 400-410 nm LED's	xx

Figure A2.5.4.2.4.2 - 365-370 and 400-410 nm LED's	xx

Figure A2.5.4.2.5.1 - 270-285 nm UVC LED	xx

Figure A2.5.4.2.5.2 - 270-285 nm UVC LED in Bioreactor	xx

Figure A2.5.4.2.5.3 - 270-285 nm UVC LED	xx

Figure A2.5.4.2.5.4 - 270-285 nm UVC LED irradiation of bacteria	xx

Figure A2.5.4.2.5.5 - 270-285 nm UVC LED assembly diagram	xx

Figure A2.5.4.2.6.1 - 370 nm UV LED	xx

Figure A2.5.4.2.7.1 - 475 nm LED's	xx

Figure A2.5.4.2.8.1 - 253.7 nm UV Lamp 	xx

Figure A2.5.4.2.8.2 - 253.7 nm UV lamp dimensions	xx

Figure A2.5.4.2.9.1 - 365-370 nm UV LED + PCB Board	xx

Figure A2.5.4.2.9.2 - 365-370 nm UV LED + PCB Board	xx

Figure A2.5.4.2.10.1 - 253.7 nm AlGaN UV Detector	xx

Figure A2.5.4.2.12.1 - 440 nm LED strip 	xx

Figure A2.5.4.2.13.1 - 440-450 nm LED 	xx

Figure A2.5.4.2.14.1 - Power Supplies for UV LED's	xx

Figure A2.5.4.2.15.1 - 200-370 nm UV Detector	xx

Figure A2.5.4.3.1.1 - Aquarium Heaters	xx

Figure A2.5.4.3.2.1 - Temperature Sensor for Entire Aquarium	xx

Figure A2.5.4.3.2.2 - Wiring Diagram for Temperature Sensor 	xx

Figure A2.5.4.3.3.1 - Temperature and Humidity Sensor 	xx

Figure A2.5.4.4.1 - Load cell sensor	xx

Figure A2.5.4.4.2 - Wiring diagram for load cell sensor 	xx

Figure A2.5.4.4.3 - Load cell wiring	xx

Figure A2.5.4.4.4 - Dimensions of load cell sensor	xx

Figure A2.5.4.5.1.1 - Generic MG811 Carbon Dioxide Gas Sensor and Shield	xx

Figure A2.5.4.5.1.2 - MQ-7 Carbon Dioxide Gas Sensor and Shield	xx

Figure A2.5.4.5.2.1 - Open Drop for Concentration Monitoring	xx

Figure A2.5.4.5.3.1 - Particle Size Analyzer for Gas Sampling Above Aquarium	xx

Figure A2.5.4.5.3.2 - Wiring Diagram for Particle Size Analyzer 	xx

Figure A2.5.4.5.4.1 - Barometric Pressure Sensors With Built-In Shield	xx

Figure A2.5.4.5.4.2 - Barometric Pressure Sensors Wiring Diagram	xx

Figure A2.5.4.6.1.1 - 1/2" NPT Hall Sensor-Based Flowmeter 	xx

Figure A2.5.4.6.1.2 - Flowmeter Shield	xx

Figure A2.5.4.6.2.1 - 0.15 - 1.50 L/min Flowmeter	xx

Figure A2.5.4.6.2.2: Wiring diagram for flowmeters 	xx

Figure A2.5.4.6.3.1 - Teflon Stirrer for Entire Aquarium	xx

Figure A2.5.4.6.4.1 - Tachometer Shield	xx

Figure A2.5.4.7.1.1 - Arduino UNO	xx

Figure A2.5.4.7.2.1 - Arduino ESP8266 WiFi Card	xx

Figure A2.5.4.7.2.2 - Arduino ESP8266 WiFi Card	xx

Figure A2.5.4.7.2.3 - Arduino ESP8266 WiFi Card Wiring Diagram	xx

Figure A2.5.4.7.3.1 - AS-6408 Multiplexer	xx

Figure A2.5.4.7.4.1 - Arduino Uno with Breadboards	xx

Figure A2.5.4.7.5.1 - Electrical Signal Isolators (also for solid state relays)	xx

Figure A2.5.4.7.6.1 - BNC Adapters to Breadboards (for pH, EC, ORP, and DO)	xx

Figure A2.5.4.7.7.1 - USB to Breadboard Adapters for Endoscopic Microscopes	xx

Figure A2.5.4.7.8.1 - 1560 Tie Point Solderless Breadboard 	xx

Figure A2.5.4.7.9.1 - Arduino Kit	xx

Figure A2.5.4.7.10.1 - Serial to USB Panel Mount Connector	xx

Figure A2.5.4.8.1.1 - Waterproof Cable Gland (PG Gland) Connectors	xx

Figure A2.5.4.8.2.1 - Waterproof USB Bulkhead Connector	xx

Figure A2.5.4.8.3.1 - LeMoTech Project Box Enclosure	xx

Figure A2.5.4.9.1.1 - Electrically Actuated Shutoff Valves	xx

Figure A2.5.4.9.2.1 - Manually Operated Valves 	xx

Figure A2.5.4.9.3.1 - Manual Shutoff Valve: 12 mm to 12 or 16 mm Hose Barb	xx

Figure A2.5.4.9.4.1 - Five Valve Manifolds for Air Handling	xx

Figure A2.5.4.9.5.1 - Bubble Disk Aerators	xx

Figure A2.5.4.9.6.1 - Suction Clips for Mounting Wires and Plumbing Within Aquarium		xx

Figure A2.5.4.9.6.2 - Suction Clips for Mounting Wires and Plumbing Within Aquarium		xx

Figure A2.5.4.10.1 - Hose Barb Fitting: 1/4" x 1/2" x 1/4" Union Tee	xx

Figure A2.5.4.10.2 - Hose Barb Fitting: 1/8" x 1/16" Reducing Union	xx

Figure A2.5.4.10.3 - Hose Barb Fitting: 1/4" x 1/2" x 1/4" Union Tee	xx

Figure A2.5.4.10.4 - Hose Barb Fitting: 1/2" x 1/8" Reducing Union	xx

Figure A2.5.4.10.5 - Hose Barb Fitting: 3/8" x 1/4" Reducing Union	xx

Figure A2.5.4.10.6 - Hose Barb Fitting: 1/4" x 1/8" x 1/4" Union Tee	xx

Figure A2.5.4.10.7 - Hose Barb Fitting: 1/8" x 1/16" Reducing Union	xx

Figure A2.5.4.10.8 - Hose Barb Fitting: 1/2" x 5/16" Elbow	xx

Figure A2.5.4.10.9 - Hose Barb Fitting: 1/8" x 1/16" x 1/8" Union Tee	xx

Figure A2.5.4.10.10 - Hose Barb Fitting: 1/4"-1/8" Reducing Union	xx

Figure A2.5.4.10.11 - 6 mm-12 mm Hose Barb Tail 1/4" 1/2" NPT Female Thread Connector Joint Pipe Fitting	xx

Figure A2.5.4.10.12 - 1/4" Male Thread Pipe Fitting x 6 mm Barb Hose Tail Connector Stainless Steel NPT	xx

Figure A2.5.4.10.13 - NPT 1/4" Male Thread Pipe Fitting x 8 mm Barb Hose Tail Connector Stainless Steel	xx

Figure A2.5.4.10.14 - 2/3/4 Way Hose Barb Connectors	xx

Figure A2.5.4.10.15 - Hose Barb Connectors from 3 mm to 6 mm Tubing	xx

Figure A2.5.4.10.16 - 4 mm to 8 mm Hose Barb Connectors	xx

Figure A2.5.4.10.17 - 2 mm to 4 mm Hose Barb Connector	xx

Figure A2.5.4.11.1.1 - Silicone Tubing	xx

Figure A2.5.4.11.2.1 - Tygon Tubing	xx

Figure A2.7.3.1: P&ID example of feed distribution 	xx

Figure A2.7.4.1: P&ID example of waste collection	xx

Figure A2.7.4.2: P&ID example of return distribution	xx

Figure A2.7.4.3: P&ID of entire tissue engineering test bed	xx

Figure A3.1.1.1 - VICI Valco Instruments EMHMA-CE Multiposition Valves and Controller	xx

Figure A3.1.2.1: NI SC-2054	xx

Figure A3.1.2.2: SC-2054 Parts Locator Diagram	xx

Figure A3.1.2.3: Pin Assignments for SC-2054 I/O Connectors J1 and J2	xx

Figure A3.1.2.4: Pin assignments for SC-2054 I/O connectors J4 and J8	xx

Figure A3.1.2.5: Pin assignments for SC-2054 I/O connectors J6 and J10	xx

Figure A3.1.3.1: 100 pin ribbon cable	xx

Figure A3.1.4.1: 26 Wire Rainbow Ribbon Cable	xx

Figure A3.1.4.2: Schematic of 20-pin ribbon cable	xx

Figure A3.1.5.1: PCI-DIO-96	xx

Figure A3.2.1.1 - Valco VICI HPLC 8 Position Valve Controller	xx

Figure A4.1.2.1: Calibration curve for uncoated dye trials 1 and 2 at 380 nm	xx

Figure A4.1.2.2: C(t) for uncoated dye trial 2	xx

Figure A4.1.2.3: Polymath report for uncoated dye trial 2 solving for Area	xx

Figure A4.1.2.4: Residence time distribution, E(t), for uncoated dye trial 2	xx

Figure A4.1.2.5: Polymath report for uncoated dye trial 2	xx

Figure A4.1.2.6: Cumulative distribution curve, F(t), for uncoated dye trial 2	xx

Figure A4.1.3.1.1: Polymath report for uncoated dye trial 1	xxx

Figure A4.1.3.1.2: Polymath report for uncoated dye trial 2	xxx

Figure A4.1.3.1.3: Polymath report for uncoated dye trial 3	xxx

Figure A4.1.3.1.4: Polymath report for uncoated dye trial 4	xxx

Figure A4.1.3.1.5: Polymath report for uncoated dye trial 5	xxx

Figure A4.1.3.2.1: Polymath report for uncoated glucose trial 1	xxx

Figure A4.1.3.2.2: Polymath report for uncoated glucose trial 2	xxx

Figure A4.1.3.2.3: Polymath report for uncoated glucose trial 3	xxx

Figure A4.1.3.2.4: Polymath report for uncoated glucose trial 4	xxx

Figure A4.1.3.2.5: Polymath report for uncoated glucose trial 5	xxx

Figure A4.1.3.3.1: Polymath report for coated tracer dye trial 1	xxx

Figure A4.1.3.3.2: Polymath report for coated tracer dye trial 2	xxx

Figure A4.1.3.3.3: Polymath report for coated tracer dye trial 3	xxx

Figure A4.1.3.3.4: Polymath report for coated tracer dye trial 4	xxx

Figure A4.1.3.3.5: Polymath report for coated tracer dye trial 5	xxx

Figure A4.1.3.4.1: Polymath report for coated tracer glucose trial 1	xxx

Figure A4.1.3.4.2: Polymath report for coated tracer glucose trial 2	xxx

Figure A4.1.3.4.3: Polymath report for coated tracer glucose trial 3	xxx

Figure A4.1.3.4.4: Polymath report for coated tracer glucose trial 4	xxx

Figure A4.1.3.4.5: Polymath report for coated tracer glucose trial 5	xxx

List of Tables

Table 1.1.1: Advantages of microfluidic cell culture techniques (Poss = Possible, N/C = Not Clear) (Halldorsson, et al. 2015).	27

Table 1.1.2: Summary table for experimental investigations to the impact of shear stress on osteogenic and cell differentiation I (McCoy and O'Brien 2010)	28

Table 1.1.3: Summary table for experimental investigations to the impact of shear stress on osteogenic cell differentiation II (McCoy and O'Brien 2010)		29

Table 1.1.4: Summary table for experimental investigations to the impact of shear stress on osteogenic cell differentiation III (McCoy and O'Brien 2010)	30

Table 1.1.5: Summary table for experimental investigations to the impact of shear stress on osteogenic cell differentiation IV (McCoy and O'Brien 2010)	31

Table 1.2.1: Companies with commercial bioreactor systems (Yeatts 2011)	34

Table 2.5.1: Sensors dimensions sheet.....................................................................	58

Table 2.8.1 – Hazard and Operability Analysis	91

Table 2.13.1 – Budget Breakdown	108

Table 3.1.1: 26 pin ribbon cable wires used to automate VICI 10-way valve	115

Table 3.1.2: Example of wiring specifications for proper communication from MAX to VICI 10-way valve	115

Table 3.3.2.1 – Mass of Solids Removed	134

Table A2.5.1.4.6.1 - Vernier LabQuest 2 Specifications	216

Table A2.5.1.9.3.1 - Vernier SpectroVis Plus Spectrophotometer Specifications	236

Table A2.5.1.12.1.1 - Bioreactor Vessel Specifications	254

Table A2.5.2.1.2.1 – Flowmeter specifications 	261

Table A2.5.2.3.1 – Turbidity sensor specifications	265

Table A2.5.2.5.1: Item specifications for diaphragm pump	270

Table A2.5.3.1.1.1 - Diaphragm Pump Specifications	277

Table A2.5.4.2.5.1 - Optical and Electrical Characteristics @TJ = 25 oC	304

Table A2.5.4.7.1.1: Item specifications for Arduino Uno	350

Table A2.5.4.8.1.1 - PG Gland Specifications	366

Table A2.5.4.11.1.1 - Silicone Tubing Specifications	393

Table A2.5.4.11.2.1 - Tygon Tubing Specifications	395

Table A3.1.1.1: Item Specifications for VICI 10-Way Valve	413

Table A3.1.2.1: Item specifications of NI SC-2054	417

Table A3.1.3.1: 100 pin ribbon cable specifications	418

Table A3.1.4.1: Item specifications for 26 pin ribbon cable	419

Table A3.1.5.1: Item specifications for PCI-DIO-96	423

Notation

| A, abs. units |  | absorbance |
| --- | --- | --- |
| Area |  | area |
| C, mol/L |  | concentration |
| CGlucose, mol/L |  | concentration of glucose |
| CGlucose, exit, mol/L |  | concentration of glucose in effluent |
| CGlucose, feed, mol/L |  | concentration of glucose in feed |
| CReactor contents, mol/L |  | concentration of reactor contents |
| C(t), mol L-1 s-1 |  | concentration function of time |
| D, m2/min |  | dispersion coefficient |
| Dispersion |  | dimensionless dispersion |
| Et, min-1 |  | residence time distribution |
| F(t), min-1 |  | cumulative distribution curve |
| L, m |  | path length |
| l, m |  | characteristic length |
| m, g |  | mass final |
| m0, g |  | mass initial |
| mCells, g |  | mass of cells |
| mGlucose, g |  | mass of glucose |
| Q, L/s |  | volumetric flowrate |
| QRe-Circ, in, L/s |  | volumetric flowrate in the bioreactor recirculation loop |
| QSyringe, in, L/s |  | volumetric flowrate in the syringe pump |
| Qwaste, out, L/s |  | volumetric flowrate in the bioreactor effluent |
| -rGlucose/cell, g/cell/s |  | rate of disappearance of glucose per cell |
| -rnet ads, reactor, g/s |  | rate of adsorption in the reactor |
| -rnet ads, walls, g/s |  | rate of adsorption on the walls of the plumbing |
| u, m/s |  | fluid velocity |
| VGlucose, m3 |  | volume of glucose |
| VReactor Contents, m3 |  | volume of reactor contents |
| Greek Letters |  |  |
| α |  | absorbance vs. concentration slope |
| αdye |  | absorbance vs. concentration slope of tracer dye |
| αglucose |  | absorbance vs. concentration slope of glucose |
| σ2, no units |  | variance |
| ε , M-1 cm-1 |  | molar absorptivity |

J 	= Volumetric flux passing through the filter, mL cm-2 s-1

ΔP 	= Pressure drop across the filter, Pa

µ 	= Viscosity of the filtrate, Pa s

RT 	= Total resistance in filter, cm-1

Rm 	= Media resistance, cm-1

Rc 	= Cake resistance (reversible), cm-1

Rf 	= Fouling resistance (irreversible), cm-1

r1 	= Particles from suspension sticking to fibrous mesh

r-1 	= Particles into suspension desorbing from fibrous mesh

r2 	= Particles from suspension sticking to other particles

r-2 	= Particles into suspension desorbing from other particles

A 	= concentration of particles in suspension, g L-1

* 	= concentration of available surface sites on fibrous mesh, cm-2

A* 	= concentration of surface sites on fibrous mesh with sticking particles, cm-2

AM* = concentration of surface sites on fibrous mesh with particles

stuck as a part of a multilayer film, cm-2

L 	= number of layers of particles

f 	= fraction of the maximum multilayer particle film that is currently

occupied

fm 	= fraction of sites that have a particle on them that have 2+ particles on

them

[C] 	= capacity of filter, cm-2

ε   	= Porosity of the bed

V   	= Velocity based on the empty cross-section of the bed

dp    	= Particle diameter

Φs 	= Sphericity of the particles in the bed

Re 	= Reynolds number

kr2   	= Viscous permeability

εp 	= Percolation threshold

σ 	= Osmotic reflection coefficient

π 	= Osmotic pressure of the polarized layer

Chapter 1 – Motivation/Introduction

1.1 - Microfluidic Approaches to Tissue Engineering Test Beds

Dr. James Brenner’s “Tissue Engineering Test Bed” project looks to introduce automation to the tissue engineering development process by utilizing an Arduino-based system that includes multiple sensors that can track and control variables within the test bed, including feeding, in vivo imaging, and waste removal without having to take the cells out of incubation. Ultimately, Dr. Brenner would like to take a microfluidic approach to his test bed, but for now, the cost of appropriately sized sensors and controls is many times higher than those for their non-microfluidic counterparts.  In the meantime, Dr. Brenner's group will design fully mass balanced, non-microfluidic, automated cell culturing kits.  A summary of the advantages of microfluidic approaches to cell culturing is in Table 1.1.1 (Halldorson et al., 2015) and for experimental investigations to the impact of shear stress on osteogenic and cell differentiation in Tables 1.1.2 - 1.1.5 (McCoy and O'Brien, 2010).

Table 1.1.1: Advantages of microfluidic cell culture techniques (Poss = Possible, N/C = Not Clear) (Halldorsson, et al. 2015). Reprinted with permission.

Table 1.1.2: Summary table for experimental investigations to the impact of shear stress on osteogenic and cell differentiation I (McCoy and O'Brien 2010). Reprinted with permission.

Table 1.1.3: Summary table for experimental investigations to the impact of shear stress on osteogenic cell differentiation II (McCoy and O'Brien 2010). Reprinted with permission.

Table 1.1.4: Summary table for experimental investigations to the impact of shear stress on osteogenic cell differentiation III (McCoy and O'Brien 2010). Reprinted with permission.

Table 1.1.5: Summary table for experimental investigations to the impact of shear stress on osteogenic cell differentiation IV (McCoy and O'Brien 2010). Reprinted with permission.

## 1.2 Vanderbilt/LabSmith Inspiration

The goals of the tissue engineering test bed research and development at Florida Tech are automation and reproducibility. The first objective for Dr. Brenner’s research group is to create a fully automated test bed that emulates conditions within the human body. This includes an automated nutrient supply, automated waste removal, periodic patient monitoring like one would see in the hospital, and in vivo imaging. The second objective is to have 36 fully instrumented bioreactors running in parallel. The automation minimizes the reproducibility issues and should allow systematic evaluation of the synthesis and cell culturing variables. Thirdly and most importantly, this system design includes full mass and energy balance accounting.

This test bed development would immediately enhance research at Florida Tech by streamlining cell culturing and allowing a vast expansion in testing capabilities. Having a fully mass- and energy-balanced system will permit scale-up to commercial scale tissue engineering to achieve a 24/7 system operating without human intervention.

The biggest impact on tissue engineering is building micro-culture platforms with the ability to control micro-environmental parameters with spatial resolution emulating natural tissue (Baudoin 2011). These platforms are tools that allow further comprehension of the effects of temporal and spatial effects of cell–cell, cell–substrate, cell–extracellular matrix, and cell–soluble factor interactions on the cell life and utility (Baudoin 2011). A tissue engineering test bed avoids labor intensive methods such as manual flushing and refilling of cell cultures, which are standard for commonplace cell culture operations. The test bed generates the potential for dynamic culture and kinetics studies on microstructured tissues that will more closely mimic in vivo conditions (Baudoin 2011). If researchers are equipped with a functional tissue engineering test bed, they can “optimize culture conditions for providing renewable sources of functional cells and complex biomaterial scaffolds, as better templates for microtissue formation” (Puleo, Yeh and Wang 2007). This is the motivation for Dr. Brenner’s research group.

Currently, there is no standardized tissue engineering test bed. Table 1.2.1 lists the companies who have commercial bioreactor systems (Yeatts, 2011). These bioreactors are not all the same type of bioreactor, but they are all commercialized.  The only group who currently has automated feeding, waste removal, and imaging is a Vanderbilt group (Wikswo, McKenzie and Cliffel 2013); their project serves as a foundation on which this project is based. LabSmith commercializes much of what Wikswo and Cliffel have established at Vanderbilt. LabSmith recently won an NSF SBIR (Basic Microfluidic Education Kit 2018) to find users to beta test one of their chemical engineering plant on a breadboard kits. Dr. Brenner's research group at Florida Tech was one of the beta testers.

LabSmith specializes in microfluidic flows that are key to lab-on-a-chip and organ-on-a-chip devices. The LabSmith kit shown in Figure 1.2.1 has two syringe pumps, a valve controller, two 3-way valves, two 1 mL feed tanks, a 5 mL waste/product tank, a laminar flow visualization chip, and 1/16” inner diameter polyetheretherketone (PEEK) tubing. It can be built and tested in one hour and served as an expensive prototype for dispensing feedstocks to the automated tissue engineering test bed project. Though the automated tissue engineering test bed will not be as small as the LabSmith kit, it is a goal to achieve this scale in future externally funded work after this effort is completed.

Table 1.2.1: Companies with commercial bioreactor systems (Yeatts 2011)

Permission license number: 4574410899030

Figure 1.2.1: LabSmith basic microfluidic kit (Basic Microfluidic Education Kit 2018) Permission to use image from LabSmith (2019).

Wikswo, McKenzie, and Cliffel’s setup (2013), now commercialized by LabSmith, interfaces an inverted $8000 Nikon microscope with a light source viewing samples from the bottom through the walls of their bioreactors, but inside their incubated volume (Figure 1.2.2).  To implement such microscopy for a multi-reactor system would require a motorized stage inside the incubated volume because the cost of a microscope for each bioreactor would be prohibitive. The Wikswo/Cliffel bioreactors are not much bigger than the glass slide that serves as a substrate for each tissue sample. In theory, scale-up to more than a dozen bioreactors in parallel would not be practical from a size or cost standpoint.

Figure 1.2.2: Vanderbilt microscope setup (Wikswo, McKenzie and Cliffel 2013).  Open Access.

Vanderbilt’s bioreactor system schematic is shown in Figure 1.2.3; on the left of the schematic, red items represent the sources similar to arteries, while on the right, the blue items represent the sinks, acting like veins (Wikswo, McKenzie and Cliffel 2013). So, intuitively, the path of flow goes from sources to sink, or from arteries to veins. On the left is a multiport valve with eight inputs and one output, while on the right is a multiport valve with one input and eight outputs. Not shown is the use of 3-way multiport valves for the distribution of arterial supply and collection of venous return. This document builds upon their multiport use and describes the modifications in Section 1.3. Note the use of multiport valves, various feedstocks, a bubble trap, a recirculation loop, and two forms of concentration analyzers, µClinical analyzer and the mass spectrometer (MS).

Figure 1.2.3 - a) Vanderbilt research group tissue engineering test bed design (left) and b) concentration monitoring sensor (right); (Wikswo, et al., 2013); Open Access.

One of the biggest limitations with the Vanderbilt setup is that it is very expensive for the number of reactors in their system. Figure 1.2.4 details their testbed design’s utilization of the eight syringe pumps with 3-way multiport valves. The employment of multiple syringe pumps is a great idea that is expanded on in the tissue engineering testbed design described later in this thesis. Adding even more syringe pumps can add versatility to the number of types of cells, more customized growth media, growth and differentiation factors, and more.

The goal for tissue engineering test bed producers is to automate the system. LabSmith and Vanderbilt have created an excellent blueprint for what may eventually become the industry standard, but their systems are not designed for large scale production. Dr. Brenner’s group aims to increase the reproducibility by scaling up to 36 bioreactors. With a full mass-and-energy-balanced system, research and development in the tissue-engineering field can be dramatically accelerated.

Figure 1.2.4: Vanderbilt valve and pump schematic (Wikswo, McKenzie and Cliffel 2013). Open Access.

## 1.3 Improvements to Existing Test Bed Designs

As discussed previously, the only group who currently has automated feeding, waste removal, and imaging is a Vanderbilt (Wikswo, McKenzie and Cliffel 2013) group whose work serves as a foundation on which this project is based. Though Vanderbilt has excellent components in their test bed (concentration monitoring, imaging, and process flow), there certainly is room for improvement.

The first modification made from Vanderbilt’s model is the valve control system. They currently use several multiport valves to service 12 bioreactors (Wikswo, McKenzie and Cliffel 2013). In order to increase reproducibility, our system has been equipped to feed inputs and accept outputs from an array of 36 bioreactors, a number picked largely based on the number of VICI 10-way valves that Dr. Brenner could purchase affordably This improvement on the limited number of valves and valve controllers in Vanderbilt model (Wikswo, McKenzie and Cliffel 2013) has been done by using eighteen Valco Instruments Company Incorporated (VICI) multiport valves & controllers (Figure 1.3.1) to send fluid via Kloehn eight-channel syringe pumps (Figure 1.3.2) to or from bioreactors for feed and/or purge from bioreactors to waste removal. Replacing Vanderbilt’s three-way valves, $600 each (Wikswo, McKenzie and Cliffel 2013), with the VICI 10-way valves, 18 for $450 (Valco VICI Microelectric Valve Actuator 2019), lowers the cost of scaling up to a much larger bioreactor array. A significant amount of time spent on this thesis was the diagnostic testing done to achieve connectivity between the data acquisition and control software of National Instruments (NI) Measurement and Automation Explorer (MAX), NI LabView, and the VICI 10-way valves. This is discussed in detail in Section 3.1.

Figure 1.3.1: VICI 10-way valves

Another major feature of Vanderbilt’s tissue engineering testbed design is its use of eight syringe pumps for sending an array of feedstocks, including Phosphate Buffered Saline (PBS), glucose, or various media. The employment of multiple syringe pumps adds versatility to the number of types of cells, more customized growth media, growth and differentiation factors, Dulbecco’s Modified Eagle Medium (DMEM), ultrapure water, salt water, multiple dyes, multiple cell types, and drugs for toxicological testing. Vanderbilt and LabSmith use $600 syringe pumps (Basic Microfluidic Education Kit 2018). Those have been replaced with three Kloehn eight-channel syringe pumps costing $200 each (Kloehn Syringe Pump V8 Multi-Channel Syringe Pump 2019).  Because the electronics associated with the Kloehn syringe pump system are 20th century, a major part of building the tissue engineering test bed  to be accomplished by future students in Dr. Brenner’s research group will be proper automation of the Kloehn syringe pumps with data acquisition and control software of Arduino and NI LabView controls.

Figure 1.3.2: Kloehn eight channel syringe pumps

Finally, there will be major modifications to the sensors. The goal for the tissue engineering test bed at Florida Tech is to have enough sensors in the bioreactors to quantify mass and energy balances necessary for full engineering analysis and capabilities. In order to achieve this, mass balance accounting is conducted for each control volume (each bioreactor and the entire aquarium full of bioreactors). The design accounts for using sensors to measure the following parameters:  total mass, H+ concentration (via pH), each of the salt ions (Na+, K+, and Ca+2 are particularly important and require their own sensors.), total salt concentration (via electrical conductivity), glucose (for feeding), lactate (as an indicator of glucose conversion to waste), and dissolved oxygen. Therefore, each bioreactor will have a liquid level sensor, lactate sensor, pH sensor, dissolved oxygen sensor, load cells to measure the mass of the growing cells and another fot the entire reactor contents, conductivity sensor(s), and a glucose concentration sensor. Except for the concentration monitoring sensors, these sensors and controls will be purchased and Arduino-based to minimize cost. Development of some of the molecule and ion specific sensors is the focus of a Ph.D. dissertation in progress by Guochang Ye in Florida Tech’s Biomedical Engineering Program. To control and monitor all of these variables, a Hazards Operability Analysis (HAZOP) has been conducted. This will aid in the foundation for control. A partial example is shown in Section 2.8 and its corresponding appendix.

1.4 - Arduinos Lowering Sensor Cost

Figure 1.4.1: Libelium’s Open Aquarium inspiration for making inexpensive, fully instrumented bioreactors includes Arduino-based control of temperature and liquid level, as well as sensing of pH and of solution electrical conductivity (i.e. salinity; https://www.cooking-hacks.com/documentation/tutorials/open-aquarium-aquaponics-fish-tank-monitoring-arduino, 2018)

1.5 – Literature Analysis

blah

1.6 – Problem Identification

There are several phases for the tissue engineering test bed development. The hypothesis for this work is that a tissue engineering test bed can be designed and tested in which no mass of glucose accumulates on the plumbing.

The objectives for this work were/are as follows:

- Schematically design the plumbing and instrumentation of the tissue engineering test bed;

- Determine feasibility of Tygon and silicone tubing for tissue engineering test bed applications by demonstrating that no mass grows or accumulates in the system;

- Determine whether coating the tubing is necessary, and if so, eliminates adsorption of mass onto Tygon and silicone tubing walls.

Generating a full process flowsheet was the first checkpoint. The subsequent piping & instrumentation diagram (P&ID), stream numbering and mass balance list must follow. The schematics and mass balance accounting serve as the foundation for coding the data acquisition and control for the tissue engineering test bed.

Next, for the tubing to be deemed adequate, all mass entering the plumbing must exit through the plumbing. Moreover, generation minus depletion for all test cases prior to tissue growth ought to be zero as well, specifically tracer dyes and glucose. This means that the glucose does not cause algae, bacteria, or other microorganisms to grow. Tracer studies were conducted to verify this and represented the second checkpoint. First, a reactor analysis with a tracer dye of green food coloring through the Tygon and silicone tubing was conducted. Then a glucose tracer test was conducted. Testing in this manner was done to confirm or deny that the net rate adsorption of the walls is negligible. If deemed successful, Tygon tubing will be considered acceptable for tissue engineering applications as no mass accumulated in the system.

For the first tracer tests of dye and glucose, no coating was used. Afterward, the plumbing was coated to test the necessity of further preventing adsorption on the walls.  For the second tracer tests, the system was coated with polyethylene glycol (PEG) to minimize protein adhesion. Tracer dye and glucose studies were conducted after coating and ultrapure water purging to validate that there was no accumulation of mass throughout the system. These tests were to determine whether coating the plumbing was necessary and, if so, sufficient to minimize protein or glucose adhesion.

The final aspect of Adrien Hosking's thesis (ref.) was the automation of the VICI 10-way valves. It is vital that the 10-way valves interface with data acquisition and control software such as National Instruments Measurement and Automation Explorer (MAX) and ultimately LabView. In Section 3.1, diagnostic testing was conducted to establish connectivity between the VICI 10-way valves.

blah

1.7 – Big Picture Task Plan

1)   Mount valves and valve controllers to frame  DONE

2)   Finish wiring of valve controllers to valves  DONE

3)   Purchase ribbon cables to connect valve controllers to DIO cards, purchase DIO cards +

multiplexer

Purchased 5 DIO-96 cards, 20 26-pin ribbon cables, 6 SC-2054 cards to connect DIO-96

cards to 26 pin ribbon cables, 100 pin to 50/50 ribbon connectors from DIO-96’s to SC-2054’s, PCI-IMAQ-1408 image acquisition board, downloaded IMAQ 2.6.1 (the highest version compatible with IMAQ-1408 and LabView 6 – but must be installed on XP or lower), purchased breakout cable for multiple image inputs to 1408, purchased 2 AMUX-64T’s to connect to 6024e DAQ cards, purchased 10 50 to 50-pin ribbon connectors, also 20 24 VDC adapters for valve controllers; purchased Aalborg 4-channel mass flow controller power supply/readout to be converted into position controller for analog outputs to syringe pumps for solutions of collagen, Bioglass, crosslinker, and cells.

4)   Connect all from 3) and install National Instruments (NI) hardware card to computer

5)   Write or obtain LabView code to control valve positions

Valve control manuals:

http://my.fit.edu/~jbrenner/cbashur/3dprinting/valcovicivalvestn415.pdf

http://my.fit.edu/~jbrenner/cbashur/3dprinting/valcovicivalvemanual.pdf

6)   Wire multichannel syringe pump or controller to computer or NI hardware card

Syringe positions will be analog outputs

Should we use mass flow controller power supplies? Just purchased

7)   Write or use Kloehn pump .vi to control all channels of multichannel syringe pumps

8)   Mount reservoirs to breadboard DONE

9)   Put together P&ID

10) Label all valves, valve controllers, cables, tubing

11) Design/purchase automatable X-Y stage for inverted microscope

12) Write LabView or use LabSmith code for microscope

13) Plan out feeding and waste removal time schedule

14) What variables to measure in each bioreactor?

15) What variables to measure in each multichannel concentration analyzer?

16) How to sample cell growth in bioreactor?  Absorbance? At what wavelength?  Dye?

17) How to evaluate tight junction formation?  Dendrite accumulation?

18) How to simulate electrokinetic flows?  LabSmith has some ideas.

19) Residence time distribution for dye or glucose tracers

Chapter 2 – Tissue Engineering Test Bed Design

2.1 – Questions and Issues Sheet Summary

blah

2.2 – Bioreactor Design Considerations

Figure 2.2.1:  Bioreactor instrumentation and controls

In Figure 2.2.1 above, the liquid level will be sensed by a load cell (Sensor A) underneath the bioreactor and with a float. The temperatures of each bioreactor, the entire aquarium, and the aquarium headspace will be measured via sensors B, H, and I, respectively. To define the acidity mass balance, we need to measure and control pH through sensor C.  The salt mass balance will be quantified by an electrical conductivity sensor (sensor D). Sensor E, for oxidation/reduction potential, will be used to assure that the system is suitably sterile before the experiment. Glucose (sensor F) and lactate (sensor G) concentrations will be used to assess whether the cells are consuming food and excreting waste. A flow totalizer external to the bioreactor itself, but within the bioreactor control volume, will be used to measure the recirculation flow rate. The cells require enough oxygen, as measured using a dissolved oxygen sensor (K). The second hardest part will be to examine the cells under water in real time with a submersible fiber optic microscope (J).  Finally, the hardest part will be to measure the mass of growing cells using a 24-bit 1 kg maximum load cell like those used for mass balances in an underwater environment. As with a traditional covered mass balance, the stability of the recorded mass of glass slide plus scaffold plus cells is much better when there is no flow in the system. We are considering how best to “vibration isolate” the mass balance, as well as how to automatically and periodically shut off mixing to the cells for brief times to do mass measurements.

2.3 – Imaging Design Considerations

RGBLEDonoff.txt

EZO_RGB_Datasheet.pdf

Arduino-Uno-RGB-sample-code.pdf

EZO-RGB-how-to-view-rgb-output-on-your-computer.pdf

2.4 – Mass and Energy Balances

Equation 2.4.1 shows the relationship between mass flowrate, reactor volume, and glucose concentration and their interactions with each other. The overall change in mass of glucose per time, dmGlucosedt, equates to the rate of disappearance of glucose per cell, -rGlucose/cell, multiplied by the number of cells, m. CGlucose is the exit concentration of glucose. VReactor contents is the volume of reactor contents. dCGlucosedt is the differential change in concentration of glucose over time. Cglucose is the concentration of glucose in mass per volume. dVGlucosedt  is the differential change in the volume of glucose over time.

|  | dmGlucosedt=VReactor contents*dCGlucosedt+ Cglucose* dVReactor contentsdt= -rGlucose = -rGlucose/cell* mCells | (2.4.1) |
| --- | --- | --- |

Equation 2.4.1 can be rewritten as Equation 2.4.2 to show a mass in minus mass out relationship. Here, the relationship of flows in, QSyringe,in, multiplied by the glucose concentration of feed, CGlucose,feed, minus the volumetric flow out of waste, Qwaste, out, multiplied by the glucose concentration of effluent, CGlucose, exit, plus the volumetric flow within the reactor, QRe-Circ,  in, multiplied by the glucose concentration of the effluent. Again, this entire equation is equal to the rate of disappearance of glucose (or the metabolism), -rGlucose.

|  | dmGlucosedt=QSyringe, in * CGlucose, feed- Qwaste, out * CGlucose, exit+ QRe-Circ, in * CGlucose, exit+ -rnet ads, walls -rnet ads, reactor. = -rGlucose | (2.4.2) |
| --- | --- | --- |

However, there are two additional terms that must be considered: the net rate of adsorption on any walls outside each bioreactor, -rnet ads,   walls, and the net rate of adsorption within each bioreactor, -rnet ads, reactor.

Solving for the overall change in mass of glucose per time in Equation 2.4.2, most of the terms are directly measured. The volumetric flow rate for the feed is controlled by a syringe pump at known values. The effluent flow rate and recirculation rate are measured with a flow totalizer and controlled with a diaphragm pump. The concentrations of glucose for feed, effluent, and recirculation are measured with concentration sensors. These sensors are being tested by biomedical engineers at Florida Tech under the direction of Profs. Jim Brenner and Mehmet Kaya as well as by the company IORodeo that pioneered the CheapStat and RodeoStat (IORodeo 2019). The two terms that cannot be measured directly are the net rate of adsorption on any walls -rnet ads, walls, and the net rate of adsorption within each bioreactor, -rnet ads, reactor. It is imperative that these terms are negligible to determine the correct rate of disappearance of glucose, -rGlucose. Without-rGlucose, a proper mass balance accounting of the glucose cannot be achieved and cell vitality is impaired.

The predominant objective of Adrien Hosking's thesis (ref.) was to prove that the net adsorption of the walls of the tubing outside the bioreactor is equal to zero. Adsorption is defined as the process by which a solid holds molecules of a gas or liquid or solute as a thin film (Tro 2014). That means explicitly that no mass has adsorbed onto the walls of the tubing. Testing conducted as part of this thesis was conducted to prove or disprove that -rnet ads,   walls  equals zero. Fellow chemical engineering M.S. thesis candidate Princess Akande aims to demonstrate that -rnet ads,   reactor is also negligible. Additionally, chemical engineering M.S. thesis candidate Nicole Bueno hopes to further the mass balance accounting by measuring the mass of growing cells, despite the challenges associated with doing so over extended time in a salt water environment with mixing. This will quantify the relationship between the disappearance of glucose and the growth of the cells. Furthermore, when -rnet ads,   walls and -rnet ads,   reactor are calculated, the calculation of the rate of disappearance of glucose term can be verified.

- Addition of food/reactants/etc. are not continuous. They are pulses.

- Each time that a new pulse is sent, the differential equations effectively have a new initial condition.

Control Volumes

- Whole Bioreactor

- Bioreactor & Surrounding loop

- Pre-bioreactor (syringe Pump, valves, etc.)

- Post-bioreactor(concentration analysis and waste removal)

- Scaffold and cells and glass slide

- Aquarium

Mass and Energy Balance Accounting

- List all components (molecules, cells, scaffold, glass slide, ions, waste solid)

- For each component

  - Cp

  - Mass or mass flow rate

  - Molecular weight if any

  - Enthalpy (standard state and difference from standard state)

  - Mass or molar fraction

- ∆Hmix

- Heaters in aquarium

- Heat losses to bottom table

- Heat losses to surrounding air through sides

- Heat losses through top

Lid has penetrations for all sensors and impellers

- Mixing of entire aquarium

Can it be treated as uniform constant temperature bath?

- Heat rises- Each bioreactor will have “flip-top” with penetrations

Note: 4-7 *aquarium level*

Bioreactor

- Top

- Sides

- Bottom

The main objective of Dr. Brenner’s research group at Florida Tech is the construction of a fully automated tissue engineering test bed. Such a system enhances tissue engineering potential future research capabilities at this university and is a vital tool for expanding current research. The scope of Princess Akande's thesis (ref.) focuses will be on the mass balances, particularly for glucose.

Equation 1 shows the relationship between mass flowrate, reactor volume, and glucose concentration and their interactions with each other. The overall change in mass of glucose per time, dmGlucosedt, equates to the rate of disappearance of glucose per cell, -rGlucose/cell, multiplied by the number of cells, m. CGlucose is the exit concentration of glucose. VReactor contents is the volume of reactor contents. dCGlucosedt is the differential change in concentration of glucose over time. Cglucose is the concentration of glucose in mass per volume. dVGlucosedt  is the differential change in the volume of glucose over time.

|  | dmGlucosedt=VReactor contents*dCGlucosedt+ Cglucose* dVReactor contentsdt= -rGlucose = -rGlucose/cell* mCells | (3) |
| --- | --- | --- |

Equation 1 can be rewritten as Equation 2 to show a mass in minus mass out relationship. Here, the relationship of flows in, QSyringe,in, multiplied by the glucose concentration of feed, CGlucose,feed, minus the volumetric flow out of waste, Qwaste, out, multiplied by the glucose concentration of effluent, CGlucose, exit, plus the volumetric flow within the reactor, QRe-Circ,  in, multiplied by the glucose concentration of the effluent. Again, this entire equation is equal to the rate of disappearance of glucose (or the metabolism), -rGlucose.

|  | dmGlucosedt=QSyringe, in * CGlucose, feed- Qwaste, out * CGlucose, exit+ QRe-Circ, in * CGlucose, exit+ -rnet ads, walls -rnet ads, reactor. = -rGlucose | (4) |
| --- | --- | --- |

However, there are two additional terms that must be considered: the net rate of adsorption on any walls outside each bioreactor, -rnet ads,   walls, and the net rate of adsorption within each bioreactor, -rnet ads, reactor.

Solving for the overall change in mass of glucose per time in Equation 2, most of the terms are directly measured. The volumetric flow rate for the feed is controlled by a syringe pump at known values. The effluent flow rate and recirculation rate are measured with a flow totalizer and controlled with a diaphragm pump. The concentrations of glucose for feed, effluent, and recirculation are measured with concentration sensors. These sensors are being tested by biomedical engineers at Florida Tech under the direction of Profs. Brenner and Kaya as well as by the company IORodeo that pioneered the CheapStat and RodeoStat (IORodeo 2019). The two terms that cannot be measured directly are the net rate of adsorption on any walls -rnet ads, walls, and the net rate of adsorption within each bioreactor, -rnet ads, reactor. It is imperative that these terms are negligible to determine the correct rate of disappearance of glucose, -rGlucose. Without-rGlucose, a proper mass balance accounting of the glucose cannot be achieved and cell vitality is impaired.

The predominant objective of this thesis is to prove that the net adsorption of the walls of the tubing outside the bioreactor is equal to zero. Adsorption is defined as the process by which a solid holds molecules of a gas or liquid or solute as a thin film (Tro 2014). That means explicitly that no mass has adsorbed onto the walls of the tubing. Testing conducted as part of this thesis was conducted to prove or disprove that -rnet ads,   walls  equals zero. Fellow chemical engineering M.S. thesis candidate Princess Akande aims to demonstrate that -rnet ads,   reactor is also negligible. Additionally, chemical engineering M.S. thesis candidate Nicole Bueno to hopes further the mass balance accounting by measuring the mass of growing cells, despite the challenges associated with doing so over extended time in a salt water environment with mixing. This will quantify the relationship between the disappearance of glucose and the growth of the cells. Furthermore, when -rnet ads,   walls and -rnet ads,   reactor are calculated,  the calculation of the rate of disappearance of glucose term can be verified and thus validate the chemical engineering integrity of this project.  A complete list of mass and energy balances for the entire project is in Appendix 2.4.

2.5 – Instrumentation Specifications

- Exact drawing of sensor positions within bioreactor - Dimensions Sheet.xlsx

In this section specifying each of the pieces of instrumentation to be implemented into the tissue engineering test bed, this will be laid out as follows, with links in the report’s body to corresponding appendixes.  The password-protected http://my.fit.edu/~jbrenner links in subsections of Section 2.5 are copies of those from web site links in the references after each http://my.fit.edu/~jbrenner link, with fltech and jbrenner as the username and password, respectively.  Before using any of the Arduino-based sensors listed in this section, one must download the Arduino software from https://www.arduino.cc/en/Main/Software.  Arduino version 1.8.5 was used in this work and is stored at

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/arduino-1.8.5-windows.zip.

The dimensions for each of the sensors, controls, and other instrumentation are summarized in Table 2.5.1 below.  Additional specifications are in the subsections that follow and their appendixes.

Table 2.5.1: Sensors dimensions sheet

Before starting to acquire data or control any of the Libelium Open Aquarium sensors in Subsection 2.5.1, one must download openaquarium.zip and follow the directions given at

https://www.cooking-hacks.com/documentation/tutorials/open-aquarium-aquaponics-fish-tank-monitoring-arduino#step3

- Dimensions of the sensor and/or instrumentation to aid in the design of a system for sealing around that piece of equipment are in the directory:

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

- Data sheet (if available) specifying the input requirements for that instrument, as well as the voltage output (typically 0-5 VDC, or 4-20 mA converted to 1-5 VDC using a 250  resistor, are in the directory:

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

and

- Arduino code for each instrument are in an openaquarium subdirectory, even for sensors that are not from Libelium's Open Aquarium line:

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

- A diagram showing how the sensors are to be wired:

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

2.5.1 – Within Bioreactors

2.5.1.1 - Temperature

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

temperaturesensorArduino.txt

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

Open Aquarium - Aquaponics and Fish Tank Monitoring for Arduino.html

2.5.1.2 - Liquid Level

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

liquidlevel.txt

liquidlevelsee.txt

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

Open Aquarium - Aquaponics and Fish Tank Monitoring for Arduino.html

2.5.1.3 - pH

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

pHcalibrate.txt

pHgetdata.txt

pHsee.txt

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

Open Aquarium - Aquaponics and Fish Tank Monitoring for Arduino.html

2.5.1.4 - Electrical Conductivity

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

electricalconductivitycalibrate.txt

electricalconductivityread.txt

electricalconductivitysee.txt

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

Open Aquarium - Aquaponics and Fish Tank Monitoring for Arduino.html

2.5.1.5 - Dissolved Oxygen

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

DO_probe.pdf

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

DO_EZO_Datasheet.pdf

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

Arduino-Uno-DO-sample-code.pdf

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

how-to-properly-cut-probe-cables.pdf

2.5.1.6 - Oxidation-Reduction Potential

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

orp_probe.pdf, also seen in Figure A.2.5.1.6.1.

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

ORP_EZO_datasheet.pdf

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

Arduino-Uno-ORP-sample-code.pdf

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

ezo-orp-wiringdiagram.pdf

how-to-properly-cut-probe-cables.pdf

2.5.1.7 - Glucose Concentration

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

2.5.1.8 - Lactate Concentration

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

2.5.1.9 - Fiber Optic LED Microscope

Blah

2.5.1,10 - Impeller Rotation Rate

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

2.5.1.11 - Cell/Scaffold Mass

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

2.5.2 - External to Bioreactors

2.5.2.1 - Flow Totalizers

A diaphragm pump will be used for the prototype, for cost reasons; therefore, backpressure will be an issue as the filter cake builds up. This backpressure will slowly decrease the flow in the system, eventually affecting the rate of filtration. In order to account for this backpressure, a flow meter (Fig 2.5.2.1.1; DFRobot, 2019) was installed vertically before the filters. A relationship between differential pressure due to increasing cake resistance and flow rate was determined. Specifications for the flow totalizer are in Appendix A2.5.2.1.2.

Figure 2.5.2.1.1 – Hall effect water flow sensor used for flowrate monitoring (DFRobot, 2019)

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

flow_EZO_Datasheet.pdf

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

arduino_flow_sample_code.pdf

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

ezo-flow-wiringdiagram.pdf

2.5.2.2 - Differential Pressure Transducers

As the cells deposit in the filter, pressure at the effluent side of the filter starts to decrease. The pressure difference within the filter increases as time increases, affecting the efficiency of the rate of filtration. In order to monitor this pressure drop, there will be pressure transducers (Fig 2.5.2.2.1; Freescale Semiconductor, 2009) before and after the filters. A correlation study was conducted to match the differential pressure with the percentage of filter spent. The pressure transducer specifications are in Appendix A in Figures A2.5.2.2.1, A2.5.2.2.2, and A2.5.2.2.3.

Figure 2.5.2.2.1 – Differential pressure transducer (Freescale Semiconductor, 2009).

2.5.2.3 - Turbidity Sensors

Turbidity is the measure of transparency of a liquid. Typically, due to solids suspended within the liquid, the turbidity increases as the concentration of the solids increase. One should expect turbidity measurements to obey Beer’s Law. Turbidity will be used to monitor how many cells and/or solid waste are suspended within the recirculating fluid. In order to monitor this variable, a turbidity meter (Fig. 2.5.2.3.1; DFRobot, 2019) was placed before the filters. The turbidity was expected to slowly build up as the filter cake builds up. Once the force of gravity exceeds the attractive force of particle-particle interactions at the outer edge of the filter cake, the turbidity starts to increase significantly, and the filter will require backwashing. Specifications for the turbidity meter are in Appendix A in Figure A2.5.2.3.1.

Figure 2.5.2.3.1 – Turbidity sensor used for turbidity monitoring (DFRobot, 2019)

It is important to note that this turbidity sensor has difficulty reading lower turbidity values. This is due to the range of voltage that the sensor outputs. The sensor has an output voltage range of 3.3V – 5V, allowing only a range of 1.7V for measuring turbidity. In addition to this, the sensor was found to only be able to output a maximum voltage of 1.31V (DI water at 0.05 NTU).

2.5.2.4 – Capsule Filters

2.5.2.4.1 – Filtration Theory and Design Considerations

In order to fully understand the big picture of filtration, a closer look on the components and mechanisms involved within the filter while in operation is necessary. At a constant rate of flux, as solid particles get trapped by the filter membrane, the pressure drop across the filter and the osmotic pressure applied by any biosolids will start to increase (Chen, Fane, Madaeni, & Wenten, 1997). There are three distinct phases in the pressure drop progression that are observed during operation (Elmoe, Tricoli, Grunwaldt, & Pratsinis, 2009). The first phase is when particles are adsorbed into the pores, slightly increasing pressure drop. Once there are enough particles on the filter to cover the entire surface area, a “polarization” phase will take over during which particles start sticking to other particles, further increasing the differential pressure within the filter. Finally, at a critical thickness of solid cake formed, the smallest particles that would usually pass through the filter will get trapped in the cake, significantly increasing the pressure drop as illustrated by the Ergun equation (Erdim, Akgiray, & Demir, 2015):

-ΔPL=150µ1-ε2ε3Vdp2+1.75ρ1-ε2ε3Vdp  			     (2.5.2.4.1.1)

The Kozeny-Carman equation (Erdim, Akgiray, & Demir, 2015; Tien & Ramarao, 2013) is viewed as an improvement on the Ergun equation:

-ΔPL=180µΦs2dp21-ε2ε3V 		 			 	     (2.5.2.4.1.2)

where ε is the porosity of the bed, V is the velocity based on the empty cross-section of the bed, dp is the particle diameter, and Φs is the sphericity of the particles in the bed.

A comprehensive list of correlations relating pressure drop to the properties of the fibrous mesh filters (Fotovati, Hosseini, Tafreshi, & Pourdeyhimi, 2011; Benet, Badran, Pellegrino, & Vernerey, 2017) and/or the size distribution, the zeta potential, and the mechanical properties of the filtered solids (Iritani, Katagiri, Tsukamoto, & Hwang, 2014; Osterroth, Preston, Markicevic, Iliev, & Hurwitz, 2016; Erdim, Akgiray, & Demir, 2015) is found in Erdim, Akgiray, and Demir (2015). In particular, that paper shows that wall effect corrections to the Ergun and Kozeny-Carman equations are necessary, particularly at Re > 500, and are better represented by Equation 2.5.2.4.1.3:

fv=160+2.81Re0.904					      (2.5.2.4.1.3)

Whereas the aforementioned references are for particulate films growing on fibrous meshes, the viscous permeability, kr2, for molecules permeating through fibrous porous media have been best defined by Tomadakis where εp is defined as the percolation threshold (Tomadakis & Sotirchos, 1993; Tomadakis & Robertson, 2005):

kr2=ε8ln2εε-εpα+21-εpαα+1ε-εp2 					       (2.5.2.4.1.4)

A set of equations will be used to model the filters that will be put in place within the recirculation loop (Anirban Roy, 2015). These equations relate flux and pressure drop as well as filter and cake resistances while the filter is in operation.

J=ΔP-σπµ(RT) 							      (2.5.2.4.1.5)

Equation 5 shows the relationship between flux (J), pressure drop (ΔP), total resistance in the filter (RT), and an osmotic pressure term consisting of the osmotic reflection coefficient (𝝈) and the osmotic pressure of the polarized layer (𝝅) (Chen, Fane, Madaeni, & Wenten, 1997). Because the choice of diatomaceous earth—as a first, albeit inadequate, approximation for the particulate feed—has negligible osmotic pressure, the term was neglected in this work. The osmotic pressure term will need to be included when future students assess the effects of protein adhesion. For proteins, 𝝈 = 1 (Iritani, Katagiri, Tsukamoto, & Hwang, 2014).

The total resistance found within the filter can be broken down into the sum of the media resistance, cake resistance, and fouling resistance:

RT=Rm+Rc+Rf				      		      (2.5.2.4.1.6)

The media resistance within the filter can be found experimentally by running clean, distilled water through the system and measuring the pressure drop and the flow rate. Since there are no solids in the recirculation loop to filter out, neither the pressure drop nor the flow rate will change, allowing for an easy calculation for the media resistance as shown in Equation 2.5.2.4.1.7, presuming no osmotic resistance:

Rm=ΔPµJ				      			     (2.5.2.4.1.7)

where Rc and Rf=0

Fouling resistance is due to solid particles that were adsorbed in the fibrous filter media. These solid particles can get stuck inside the media in such a way that no amount of backwashing removes them. The resistance due to this irreversible fouling can be determined with the following equation:

Rf=ΔPµJ-Rm				      		      (2.5.2.4.1.8)

Once the solids get introduced into the system and start getting captured by the filter, the cake resistance increases over time, the pressure drop across the filter increases while the flux decreases. Since the resistance applied by the media stays the same throughout operation given a specific pressure drop and flux, the resistance made by the cake of solid can be determined:

J=ΔPµRm+Rc+Rf⟹Rc=ΔPµJ-Rm-Rf			       (2.5.2.4.1.9)

Mass balance accounting needs to be done on the filters for the solid concentration within the recirculating fluid and the concentration of available surface sites on the fibrous mesh. I will assume that the solids are of uniform size, charge, and reactivity. With these, the rate of filter usage can be determined. The mass balances are as follows:

VsuspdAdt=-r1+r-1-r2+r-2				       (2.5.2.4.1.10)

VsuspdAdt=Vsusp(-k1A*+k-1A*+AM*-k2AA*+AM*+k-2AM*) 	     (2.5.2.4.1.11)

dAdt=-k1A*+k-1[A*](1+fm)-k2A[A*](1+fm)+k-2[A*]fm       								(2.5.2.4.1.12)

where:

Ao=A*1+fmL-1f+A⟹A*=Ao-[A]1+fmL-1f	   (2.5.2.4.1.13)

f is the fraction of the maximum multilayer particle film that is currently occupied while fm is the fraction of sites that have more than one layer of particle on them. Substituting Equation 2.5.2.4.1.13 into Equation 2.5.2.4.1.12 and de-dimensionalizing Equation 2.5.2.4.1.12 yields:

AodAAodt=-k1AoAAoC*C+k-11+fm1-AAo1+fmL-1fAo-                                k-2AoAAo1-AAo1+fmL-1fAo+k21-AAo1+fmL-1fAo	 (2.5.2.4.1.14)

This mass balance evaluates the rate of change in the solid concentration within the fluid. It includes the rate of adsorption and desorption of solid particles to and from the fibrous mesh, as well as from other solid particles. These equations also account for any extra layers of solid particles over the initial film.

Similar to the mass balance on the concentration of suspended solid in the fluid, the mass balance on the concentration of available surface sites on the filter also contains the rate of adsorption and desorption as defined in the following equations:

Boundary Conditions:

ft=∞=1
ft=o=0
At=0=Ao ⟹AAot=0=1
*t=0=C
fmt=0=0

α*d*dt=-r1+r-1				       		          (2.5.2.4.1.15)

where: α=ρsphere(43πrsphere3)4πr2, assuming spherical particles

α*d*dt=-k1A*+k-1A*+AM*		          (2.5.2.4.1.16)

α*d*dt=-k1AoAAo*+k-1A*1+fm		          (2.5.2.4.1.17)

α*[C]d*Cdt=-k1AoAAo[C]*C+k-1[C]A*C(1+fm)    (2.5.2.4.1.18)

The set of equations for multilayer adsorption does not lead to an analytical solution.  For the single layer case (fm≈0), the film doesn’t build up into multilayers, and L≤1:

Keq=k1[A]eq[*]eq[C][C]k-1A*eq+AM*eq			      		          (2.5.2.4.1.19)

At equilibrium, for the case where there is no multilayer particle film formation:

k1A*=k-1A*⟹k-1k1=A*A*=Keq			          (2.5.2.4.1.20)

d[A]Aodt=-d[*][C]dt						          (2.5.2.4.1.21)

[A]Ao-AoAo=-*C-CC			       		          (2.5.2.4.1.22)

2-AAo=*C							          (2.5.2.4.1.23)

From Equation 18, for the case of no multilayer adsorption:

[C]d*Cdt=-k1AoAAo[C]*C+k-1[C]A*C		          (2.5.2.4.1.24)

d2-[A]Aodt=-k1AoAAo2-[A]Ao+k1Keq-1-[A]Ao	          (2.5.2.4.1.25)

-dAAodt=-k1Ao2AAo+k1AoAAo2-k1Keq-k1KeqAAo   (2.5.2.4.1.26)

dAAodt=-k1AoAAo2+(2k1Ao+k1Keq) AAo+k1Keq	          (2.5.2.4.1.27)

dxaAAo2+bAAo+c=-k1Ao dt				          (2.5.2.4.1.28)

where:

a=1; b=-1+KeqAo; c=-KeqAo

Upon integration, for the case with no adsorption of particles on other particles:

1a(p-q)ln{qpAAo-pAAo-q}=-k1Aot			       	          (2.5.2.4.1.29)

where p=-b-b2-4ac2a and q=-b+b2-4ac2a

Of the different types of cell death, necrosis is the one requires the most attention because it affects the filtration system of the tissue engineering test bed the most. Cells undergoing necrosis swell, then rupture, releasing their contents out to the environment (Kroemer, et al., 2009). It is important to remove this dead cell debris from the media because of the inflammatory response it stimulates on nearby healthy cells (Rock & Kono, 2008). Inflammation of healthy cells can be a detriment by impairing the natural healing process of the tissue or, in some cases, killing healthy host cells (Martin & Leibovich, 2005).

This nanosized cellular debris can permeate through most of a particular film built upon a porous fibrous mesh before completely blocking the pore. This phenomena, called biofouling, can cause major problems to the filtration system by reducing membrane flux through the filter (Abd El Aleem, Al-Sugair, & Alahmad, 1998) or increasing the pressure drop across the system, affecting filtration performance (Herzberg & Elimelech, 2007). Another significant contributor of biofouling in the filter membrane is the adhesion of extracellular polymeric substances (EPS) such as polysaccharides, proteins, nucleic acids, and lipids (Nguyen, Roddick, & Fan, 2012).

2.5.2.4.2 - Capsule Filter Specifications

A 5-micron pleated capsule filter (Fig. 2.5.2.4.2.1; EBay capsule filter, 2019) is to be used for the filter in this thesis and installed horizontally. In order to maximize operation efficiency and reduce the amount of time a bioreactor is down for maintenance, there will be two capsule filters installed in each recirculation system in parallel. With a pore diameter of 5 microns, it will be assumed that most, if not all, of the cells and accumulated within the bioreactor system will be filtered out. Finer filters of 0.22-, 0.45-, and/or 2-micron diameter are roughly 10 times more expensive and would likely not filter out proteins released until the proteins have aggregated some. The construction of this filter allows backwashing by switching the influent and effluent sides of the filter and have liquid flow opposite of the filter’s normal flow pattern.

Figure 2.5.2.4.2.1 – 5-micron capsule filter (EBay capsule filter, 2019)

2.5.2.5 – Diaphragm Pumps

In order to make this thesis, and by extension the entire project, a fiscally viable, cost-effective pump solution is required. As such, a diaphragm pump (Fig. 2.5.2.5.1; EBay diaphragm pump, 2019) was selected as the pump for the recirculation loop. The pumps to be used in this thesis can pump water at a rate of ~25 mL/sec (1.5 L/min) without any resistances. When installed in the recirculation loop, the flow rate reduces to ~15 mL/sec (0.9 L/min). This is due to all the resistances introduced by the filter, the connections, and the increased length of tubing.

Figure 2.5.2.5.1  – Diaphragm pump (EBay diaphragm pump, 2019)

2.5.2.6 – Diatomaceous Earth

In order to successfully simulate the operation of the bioreactor with a recirculation loop, solids that have the same range of particle sizes as cells are paramount. For this thesis, diatomaceous earth will be used to mimic the presence of cells in the system. Diatomaceous earth is expected to behave similarly to cells and solid wastes as it gets filtered out of the system. The bag of food grade diatomaceous earth is made by DiatomaceousEarth and was purchased from amazon.com.  As purchased, the diatomaceous earth has a lognormal distribution of particle sizes ranging from 1 µm to over 100 µm (Dragan Petrovic, 2011). Because some of the smallest particles got trapped in the 5-micron filter media mesh in the first runs, the runs reported in later chapters were for diatomaceous earth that had been meshed with a 270 mesh (50 m) screen.  More detail about the diatomaceous earth (Appendix A2.5.2.6), glucose (Appendix A2.5.2.7), polyethylene glycol (Appendix A2.5.2.8), and model protein (Appendix A2.5.2.9) for protein adhesion studies are in the appendixes.

2.5.3 - For Feed Tanks

2.5.3.1 - Peristaltic Pumps

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

EZO_PMP_Datasheet.pdf

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

peristalticpump.txt

Arduino-Uno-PMP-sample-code.pdf

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

2.5.3.2 - Syringe Pumps

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

2.5.3.3 - Feed Tanks

2.5.4 - Aquarium Level

2.5.4.1 - Colorimetric Imaging of Unwanted Biofilm Growth

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

2.5.4.2 - UV LED Sterilization

2.5.4,3 - Heaters

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/dimensions/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/datasheet/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/wiring/

2.6 – Bioreactor Process Flowsheet

Figure 2.6.1 – Bioreactor process flow diagram

Figure 2.6.1 shows the process flow for the recirculation unit of each bioreactor in the tissue engineering test bed design. The fluids inside the bioreactor is pumped out and into a tee where the flow can either be directed to the recirculation filters or to a purifier at the end of the system. There is a flow meter installed after the pump to measure the rate of flow throughout operation. A turbidity meter is also in place to monitor the concentration of solids within the fluid. When the flow is valved to the recirculation loop, it is split again into two streams, one for each filter. Although there will be two filters installed for each bioreactor, only one will be in operation at any given time, while the other one is available for backup when maintenance is required on the primary filter. Valves will be in place before and after each filter for isolation. Pressure transducers will be installed on the feed and effluent ends of the filters to measure pressure drop between the two points. A full piping & instrumentation diagram (P&ID) analysis is in the next section.

2.7 – Piping and Instrumentation Diagram

The schematic designs of the plumbing and instrumentation diagrams for the tissue engineering test bed are shown in the P&IDs of Section 2.7 as follows. In fulfillment of the automation requirements of this tissue engineering test bed, a proper oxygenation of feedstocks system (later in Figure 2.7.3.1), feedstock reservoir distribution to thirty six bioreactor inputs (later in Figure 2.7.3.2 and Figure 2.7.3.3), thirty-six instrumented bioreactors (example later in Figure 2.7.2.1), waste collection from the thirty-six bioreactors (Figure 2.7.4.1), purification system (Figure 2.7.4.1), and return system (Figure 2.7.4.1) are all schematically designed.  The accompanying mass balance (Sections 2.4 and 2.9) and stream numbering for all the streams in the tissue engineering test bed are accounted.  The master Visio file that contains all the piping and instrumentation diagrams (P&ID's) is VisioBioreactorReal.vsdx.

This section describes the piping and instrumentation design for the tissue engineering test bed. This is a crucial part of the foundation for coding the test bed’s data acquisition and control system in LabView. All control instruments in this tissue engineering test bed will transmit signals to LabView programming via computer connection for controlled decisions. Likewise, for all the valves, a transmitter will send signals to the LabView code. In both instances, LabView will make controlled decisions. Those same instruments and valves will be energized though LabView programming using relays to solenoids to control instrument and valve operations.

On each ten-way valve, one can only control seven inputs or outputs because of electrical connection limitations.  Moreover, to have so many bioreactors in parallel, one input or output has been reserved for "daisy chaining" to the next 10-way valve.

2.7.1 – Within Bioreactors

There is no piping within each bioreactor to describe except for the inlet and outlet penetrations.  Instrumentation within each bioreactor was included individually in Section 2.5.1 and is included as a group within Figure 2.7.2.1.

2.7.2 – External to Bioreactors

The P&ID for an individual bioreactor and its recirculation system is shown in Figure 2.7.2.1. For all valves pictured, a transmitter will send signals to the LabView code for controlled decisions. LabView relays to a solenoid on each valve to perform an operation. Not pictured are the solenoids of all the valves.

Streams Around Each Bioreactor

Stream numbers for piping around the bioreactors have four digits.  The two digits refer to where the stream is with respect to the bioreactor. The last two digits refer to the bioreactor 01-36 as outlined as follows. For example, streams around Bioreactor-01 (last two digits = 01):

- Feed before 1st valve  	01-01

- After 1st valve + before 1st mixing pt.  	02-01

- Above  2nd valve + below 1st mixing point  	03-01

- After 1st mixing point = 	04-01

- Above 2nd mixing point, below 3rd valve  	05-01

- Bioreactor feed  	06-01

- Bioreactor exit  	07-01

- After bioreactor + 4th valve, before pump  	08-01

- After pump, before flow meter 1 	09-01

- After flow meter 1, before  turbidity meter 	10-01

- After turbidity meter, before 3rd mixing point	11-01

- After 3rd mixing point, before 5th valve	12-01

- After turbidity meter, going up from 4th mixing point (a + b)	13-01

- After 1st vertical valve +  before capsule filter 1 (a + b)  	14-01

- Waste stream from capsule filter (a + b)	15-01

- After capsule filter 1 filter (a + b)	16-01

- After 6th valve and before 2nd flow meter (a + b)	17-01

- After 2nd flow meter and before 2nd mixing point 	18-01

- Exit to concentration analyzer  	19-01

- Return from purifier 	20-01

Figure 2.7.2.1: P&ID of bioreactor external recirculation loop

There are several sequences of the bioreactor operation. At first, all valves are closed. The first sequence is filling the bioreactor (R-01). LabView relays to the solenoid of valve 1 to open the valve. Fresh feed is pumped into R-01 by the syringe pumps via streams 01-01, 02-01, 04-01, and 06-01. When R-01 has reached a sufficient level, the level sensor in R-01 transmits a signal to LabView where LabView makes the controlled decision to close valve 1. LabView relays to the solenoid of valve 1 to close the valve and stop filling R-01.

Now that R-01 is at a sufficient level, Sequence 2, recirculation of R-01 can begin. LabView relays to the solenoid of valve 1 to close the valve and the solenoids of valves 2, 3, 4, and 7 to open the valves. A diaphragm pump (P-01) recirculates the reactor contents through to the primary filtration system of the recirculation loop to mimic aspects of blood circulation. The path of low through recirculation loop is through streams 07-01 – 11-01, 13-01 – 17-01, and 06-01 back into the R-01. When the liquid becomes sufficiently turbid because of too much accumulation of entrained solids, such solids will be filtered through a 5 micron capsule filter (CPF-01) into a solid waste tank that will need to periodically emptied as described in the M.S. thesis work of Jonah Melegrito (2019).

When the capsule filter is too contaminated, a large pressure drop is measured across it by the two pressure transducers (PPT-01 and PPT-02). This means the CPF-01 is compromised. If this happens, the pressure transducers will transmit a signal to LabView where LabView makes the controlled decision to close valves 3 and 4 and open valves 5 and 6. LabView relays to the solenoids of valves 3 and 4 to close valves 3 and 4 and the solenoids of valves 5 and 6 to open  valves 5 and 6, switching the flow out of the bioreactor to a parallel capsule filter (CPF-01b) for the same filtering purposes within the recirculation loop. The rest of the flow through the recirculation loop will remain unaltered.

The control action within each bioreactor will affect the following sensors within each bioreactor: oxygen reduction potential, pH to test acidity, dissolved oxygen (DO) to ensure adequate oxygenation, a level sensor to control reactor liquid volume, load cells to measure the mass of cells and the reactor contents, electrical conductivity (EC) for salt concentration control, and glucose concentration to sustain cells. As an example, when any sensor besides temperature in R-01 transmits a signal to LabView, LabView makes the controlled decision to pump the required feedstock to satisfy the particular need of R-01. LabView relays to the solenoid of valve 1 and the solenoid of the appropriate syringe pump to open the valve and begin filling R-01with the appropriate feedstock.

Sequence 3 is sending liquid from the bioreactor to the secondary filtration system. When the liquid level within the R-01 is too high or if both capsule filters are compromised, control action sends waste to the purifier ("kidney"- K-01) common to all bioreactors through stream 19-01. LabView relays to the solenoids of valves 3 or 5, depending on which filter path is in use, to close the respective valve and the solenoids of valve 8 to open the valve to allow fluid to flow through the effluent of R-01 to common waste collection system.

Sequence 4 of bioreactor operation is recycling fluid into the feed. At first all valves are closed. LabView relays to the solenoid of valve 9 to open the valve. Recycled feed is pumped from the purification system to R-01 through stream 19-01.

2.7.3 – For Feed Tanks

The oxygen add-in system (Figure 2.7.3.1) allows for the saturation of feedstocks with oxygen. The valves of the oxygen add-in are numbered 1-8. There are several sequences that take place in the oxygen add-in system. Initially, all valves are closed. For the first sequence, the bubbler tank (RBT-01) is filled. LabView relays to the solenoids of valves 1 and 2 the open the valves. Feedstock is pumped into the oxygen add-in via peristaltic pump (PP1-01) from stream 501 through stream 507.

Once RBT-01 is sufficiently filled, the feedstock is oxygenated in the second sequence. Feedstock is pumped out of the RBT-01 and through a recirculation loop that circulates the fluid through RBT-01 via peristaltic pump PP1-01, alternating between counterclockwise flow from scream 508-509 then stream 504-507, and clockwise flow from stream 507-504 and then from 509-508. LabView relays to the solenoids of valves 2 and 3 to open the valves and the solenoids of valves 1 and 4 to close the valves. Oxygen is added in to the RBT-01 to oxygenate the recirculating feedstock, while gas is simultaneously released. Oxygen feed flows through streams 512-514 and gas is released through streams 515-517. While the feedstock is recirculated, LabView relays to the solenoids of valves 5 and 6 to open the valves to add oxygen. LabView also relays to the solenoids of valves 7 and 8 to open the valves to release gas. Oxygenating the feedstock in this manner is a method common for zeta potential analyzers.

Figure 2.7.3.1: Oxygen add-in system P&ID

Once the feedstock is saturated, Sequence 3 initiates, sending the oxygenated feedstock to the syringe pumps and feedstock reservoirs. LabView relays to the solenoids of valves 5 and 6 to close the valves of the oxygen feed stream, and LabView relays to the solenoids of valves 7 and 8 to close the valves of the gas release stream. Then LabView relays to the solenoids of valves 2 and valve 4 to close valve 2 and open valve 4. and allow PP1-01 to pump the contents of streams 509, 504, 505-507, and the RBT-01 to the syringe pumps through streams 508-509, 504-505, and 510-511.

From the oxygen add-in system, liquid feedstock flows to the syringe pumps and feedstock reservoirs. The feedstocks are connected to common VICI ten-way valves shown previously in Figure 1.3.1. Figure 2.7.3.2 shows the flowsheet of the various feedstocks flowing through these ten-way valves.

The feeds will include: ultrapure water for purges, phosphate buffered saline (PBS) for normal operation, a place holder for sterilization and concentration calibrations, glucose for feeding, ethyl lactate as a water-rinsible degreaser and typical of a cellular waste product, dyes for imaging, fluorescein isothiocyanate (FITC) for staining, Alamar Blue to measure cell proliferation and cytotoxicity, Phalloidin for conjugated staining of actin proteins with FITC, Alizarin Red S for histological characterization of calcium, growth and differentiation factors for cell culture, and six placeholders for future research work at Florida Tech that might include drugs for toxicological testing.

The P&ID for the same section is shown in Figure 2.7.3.3. For all the syringe pumps and VICI 10-way valves pictured, a transmitter will send signals to the LabView code for controlled decisionmaking. LabView relays to a solenoid on each instrument to perform an operation. Not pictured are the solenoids of all the syringe pumps and VICI 10-way valves.

LabView relays to the solenoids of syringe pump 1 (S-101) to initiate the pumping of feedstock to the VICI 10-way valve V-100. LabView also sends a signal to the solenoid of the V-100 to toggle the position of the VICI 10-way valve to align the common output stream 008 with a subject feedstock. The subject feedstocks of the feedstock reservoirs connected to S-101 are pumped to the array of bioreactors by S-101 through streams 008, 015, 022, and common distribution stream 123.

Likewise, LabView relays to the solenoids of syringe pump 1 (S-102) to initiate the pumping of feedstock to the VICI 10-way valve V-101. LabView also sends a signal to the solenoid of the V-101 to toggle the position of the VICI 10-way valve to align

Figure 2.7.3.2: Flowsheet of feedstocks via syringe pump reservoirs

the common output stream 015 with a subject feedstock. The subject feedstocks of the feedstock reservoirs connected to S-102 are pumped to the array of bioreactors by S-102 through streams 015, 022, and common distribution stream 123.

Finally, LabView relays to the solenoids of syringe pump 1 (S-103) to initiate the pumping of feedstock to the VICI 10-way valve V-102. LabView also sends a signal to the solenoid of the V-102 to toggle the position of the VICI 10-way valve to align the common output stream 022 with a subject feedstock. The subject feedstocks of the feedstock reservoirs connected to S-103 are pumped to the array of bioreactors by S-103 through streams 022 and common distribution stream 123.

The feeds will be distributed to the bioreactors via multiple VICI 10-way valves. Figure 2.7.3.3 shows the distribution from the common output of feedstocks, stream 123, to the first six bioreactors as well as stream 124, which daisy chains to the next distribution valve. The inputs ultimately reach a control volume around each bioreactor. Though the scope of this thesis work does not directly integrate with the bioreactors, it is imperative to the overall design, construction, and implementation to know the design, functionality, and material selection.

Figure 2.7.3.3: P&ID connecting feedstocks to common output

The piping and instrumentation diagram level of detail for the feed tanks is given in Figure A2.7.3.1. Common output stream 123 from 10-way valve V-103 connects to bioreactors 1-6 via positions 1-6 and streams 01-01 - 06-01 on 10-way valve #4.  Position 7 (stream 124) will be daisy-chained to 10-way valve V-104 input so that it can be sent to bioreactors 7-12 on positions 1-6 via streams 07-01 - 12-01.  Position 7 on 10-way valve V-104 (stream 125) is further daisy-chained to 10-way valve V-105 input so that it can be sent to bioreactors 13-18 via positions 1-6 on 10-way valve V-105 and via streams 03-01 - 18-01.  Position 7 on 10-way valve V-105 (stream 126) is further daisy-chained to 10-way valve V-106 input to bioreactors 19-24 on positions 1-6 via streams 19-01 – 24-01.  Position 7 (stream 127) will be further daisy-chained to 10-way valve V-107 input and connects to bioreactors 25-30 via positions 1-6 on 10-way valve V-107 via streams 25-01 – 30-01.  Position 7 (stream 128) will be further daisy-chained as input to 10-way valve V-108 to connect to bioreactors 31-36 via positions 1-6 and via streams 31-01 – 36-01.  Position 7 on 10-way valve #9 will not be used. Positions 8 through 10 on each 10-way valve will not be used because the National Instruments hardware discussed in Section 3.1 can only send 8 “on” / “off” signals (0 = all off; 1-7 = on for reactors 1-7, respectively).

2.7.4 – Aquarium Level

From the array of bioreactors, there is a VICI ten-way valve that sends the bioreactor outputs (Figure 2.7.4.1) to a common secondary concentration analyzer and a liquid purifier (K-01), similar to a kidney. The secondary concentration analyzer (A-01) will give readings for the exit concentration from each bioreactor to confirm the concentration analysis from each bioreactor’s concentration analyzer before sending the purified liquid back to the bioreactors. Independent secondary concentration analysis is likely to be a high performance liquid chromatographic (HPLC) separation to be done by future students in Dr. Brenner’s research group. Figure 2.7.4.1 shows the flowsheet of this section of the test bed, and Figure 2.7.4.2 shows the piping & instrumentation diagram of the same section.

Figure 2.7.4.1: Flowsheet of output streams flowing through common filtration system

Figure 2.7.4.2: P&ID of purification system. Stream 234 flows into concentration analyzer A-01 and outputs to the purifier (kidney) K-01 through stream 335. Waste from the kidney exits via stream 336 while recycled fluid flows back to A-01 through stream 337 for re-analysis. Fluid will either be sent back to K-01 via stream 335 or, if acceptable, it will flow out of A-01 via stream 438 for redistribution.

Figure 2.7.4.2 shows stream 234 flowing into concentration analyzer A-01 and outputting to the purifier (kidney) K-01 through stream 335. Waste from the kidney exits via stream 336 while recycled fluid flows back to A-01 through stream 337 for reanalysis. Either fluid will be sent back to K-01 via stream 335 or, if acceptable, it will flow out of A-01 via stream 438 for redistribution.

The piping and instrumentation diagram information for this section is seen in Figure A2.7.4.1 in Appendix A2.7.4. Outputs from bioreactors 31-36 will go to positions 1-6 on 10-way valve V-109 as inputs via streams 19-31 - 19-36.  The common output from 10-way valve V-109 will go to position #7 on 10-way valve #11 via stream 229.

Outputs from bioreactors 25-30 will go to positions 1-6 on 10-way valve V-110 as inputs via streams 19-25 – 19-30.  The common output from 10-way valve V-110 will go to position #7 on 10-way valve #12 via stream 230.

Outputs from bioreactors 19-24 will go to positions 1-6 on 10-way valve V-111 as inputs via streams 19-19 – 19-24.  The common output from 10-way valve V-111 will go to position #7 on 10-way valve #13 via stream 231.

Outputs from bioreactors 13-18 will go to positions 1-6 on 10-way valve V-112 as inputs via streams 19-13 – 19-18.  The common output from 10-way valve V-112 will go to position #7 on 10-way valve #14 via stream 232.

Outputs from bioreactors 7-13 will go to positions 1-6 on 10-way valve V-113 as inputs via streams 19-07 – 19-12. The common output from 10-way valve V-113 will go to position #7 on 10-way valve #14 via stream 233.

Outputs from bioreactors 1-6 will go to positions 1-6 on 10-way valve V-114 as inputs via streams 19-01 – 19-06.  The common output from 10-way valve V-114 will go to the secondary concentration analyzer and then waste via stream 234.

Kidney Purification

As seen previously in Figure 2.7.4.2, the daisy-chained output from 10-way valve #15 in stream 234 enters the concentration analyzer and outputs to stream 335, which enters the purifier kidney. Stream 336 exits the kidney to waste, whereas stream 337 exits the kidney and re-enters the concentration analyzer.

Return Distribution

As seen in Figure A2.7.4.2, stream 438 exits the concentration analyzer and enters the recycle distribution back to all 36 bioreactors.  Common output stream 438 goes through 10-way valve V-115 and connects to bioreactors 1-6 on valve positions 1-6 via streams 20-01 - 20-06.  Position 7 (stream 439) is daisy-chained to 10-way valve V-116 so that it can be sent to bioreactors 7-12 on positions 1-6 via streams 20-07 - 20-12.  Position 7 (stream 440) is daisy-chained to 10-way valve V-117 so that it can be sent to bioreactors 13-18 on positions 1-6 via streams 20-13 - 20-18.  Position 7 (stream 441) is daisy-chained to 10-way valve V-118 to bioreactors 19-24 on positions 1-6 via streams 20-19 – 20-24.  Position 7 (stream 442) is further daisy-chained to 10-way valve V-119 to bioreactors 25-30 on positions 1-6 via streams 20-25 – 20-30.  Position 7 (stream 443) is daisy-chained to 10-way valve V-120 to bioreactors 31-36 on positions 1-6 via streams 20-31 – 20-36.  Position 7 on valve V-120 will not be used.  A P&ID for the entire tissue engineering test bed is in Figure A2.7.4.3.

2.8 - HAZardous OPerability (HAZOP) Analysis

Given the complexity of the experimental apparatus, a hazardous operability, or HAZOP, study was merited.  A partial example of the HAZOP study is shown in Table 2.8.1. An incomplete, but more thorough example than Table A2.8.1 is in Appendix 2.8. HAZOP studies involve applications of each of the process variables and key words or phrases such as "too high", "too low", "not", etc.  For example, if there is not enough flow through the flowmeter for the recirculation loop, then the peristaltic pump will eventually burn out.  Such a lack of recirculation will kill any cells eventually as well.  Moreover, the lack of recirculation is a likely indicator that there is an obstruction in the system causing a large pressure drop.  The current HAZOP study covers many, but certainly not all, of the potential hazards associated with this work. Table 2.8.1 shows how the group will approach the problem — starting with identifying the problem, then discerning probable causes for each problem, and finally finding out possible solutions to each of the identified problems.

Table 2.8.1 – Hazard and Operability Analysis

1. List of control volumes, unit operations, etc.

a. All mixing points (create list that refers to stream #’s)

b. 10-way valves with common inputs (going from O2 bubble trap to 			bioreactors)

c. 10-way valves with common outputs (going from feedstocks to O2 bubbler, 		going to common waste from bioreactors)

d. 2-way valve

e. Solid waste filter

f. Pump and flow totalizer

g. Feed tank

1. Ultrapure water

2. Phosphate buffered saline (PBS)

3. Ethylene oxide

4. Glucose

5. Ethyl lactate

6. Blue food coloring

7. Red food coloring

8. Fluorescein isothiocyanate (FITC)

9. Alamar blue

10. Phalloidin

11. Alizarin red S

12. Growth factor

13. Differentiation factor

2. Sublist of everything that go wrong for each control volume

a. Clog at any fitting or of any section of tube

Characterized by low flow or high pressure

If small problem, then check to see if liquid or solid waste filters are 		malfunctioning.  If so, then try to relieve the excess pressure by

emptying out relevant filters.  If unable to do so, take that section of 		tubing and control volumes that that section affects offline.  Go to 			Section 5.  If larger problem, then take that section of tubing and 			control volumes that that section affects offline.  Go to Section 5.

b. Leak out of system

Sense leak out of aquarium with Open Aquarium liquid leak sensor.  If 		sensed, go to Section 5.

c. Too low a temperature

If any temperature reads 2oC below the external room ambient 			temperature other than the temperature of cooling water, then that 			thermocouple is in need of recalibration.  Get to a safe condition, and 		go to Section 5.  If the temperature of cooling water < -2oC, then that 		thermocouple is in need of recalibration.  Get to a safe condition, and 		go to Section 5.

d. Too high a temperature

If a temperature reads above 43oC, get the bioreactor or aquarium to a 		safe condition and proceed to Section 5.  If the temperature reads 			above 43oC and goes up by more than 2oC per minute, then the 			thermocouple has an open circuit.

e. Biofilm growth anywhere it doesn’t belong

This can either be detected visually by the user, by accumulation of 		mass in the system in places where it should not occur, or by 			colorimetric detection by an imaging camera.  In any of these cases, 		the ...

f. Solid waste accumulation in places it doesn’t belong

g. Solid waste filter cannot be emptied

h. Liquid waste filter doesn’t filter correctly

This may be assessed by a liquid level controller, or by a user or 			imaging camera detecting measurable color changes at unexpected 			times.

i. Solid waste filter doesn’t filter correctly

This may be assessed by a level controller in each solid waste filter, or 		by a user or imaging camera detecting measurable color changes at 		unexpected times.

j. Gas bubbles

k. Leak through valve

l. Syringe pump jammed

m. Power outage

n. Robotic arm malfunction

o. Wires get electrically shorted

p. Recirculation pump

q. System no longer sterile

r. Cells don’t grow

s. Load cell not sensitive enough

t. Microscope imaging not sensitive enough without dyes

u. Dyes damage samples over long term

v. Microscope imaging not sensitive enough even with dyes

w. Computer/sensor no longer communicating

x. Valve actuation not working correctly

3. Categorized set of responses for high-high, high, normal, low, and low-low 	conditions

4. List of what must be done to get situation to safe condition

a. If necessary, turn on backup power & estimate how long backup power will 		last before it fails

5. Actions that ought to be done to save data, record unsafe condition, etc.

a. Get relevant sections to safe condition

b. E-mail, text, and call Dr. Brenner and relevant students

c. Write sensor reading that prompted "unsafe" report to run log notes

d. Save file up to that point

e. Try to assess cause of unsafe condition

f. Shut down system, if necessary, when safe condition met

2.9 – National Instruments LabView Code

Streams Around Each Bioreactor:

Stream numbers have three digits.  The first digit refers to where the stream is with respect to the bioreactor, as outlined in the example below.  The last two digits = 22 + Bioreactor # + # of Common Outputs that are required to get to that bioreactor.  Bioreactor #1 is off of the first 10-way valve, and therefore has one common output, so the last two digits for each stream = 22 + 1 + 1 = 24.  One has to go through six common outputs (five levels of nesting) to get to Bioreactor 36, so the last two digits of stream numbers associated with Bioreactor 36 = 22 + 36 + 6 = 64.

For example, Bioreactor #1:

Fresh Feed to Bioreactor = Stream 124;

Reactor Feed = Fresh Feed + Recycle Feed = Stream 224;

Recycle Feed (from Flow Totalizer) = Stream 324;

Reactor Effluent = Liquid Waste + Recycled Effluent = Stream 424;

Recycled Effluent goes to Solid Waste Filter = Stream 524;

Liquid Exiting Solid Waste Filter goes to Peristaltic Pump = Stream 624;

Solid Exiting Solid Waste = Stream 724;

Liquid Exiting Peristaltic Pump goes to Flow Totalizer = Stream 824; and

Liquid Waste = Stream 924.

hR=Height of Fluid in Bioreactor

lR=Length of Bioreactor

wR=Width of Bioreactor

hfilter=Height of Solid in Solid Waste Filter

dfilter=Diameter of Solid Waste Filter

The algebraic steady state mass balances for the mass flow rates on each stream surrounding the bioreactors are as follows in Equations X-XX:

mstream #=Mass Flow Rate of Stream #

m124=Syringe Feed Rate

m324=Peristaltic Pump Recirculation Rate

m224=m124 + m324

m524=-m324

m924=-m124

m824=m324

m624=m824

The differential equations on the height of fluid in the bioreactor and the height of solid in the solid waste filter are not difficult to write out mathematically (Eqns. X-XX).

dhRdt = m224+m424/(liquid*lR*wR)	Differential mass balance on bioreactor

IF (hR>hR,set) THEN m424=m924+m524 ELSE (0)	Reactor fills up to set height

(hRt= 0)=0 				Bioreactor initial condition

dhfilterdt = m524+m624-m724/(solid**dfilter24)

Differential mass balance on solid waste filter

IF (hsolid>hsolid,set) THEN m724 ≠0 ELSE (0)	Solid waste filter emptying

(hsolidt= 0)=0 				Solid waste filter initial condition

In terms of data collection, these heights must be collected periodically, and their derivatives must be calculated by approximating the slope of height vs. time over the last several data collection increments.  The worst case from a safety standpoint would correspond to overfilling the liquid in a reactor; one of the more likely times that this might happen is during startup.  Data collection rates at an interval of one second are not difficult to achieve, and as the ratio of fresh feed flowrate to the recirculation flow rate, to the size of the bioreactor, and to the possible range of mixing rates are quite low, achieving liquid level and temperature control within the reactors should not be challenging for this research.  Dr. Brenner's long term goal of miniaturizing the bioreactor volumes via the use of microfluidics, for which flowrates are in the laminar flow regime and for which mixing is almost nonexistent, means that liquid level and temperature control will have to be re-examined when such miniaturization is undertaken.

Solid waste removal from the human body occurs in roughly once per day intervals on the order of 1% of one's body mass per day.  As most of what winds up as solid waste is indigestible food, the percentage of solid waste in the system being constructed should be nearly zero.  Babies do not even get pureed food until several months after birth.  The tissues to be developed in this study will be neonatal.  Dulbecco's modified eagle medium (DMEM), the medium used for most cell cultures, is primarily a glucose solution and contains no solids.  The only sources of solids inside a neonatal culture would be either a) results of condensation/gelation of food sources, perhaps such as mucopolysaccharides experienced during nasal congestion or b) sloughing off of dead tissue due to apoptosis or necrosis.  As the dynamics of filling the solid waste filter should be zero or at least very slow, experimentally it may be difficult to compute the derivative of height of the solid waste filter with respect to time if the height data is somewhat noisy.  To help calculate derivatives for both mass balances, the last ten heights will be sent to BioreactorMassBalance.vi.

- List all subroutines that must be executed

a. Syringe pumps(TIM)

- Requirements:

  - What we need this VI to accomplish

    - Send Digital signal to Syringe Pumps (pulse on, pulse off)

    - Incorporate a timer

    - Show current amount of volumes of each solution( a visual and numeric indicator for each type of solution?)

    - Send error message/warning signal if any amount of fluid is low either to controlling computer or via text

- While loop with a timer? Condition could be tied with residing volume of solution,  or readings from other sensors.

- This VI is tied with the following sub VI’s:

  - pH reading

  - Glucose Reading

  - Lactate reading

- VI outline

  - Previous loop would send data of volume to next iteration

  - Previous loop will send sensor data to next iteration

  - Loop should send out a warning if any amount of fluid is low

  - First, have a constant to display the initial volume in the syringe pumps. (This is either known or measured by some device)

  - This constant will be used just once to initialize both the numeric and visual display of volume in the syringe pumps

B. Valve positions(TIM)

- Activates at a set-time

- With  a time delay built in to allow each valve sufficient time to rotate

C. Individual bioreactor data acquisition and control

A-D,H,I (open aquarium)

E,J,K (atlas scientific)

External microscope sensor (ebay)

- Flowchart for how subroutines will connect to master wiring diagram

- Flowcharts for individual subroutines (inputs, outputs)

- Write code for individual subroutines

- Compose layout of Master Front Panel

  - Bioreactor (1,2…..36)

  - Sensor Readings

    - Temperature(NI)

    - Liquid Level(NI)

    - pH(NI)

    - Conductivity(NI)

    - Dissolved oxygen

    - ORP(Oxidation Reduction Potential)

    - Glucose

    - Lactate

    - Rotation Rate

    - Flow Totalizer

    - Syringe Pumps

  - Schematic Diagram

  - Latest image of cells

  - Mass of cells on the Substrate

- Convert P + ID/ Flowsheet to write mass/energy balances(FIRST DO IN EXCEL) (Jonah/Adrien)

  - Mix points

  - Unit Ops

  - Partial sections of the entire process

  - Overall global mass/Energy

  - Code should write output from Instrumentation, into a format consistent with the excel spreadsheet(using strings)

Should have data for each time increment, each time reactor, and sensor. Will write to a file every so many loops

Loop Layout

For each time increment we record data

For each bioreactor

For each sensor

Write one line that includes sensor output

- Figure out how many instruments we need

- Calculations of unmeasured mass + energy balance (FIRST DO BY HAND)

- Has to include surroundings as well

- Write code for Master Wiring Diagram

- Write Code for calculating the accumulation of “stuff” in valves (converting pressure drop to mass accumulation)

Skeleton of Systematics

- Master Program- h2frontpanel

  - Display

- Mass Balance Accounting Global(on each control volume)+Energy Accounting-Called by 1, Called by 6

(controller can be at time= 0 send a determined amount of fluid to each bioreactor for a set amount of time, alternate between each bioreactor with regards to the times. If Bioreactor gets pulses of glucoses for half an hour, next Bioreactor will get it a half an hour later, they will all be phased with respect to each other. This will cut down on the amount of valve switches.

  - Segment for syringe pump + feed tank

  - Around each Bioreactor

  - Around each bioreactor and peristaltic pump and filter

  - Control volume for the Kidney and post process concentration analyzer(Might be an ion chromatography unit).Will need just one around each separately (Just like for the aforementioned bioreactor loop)

  - l

- Accounting Inside each Bioreactor- Called by 2

  - Accounting for all sensors

  - VI for Load Cell

  - VI for impeller(rate of mixing)

  - Change in translucency of the Bioreactor as a function of time

    - Will coat inside of walls that will be repulsive to the cells

    - Overtime coating will wear down

  - Change in translucency of fluid (Inside Bioreactor)

- Energy Accounting-Called by 2,1

  - Any external source of energy that might affect the reactor- multiple heaters

    - Treat heaters as point sources

  - Heat transfer inside/outside of the reactor

    - Natural Convection- not necessarily the same for each wall. Also, the bottom might have a different heat transfer to the table than to the surrounding air

    - Conduction heat loss through walls as well

  - The rate of work done on the system(shaft work, flow streams, boundary work)

    - Mixing of streams

    - Flowrate Of the peristaltic pumps

    - Fluids going in and out of reactor (Flow Rate, temperature, heat capacity, mass)

    - Work done by impeller

  - Pressure

- Image Acquisition-Called by 1

  - IMAQ Card:: IMAQ-A6822:: Camera

  - Acquire Image via camera

    - LED camera

    - Digital Microscope Camera

  - Timed/Manually controlled

  - Stored into a file that is yet to be named

- Valve Control/valve Sequencing- Calls 2

  - Turn Valves on/off

  - Switching of valve positions

  - Vary amount of pulses

  - Have cases for other valve positions/openings

  - Include a time delay -  can reference the PDF instructions

- VI for Scaffold Printing

- Emergency VI

  - All of the alarms-based off of old VI design

    - Flow Rate

      - Is the flow rate going down due to some sort of blockage?

      - Is flow rate going down due to accumulation of waste, can be partly checked via concentration analysis as well as translucency measurement.

    - Pressure

      - Is pressure going up due to some sort of blockage?

      - Is pressure

    - Concentration(s)

    - Temperature

    - Liquid Level

  - Actions based on each alarm

- Syringe Pump VI

  - Vary amount of pulses

- Master Data Acquisition(right underneath main program level)

  - File writer

    - Input data arrays

  - Metronome timer

    - Time delay. Timer for things like feeding, waste control. Want the sequence of events to run every certain time.

  - A sequencer

    - What events take place in what order

      - Feeding

      - Liquid Level control

      - Waste Control

      - Scaffold Input

      - Dye Input

      - Drug Input

      - Image Acquisition

      - Sensor Data Input (Continuous)

      - Valve Switching

- Curve fitting VI

  - Last N data points and fit them should be a polynomial function

  - Calculate the derivatives, dmass/dt or dE/dt, and fit to a polynomial function over a set of data points

  - Ideal a flat line, but will not always be this

  - Accumulator VI’s for mass in each control volume Reactor, or the entire aquarium, etc, each control volume discussed previously.

    - Calculating enthalpies of each stream, masses of each stream.

    - Put together an excel spreadsheet that would summarize all sensor inputs that would be required in all mass and energy balances. (Labview should write to an excel file)

  - Once data points fitted to a function, will take derivative of that polynomial function to see if mass and energy mass balances closed in the time interval

    - You collect data in last minute at 1 second intervals

    - Did the accounting actually say that mass in - mass out over the last 60 seconds

    - Or did we some sort of change?

    - If filling a Bioreactor with fluid, there will be a change of fluid level as a function of time. Taking all( in -out + generation + accumulation = 0)

    - Might have to integrate certain things like change in mass over a certain a time

    - Need to pay special account to some derivatives (such as emptying of tank)

Control Volumes

- Whole Bioreactor

- Bioreactor & Surrounding loop

- Pre-bioreactor (syringe Pump, valves, etc.)

- Post-bioreactor(concentration analysis and waste removal)

- Scaffold and cells and glass slide

- Aquarium

Mass and Energy Balance Accounting

- List all components (molecules, cells, scaffold, glass slide, ions, waste solid)

- For each component

  - Cp

  - Mass or mass flow rate

  - Molecular weight if any

  - Enthalpy (standard state and difference from standard state)

  - Mass or molar fraction

- ∆Hmix

- Heaters in aquarium

- Heat losses to bottom table

- Heat losses to surrounding air through sides

- Heat losses through top

Lid has penetrations for all sensors and impellers

- Mixing of entire aquarium

Can it be treated as uniform constant temperature bath?

- Heat rises- Each bioreactor will have “flip-top” with penetrations

Note: 4-7 *aquarium level*

Bioreactor

- Top

- Sides

- Bottom

BioreactorMassBalance.vi

Inputs: 	SyringeFeedRate, RecirculationRate, lR, wR, liquid, solid, dfilter,

BioreactorNumber, hR,set, hsolid,set, hR,old, hsolid,old (last two are vectors of 		last N = 10 heights and act as initial conditions)

Outputs:	hR,new, hsolid,new (output heights act as initial conditions for next actions)

2.10 – National Instruments Data Acquisition System

- Describe what wires from what sensors go where on what DAQ (Data Acquisition) board

- AMUX- 64T  positions 0-63 [64 total] (spreadsheet)

Which wires from which sensors go to which positions on the AMUX- 64T

2.11 - Wireless Data Transmission from Sensors

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

rovingwifipro1.txt

rovingwifiprogetactuatorstate.txt

configdatabase.txt

mysql.txt

2.12 - Other Arduino Code

http://my.fit.edu/~jbrenner/bashur/3dprinting/sensorarduino/openaquariumcode/

timeget.txt

timeset.txt

remotecontrol.txt

powerstrip.txt

powerstrip2.txt

powerstrip3.txt

LEDlamp.txt

leaksensoralarm.txt

2.13 - Budget

One of the main goals of the entire tissue engineering test bed project is to create a working bioreactor setup with cost-efficient alternatives. The prototype also abides to this goal. The following is the breakdown of the cost of making the prototype recirculation loop:

Table 2.13.1 – Budget Breakdown

|  | Number of items used | Cost per piece ($) | Total cost ($) |
| --- | --- | --- | --- |
| Pump | 1 | 4.16 | 4.16 |
| Turbidity sensor | 1 | 11.99 | 11.99 |
| Differential pressure transducer | 1 | 17.99 | 17.99 |
| Flow meter | 1 | 3.13 | 3.13 |
| Valves | 5 | 0.25 | 1.25 |
| Arduino Uno | 1 | 4.16 | 4.16 |
| Tubing | 1 | 4.29 | 4.29 |
| Filter | 2 | 12.50 | 25.00 |
| Tee hose barb fitting | 2 | 2.49 | 4.98 |
| Tube connectors | 4 | 0.34 | 1.37 |
|  |  | Total | 78.32 |

- Excel Sheets

Chapter 3 – Subsystems Fabrication and Validation

Blah

3.1 - Valve Actuation System Assembly and Validation

A crucial instrument in the overall success of this tissue engineering test bed is the implementation of Valco Instruments Co. Inc. (VICI) 10-way valves. Traditionally these valves are controlled manually, but for the tissue engineering test bed, the valves will have automated control. Figure 3.1.1 shows the flowsheet schematic for connection from the computer to the VICI 10-way valves controlling the distribution to bioreactors 1-36. For example automated control writes from the computer through a National Instruments (NI) interconnect 96-pin digital input/output interface (PCI-DIO-96) to a NI SC-2054 cable adapter via pins 1-50 of a 100 pin ribbon cable and then to bioreactors 1-6 via port C:J4 in Figure 3.1.1. The parts locator diagram for the SC-2054 is shown in Figure A3.1.2.2.

Figure 3.1.1: Wiring diagram for control of 10-way valves responsible for feed distribution to bioreactors 1-36

This requires an interaction with National Instruments (NI) LabView, a data acquisition and control software. Additionally, Measurement and Automation Explorer (MAX), by National Instruments, can be utilized for testing connectivity of the 10-way valve. To connect to either software, a series of connections must be made.

First, a DIO-96 port is installed into the PCI slot of a computer. Thereafter, a 100 pin ribbon cable is connected to the DIO-96. This R1005050 cable splits into two 50 pin cables. Pins 51-100 of the cable were connected to a National Instruments SC-2054 cable adapter. This allows a 26-pin ribbon cable, built for the control of the VICI 10-way valve, to connect to the 10-way valve. Thus, the 10-way valve was connected to the computer. MAX was run to test connectivity. Figure 3.1.2 shows how the 10-way valve connects to MAX.

Figure 3.1.2: Complete wiring connection from 10-way valve to NI MAX software

Though connecting the controller of the 10-way valve to the computer leads to automated control, it comes at a cost. Due to the nature of the pin outs on the SC-2054, only eight pins can read or write digital input or output in the form of 5 V or 0 V for on and off. This means that that the computer can only send signals to control eight pins of each 26 pin connection on the SC-2054, and in theory eight positions on the 10-way valve. Diagnostic testing was performed to test the connectivity between devices as follows.

Pertaining to the diagnostic testing of the 10-way valves, electronic communication issues between devices were present. LabView and MAX are integrated for National Instruments and not VICI, there are communication difficulties between the connection points on the pins of the NI SC-2054 and the ribbon 26 pin ribbon cable connecting the 10-way controller to the SC-2054. For example, based on the pin assignments of the 100-pin cable, pins 51-100 have been connected into the SC-2054 cable adapter in J1 (Port names are shown in Figure A3.1.2.2 and pin assignments for J1 are shown in Figure A3.1.2.3). Additionally, the 26-pin ribbon cable connecting the 10-way valve controller to the SC-2054 has been connected to port J6 (Figure A3.1.2.2). Based on this orientation, the path for the channel on MAX must be set to CPA in correspondence with the pin assignments for the SC-2054 Input/Output (I/O) connectors to J6 (Figure A3.1.2.4). Running the Test Panel function on MAX and writing to port, the 8 pins of port 6 (CPA0 to CPA7 Figure A3.1.2.2) yields some connection issues. First, pins CPA7 to CPA3 read “on” in MAX, signifying they have a 5 V connection. However, the test panel was not told to write to those pins. This is shown in Figure 3.1.3. Upon further inspection, measuring the voltages of the corresponding pins with a volt meter, voltages of approximately 5 V, 5 V, 2.5 V, 2.5 V, and 5 V were measured.

Figure 3.1.3: Complete wiring connection from 10-way valve to NI MAX software showing communication issue between NI SC-2054 and VICI 10-way valve

Testing the voltages of the pins of the SC-2054 without the 26-pin ribbon cable and 10-way valve connected yielded correct communication between MAX, the PCI-DIO-96, the 100-pin ribbon cable, and the SC-2054. Conversely, testing the manual control of the VICI 10-way vales with a switch yielded correct function of the valve. Therefore, the logic became the wiring of the 26-pin ribbon cable connecting the VICI 10-way valve to the NI SC-2054 was the issue. Further diagnostic testing verified the communication obstruction between the NI and VICI device systems. The wires of the 26-pin ribbon cable needed to be modified to establish proper communication from MAX to the VICI 10-way valve.

The explicit problem was that each valve position requires a certain pattern of input to the pins of the 10-way valve controller via the 26-pin ribbon cable. To toggle between valve positions, multiple wirings need to send signals via one position signal on MAX. Without modification, MAX can only send one wiring signal through one pin. For example, position 7 on MAX can only send one signal to one pin assignment. In this case MAX sends a signal to port A position 7 (PA7) on the SC-2054 (see Figure A3.1.2.3), and therefore only one wire connected to the VICI 10-way valve can receive this individual signal from MAX.

Testing determined that only four total wires are used to automate the VICI 10-way valve.  Using the numbering and color scheme for the wiring diagram of a 26-pin rainbow ribbon cable (Figure A3.1.4.1), the wires were able to be defined in Table 3.1.1.

Table 3.1.1: 26 pin ribbon cable wires used to automate VICI 10-way valve

| Wire color | Wire number | Binary Coded Decimal (BCD) |
| --- | --- | --- |
| black | 10 | 0 |
| yellow | 14 | 4 |
| blue | 16 | 2 |
| gray | 18 | 1 |

Testing also determined the pattern of signal inputs necessary for each valve position on the VICI 10-way valve. An example of the rewiring required to control the VICI 10-way valve through J6 Port A on the NI SC-2054 is shown in Table 3.1.2. The wiring for the 26-pin ribbon cable is shown in Figure A3.1.4.1 and A3.1.4.2.

Table 3.1.2: Example of wiring specifications for proper communication from MAX to VICI 10-way valve

| MAX test panel position number | Pin assignment for J6 Port A | Pin number for J6 Port A | VICI 10-way valve, valve position | 26-pin ribbon cable Wires that need voltage to execute action | Binary Coded Decimal (BCD) combination |
| --- | --- | --- | --- | --- | --- |
| 7 | PA7 | 9 | 7 | 10 + 14 +16 +18 | 0 + 4 + 2 + 1 |
| 6 | PA6 | 11 | 6 | 10 +14 +16 | 0 + 4 + 2 |
| 5 | PA5 | 13 | 5 | 10 + 14+ 18 | 0 + 4 + 1 |
| 4 | PA4 | 15 | 4 | 10 + 14 | 0 + 4 |
| 3 | PA3 | 17 | 3 | 10 + 16 +18 | 0 + 2 + 1 |
| 2 | PA2 | 19 | 2 | 10 + 16 | 0 + 2 |
| 1 | PA1 | 21 | 1 | 10 + 18 | 0 + 1 |
| 0 | PA0 | 23 | 0 | 10 | 0 |

Reading Table 3.1.2 proceeds as follows: Writing to J6 port A, the position number 7 on MAX test panel pin corresponds to pin assignment PA7 on the NI SC-2054. The corresponding pin number for this position is pin number 9. Position 7 on MAX test panel can send an “on” or “off” of 5 V or 0 V, respectively, to pin number 9. This individual pin, # 9 is used to set the VICI 10-way valve to position 7 by multiple signal inputs. Diagnostic testing has determined position 7 on the VICI 10-way valve requires 5 V signal input sent to wire numbers 10, 14, 16, and 18 from MAX position 7. The corresponding BCD combination is 0 + 4 + 2 + 1 = 7.

Lastly, defining the communication pathway between all the NI equipment and the VICI equipment was established. Tables 3.1.1 and 3.1.2 show the wiring combinations required to control the VICI 10-way valve position through one signal output from MAX, despite needing to send signals to multiple wires on the VICI 10-way valve controller. This can most likely be accomplished by implementing a printed circuit board (PCB) Phoenix contact terminal block. This device works by splitting one wire into multiple wires, allowing for multiple connection from one wire. This means splitting one signal from MAX into multiple signal from the same source. For example, the PCB Phoenix contact terminal block can take position 7 on MAX and send the signal from pin 9 to wires 10, 14, 16, and 18 on the VICI 10-way valve controller simultaneously. By doing this, communication is properly established between NI and VICI equipment, and the VICI 10-way valves may be automated for tissue engineering test bed applications.

3.2 - Syringe Pump System Assembly and Validation

Blah

3.3 - Sensor Subsystem Assembly

  - Collecting data for an array of sensors

Figure 9 (a): Project layout for collecting data from an array of sensors

Figure (9b): Project box layout for collecting data from an array of sensors

The figures above are the project box layout for collecting data from the sensors into a computer to record and save the data acquired from the bioreactor. The sensors are connected to their respective shields and inserted to a breadboard. The breadboard is connected to a power supply and an Arduino UNO that reads the voltage signals from each sensor and converts it to data that can be understood. For example, the pH sensor is connected to its shield via BNC type connector and inserted into the breadboard. The breadboard already connected to a power supply is connected to an Arduino UNO board that collects the data signals. This data signal goes from Arduino via USB or WiFi to the computer and the written code is executed, resulting into a real time display of the pH of the solution. Each component is described more extensively in Appendix B.

The first board (Figure 9(a)) has the electrical conductivity, dissolved oxygen, pH, flow totalizer, and concentration sensors and shield mounted on it. The purple shields at the top of the breadboard are voltage isolators, these helps make sure that the voltage signals from the various sensors do not interfere. At the bottom of sensor are inputs points of the sensors. After the first row of the purple voltage isolators, the second row below from left to right has the green shields for mixer and peristaltic pump (recirculation pump), followed by the dark blue shield for flow totalizer for measuring recirculation flow rate, the black middle shield for all the concentration sensors, the white shield for microscope, the yellow shield for dissolved oxygen, and the last blue shield for the tachometer: for the mixer stirring rate to assure proper mass transfer and mixing of both glucose and oxygen.

The second board, from left to right, we have the orange pH shield, the red load cell for the cell plus glass slide plus scaffold, the green electrical conductivity shield, another red shield; 5 kg load cell for mass of entire reactor content, the blue oxidation reduction shield. Finally, the temperature shield slightly towards the top of the blue oxidation reduction shield. The purple shields are voltage isolators. Below the red line on the breadboard, we take signals from each sensor and wire those to the analog in channels in the Arduino Uno.

  - Initial sketches for the configuration of bioreactor

The figure below shows the crude initial sketches for the configuration of the bioreactor. Figure 10(a) is very similar to that shown in (Qian et al., 2016). This configuration was revised because the occurrence of  dead volumes in the edges of the reactor will be very prominent, and some sensors might not read sample data effectiviely as they are of different lengths. Dead volume within a bioreactor minimize the dimension of the bioreactor. In other words, they make the reactor seem smaller than it is, thus resulting in errors during calulation. The second configuration (Figure 10(b)) was made with the idea that the shorter and “more important” sensors will be placed directly above the load cell for accurate reading. Unfortunately, just as in the previous configuration, the occurrence of  dead volumes in the edges of the reactor will be very prominent. The third configuration (Figure 10(c)) was also made to accommodate the shorter sensors so they could be at the same length within the bioreactor. For the first three configuration, the problem of dead volumes is eminent. The most recent configuration shown in Figure 10 (d) contains a clear cylindrical container similar to that used for storing pickled foods. Holes will be drilled through the top to hold the various sensors and that will be connected to the project box layout in Figure 9(a&b). It is expected that the cylindrical shape of this reactor should minimize the dead volume within the reactor.

Insert drawings

The bioreactor now in use is of dimensions 17.5 x 13.5 x 4 inches as shown in fig (). The lid - fig (10) - was 3D printed with holes to hold the sensors and fittings. The bioreactor consists of a plastic container with a lid. This lid is penetrated with (from left to right)  the pH, oxidation reduction potential (ORP), Electrical conductivity (EC), and Dissolved Oxygen (DO) sensors; an impeller; tubes for fluid flow in and out, and mass balance wires; and a microscope USB data collecting port. Inside the plastic container is a microscope stand. Mounted on the stand are four granite cubes (for vibration isolation) which have two 1 kg load cells and a plastic glass holder on them; they are all bound together to the microscope base with a 1.5 inch countersink screw. The wires from the load cells are inserted into a heat shrink tubing to seal from water and attached to the sides of the bioreactor to minimize cluttering. The wire for the microscope is wrapped into a PVC pipe tucked behind the microscope stand and properly sealed with flex seal from water.

E

E

D

D

c

c

c

c

c

c

c

c

c

c

B

B

A

A

- A

- A

A

A

A

A

A

A

Figure 10: 3D model of bioreactor lid

The four holes labeled “A” for the DO, ORP, EC, and pH sensors are 0.846 inches in diameter, the relatively smaller holes labeled “C” for the fluid in and out and mass balances are 0.551 inches in diameter, the hole almost at the center for the mixer labeled “B” is 0.950 inches in diameter, the hole labeled “D” for the microscope USB port is 1.075 inches in diameter, and the rectangular hole labeled “E” for the glucose and lactate concentration as a length of 1.270 inches and a breadth of 0.750 inches. The entire lid as a perimeter of 24.011 inches, a total loop length of 53.707 inches and an area of 28.458 inches squared.

Insert CAD drawing of entire bioreactor

Design and Materials

The size and number of sensors caused space constraints that made using the original mason jar structure as the bioreactor not feasible. Therefore, the jar was substituted by a cereal container with its own lid from Amazon. It has length and width X and X respectively, shown in Figure 1b. However, since the cereal container lid has dents and curves it does not have the space we needed to fit all the sensors, the mixer, the input and output flow, and the load cell wires. Figure 1 demonstrates how the pg cable glands are so packed they cannot be tightly secured and sealed, so we 3D printed a new lid.

Figure 1a. Original lid for cereal container

Figure 1b. Cereal container body

The first attempt had some defects around the edges of the sensor holes. This may have been due to changes in temperature of the printing room. These defects did not allow the pg13.5 cable glands to seal properly. So, we reprinted the lid, for the next design the hole for the fluid in and out as well as the one for the load cell wires were made to fit PG7 cable glands as shown in Figure 2.

Figure 2. The second printed lid

A main issue was getting the lid to stay put. Although the lid fit, it would proceed to pop up after a few seconds. The solution options explored were to make the inner lips of the lid longer as well as using an O-ring to seal and help the lid stay tight. The last solution considered was a fastener clip or latch to secure the lid in place, but this would require additional design and longer printing time which leaves more space for printing errors. However, the final printed lid has a better fit and hence will not need a latch but will still use an O-ring to guarantee the seal.

Load Cell

Figure 3a. Initial load cell setup

A fundamental part of this project is the mass measurements to be obtained from two load cells. One load cell is to be placed inside the reactor directly beneath the microscope. The other load cell will be placed on the outside, under the complete experimental setup. These will help quantify and confirm that cells are in fact growing, by measuring the initial mass and subtracting that from later mass measurements. The initial setup for the inner load cell consists of the microscope stand with rubberized feet, granite blocks the plastic glass slide holder, and the glass slide as seen in Figure 3a. This load cell has a weight range of 5 kilograms and it records measurements up to 4 significant figures.

At the time of testing and collecting data from the load cell, it was discovered that the load cell works with a cantilever mechanism. This forced the team to change the setup to the one seen in Figure 3b. This new setup replaces the plastic glass slide holder and glass slide for a 1 in x 1 in cover slip. Both granite blocks used for vibration isolation will now only be placed on one end of the load cell since it operates as a cantilever it cannot have anything directly beneath it on the weighing side. Moreover, the load cell was placed on the side of the base rather than the center because the mixer, located on the right/left requires space.

Figure 3b. Actual load cell set up

The outer load cell: kgs? Sig figs? Cantilever mechanism?

System interface to record values with Arduino.

Each load cell comes with four wires, since these are not long enough to exit the reactor as is, it was required to solder new wires to them. Although this adds on to the resistance on the wires it was a simple, straightforward way to extract them from inside the reactor. The soldered wires are long enough to be sealed in a tube which starts at the load cell and ends outside the reactor lid through one of the holes.

Vibration Isolation

As seen from the previous images the whole load cell set up is secured with screws,

Figure 2a shows the use of round headed screws that would inevitably interfere with the glass slide. Although the issue could be addressed by replacing them with flat head screws, the second setup displayed in Figure 2b does not require it.

The first step towards isolating the setup from vibration is the rubberized feet on the microscope base. Fortunately, the base came with the rubberized feet included. Another component contributing to the vibration isolation is the granite blocks, located directly under the load cell.

Figure 2A - Schematic of retractable shield inside reactor

In addition, the greatest isolation component is the draft shield located inside the bioreactor covering the microscope and load cell. Its purpose is to isolate the set up at the time of recording mass measurements and performing cell imaging. The shield will be made of GLASS and will be automated with a NEMA stepper motor that will be secured on an outside structure with aluminum railings and rods. This means the shield doors will remained raised, except for when it is time to gather data. The shield doors will then be lowered isolating the microscope and load cell setup. The railings and support structure will be assembled around the apparatus in a square shape. The reason behind this choice of equipment is the easy accessibility to the parts and pieces that can be purchased online or at the Florida Tech campus from Mr. Gallagher.

Another aspect of the vibration isolation incorporated into this experiment is a sand box which will hold the bioreactor. The “box” will not only have traditional sand but also an upper layer of kinetic sand. It has the aspect of wet sand, and includes useful properties like not sticking to anything, not drying out and the ability of being molded into anything. Using these two types of sand is just another step towards reducing vibration as much as possible.

The vibration isolation for the outer load cell includes…

Blah

3.4 – Individual Sensor Validation

Blah

3.4.1 – Temperature

Blah

3.4.2 – Liquid Level

Blah

3.4.3 – pH

Blah

3.4.4 – Electrical Conductivity

Blah

3.4.5 – Dissolved Oxygen

Blah

3.4.6 – Oxidation-Reduction Potential

Blah

3.4.7 – Glucose Concentration

Blah

3.4.8 – Lactate Concentration

Blah

3.4.9 – Impeller Rotation Rate

Blah

3.4.10 – Flow Totalizers

Blah

3.4.11 – Syringe Pumps

Blah

3.4.12 – Colorimetric Imaging of Unwanted Biofilm Growth

Blah

3.4.13 – Cell/Scaffold Mass

Blah

3.5 – Imaging

Blah

3.5.1 – Validation of Imaging with Robotic Arm

Blah

3.5.2 – Validation of Imaging without Robotic Arm

Blah

3.3 – Filtration System Assembly

Figure 3.3.1 – Experimental setup for filtration testing

Figure 3.3.1 shows the setup for the experiments that were conducted. A “bioreactor” will be constantly mixed by an electric stirrer. As the solid is kept suspended in the liquid, the water is pumped by the diaphragm pump. This water goes through the flow meter, then the filter, then back to the “bioreactor”. There are a couple tees in the influent and effluent side of the filter where the liquid flows into the high pressure side and the low pressure side of the pressure transducer.

3.3.1 – Filtration System Validation Test Overview

Multiple tests were done in order to achieve an accurate representation of the variances involved within the filters and the sensors. Using the same weight of solids on each experiment and using fresh filters on each run, a comparison on the data gathered was done to determine whether each test’s data were comparable within experimental error. Achieving similar data over multiple tests showed that the tests (and in extension, the sensors and the filters) are reliable and are not affected by unknown significant variances.

3.3.2 – Filter Validation

The two most important pieces of information needed in a filtration system are a) knowing what the upper control limits (UCL’s) on each variable are supposed to be, and b) how many times a filter can be backwashed before the filtration efficiency drops too low. The step-by-step procedure for finding out the number of times a filter can be backwashed is as follows:

- Add diatomaceous earth to 1 liter of liquid in the mixing tank.

- Turn on the pump.

- Allow the system to filter as much solid as it can until either the flow of liquid stops due to backpressure or the value of turbidity stops decreasing.

- Turn off the pump.

- Remove filter from the filtration system, and prepare to backwash.

  - Install the filter to the backwash pump with the EFFLUENT of the filter connected to the FLOW OUT side of the pump.

  - Turn on the backwash pump for at least 10 minutes.

  - While the pump is running, agitate the filter to remove as much solid in the filter as possible.

  - After 10 minutes of backwashing, remove filter from the backwash system, and reinstall it into the filtration system.

- Repeat steps 2-5.

- If the turbidity reaches the same steady-state point as the previous run and is not the minimum turbidity that the sensor can measure (when all solids are filtered out), repeat the entire experiment with less mass of diatomaceous earth.

- When the turbidity reaches the minimum turbidity that the sensor can measure, the number of backwashes performed is the maximum limit a filter can undergo while still being able to filter solids effectively.

In order to know when a filter requires a backwash, an upper control limit must be set. The upper control limits will be determined in this thesis by taking the highest steady-state differential pressure or lowest steady-state turbidity on each backwash number from the filter tests.

3.3.3 – Individual Sensor Validation

The concentration of particles in the solution can be quantified as a function of turbidity. Figure 3.3.3.1 and 3.3.1.2 shows two possible options to consider for the relationship of particle concentration and turbidity. A third option (linear equation of Fig 3.3.1.1) according to Beer’s Law is possible, but the equation does not fit the points very well. For the rest of the experiments, a polynomial equation was used to estimate particle concentration. Because of the nonlinearity at low particle concentrations, future work will include increasing the gain when turbidity is less than 500 NTU by a factor of five. This increased the noise as well; appropriate filtering will be done in software.

Figure 3.3.1.1 – Turbidity vs. particle concentration curve (polynomial)

Figure 3.3.1.2 – Turbidity vs. particle concentration curve (log-log)

For each individual sensor validation test, 5 g of diatomaceous earth was used to make sure that all solids were filtered out. For each run, the sensor readings were allowed to reach steady state before any solids were introduced.  The turbidity sensor, differential pressure transducer, flowmeter, and  were validated in Figure 3.3.1.3, Figure 3.3.1.4, and Figure 3.3.1.5, respectively.  Figure 3.3.1.6 shows how the flowrate decreases and pressure gradient across the filter material increases with time.

Figure 3.3.1.3 – Turbidity validation test

Figure 3.3.1.4 – Differential pressure validation test

Figure 3.3.1.5 – Flowmeter validation test

Figure 3.3.1.6 – Differential pressure and flow rate test

3.3.2 – Filter Tests

For the filter tests, 20 g of diatomaceous earth was used as the initial weight of solids mixed into 2 L of water. The initial concentration in the system is 10 g/L, or 1% w/v. A backwash time of 10 minutes was utilized, and sufficient agitation of the filter was applied until the effluent of the backwash has been clear for a few minutes.

Figure 3.3.2.1 – Filter reusability test with turbidity

Using the values of turbidity in Fig 3.3.2.1, the mass in grams adsorbed per filter run was estimated. This was achieved using the equation found in Fig 3.3.1.2. It was found that 4 g of solids were filtered out for the first few runs. Table 3.3.2.1 shows the breakdown of solid removal for every backwash (BW) done on the filter.

Table 3.3.2.1 – Mass of Solids Removed

|  | Turbidity (NTU) | Concentration (g/L) | Mass of Solid (g) | Mass Removed (g) |
| --- | --- | --- | --- | --- |
| Initial | 4294 | 10.2 | 20.4 |  |
| 1 BW | 3146 | 8.2 | 16.3 | 4.1 |
| 2 BW | 2125 | 6.1 | 12.3 | 4.0 |
| 3 BW | 1291 | 4.3 | 8.6 | 3.7 |
| 4 BW | 565 | 2.4 | 4.7 | 3.9 |
| 5 BW | 219 | 1.2 | 2.4 | 2.3 |
| Final | 40 | 0.4 | 0.7 | 1.7 |

The maximum capacity of the filter is determined to be approximately 4 grams of solids. Because the solids were not being replenished after each backwash, the mass of diatomaceous earth removed after each run continued to decrease until all solids were filtered out of the system.  The baseline differential pressure increased over multiple backwash cycles (y-intercepts on Figure 3.3.2.2) and within each filtration half-cycle.

Figure 3.3.2.2 – Filter reusability test with differential pressure

Figure 3.3.2.3 – Upper control limits (UCL’s)

The differential pressure at t = 0 in the fresh filter run was significantly lower than subsequent runs. This shows that there is irreversible fouling, Rf, after every run that even backwashing cannot remove. The upper control limit (Fig 3.3.2.3) increase after the first two backwashes is also due to this irreversible fouling.  Figure 3.3.2.4 and Figure 3.3.2.5 can be used to estimate each of the resistances listed in Equations 2.5.2.4.1.6 and 2.5.2.4.1.9.

Figure 3.3.2.4 – Differential pressure change based on flow rate

Figure 3.3.2.5 – Total resistance change based on flow rate

3.3.3 – Verification of theoretical model

In addition to the assessment of the filter capability, a verification of the filter’s conformance to the proposed model was also conducted. The model used, stated in Equation 2.5.2.4.1.29 in Chapter 2, assumed that particles in the media is adsorbed only by the filter, creating a film of solids of only one layer thick.

Figure 3.3.3.1 – Experimental data vs. theoretical model

3.3.4 – Filtration Discussion

Based on the data acquired in the validation tests, it is reasonable to accept that the turbidity sensor is accurate and precise (Fig 3.3.1.1). The sensor was also able to detect the change in turbidity immediately after the diatomaceous earth was introduced. This is important because having a sensor that is sensitive to immediate changes will help operators be alerted for any abnormal fluctuations in the system.

The differential pressure sensor validation test gave mixed results. The differential pressure transducer produced relatively stable data (Fig 3.3.1.2), which is vital due to its importance in determining the condition of the filter. However, the graph clearly shows that at higher differential pressure (more solids captured by the filter), the variance between tests starts to increase. This is most likely due to the fact that the diatomaceous earth has a range of particle sizes. Because the differential pressures between t = 0 to t = 350 seconds had very small differences, it is reasonable to assume that some tests may have had more of the larger particles than the others.

In contrast to the turbidity data, a big concern that would need to be addressed in the future is how unstable the data from the flow meter is (Fig 3.3.1.3). The flow rate values fluctuate to as much as 15 mL/sec. Despite this, the flow rate data shows a clear trend of decreasing over time. Data smoothing was done to the flow meter graph in order to see this trend easier. A moving average with n = 25 is applied to the data on the first run (Fig 3.3.4.1) to demonstrate the effects of data smoothing to unstable data.

Figure 3.3.4.1 – Moving average of flow rate validation test (run 1), n = 25

The decrease of flow rate over time is to be expected because of the sensitivity of the pump to backpressure. To document this, a test to track changes in differential pressure and flow rate was done (Fig 3.3.1.4). As differential pressure increases, the flow rate decreases. This is substantial because a lower rate of flow decreases filtration rate, which then reduces the efficiency of the filter.

Figure 3.3.2.1 and Figure 3.3.2.2 shows the number of backwashes a filter can undergo while still able to filter effectively. Additional tests were done with the weight of the solid introduced were higher, and they showed that 20 g of diatomaceous earth in 2 L of water is the maximum concentration that the filter can clean. Therefore, a filter can be backwashed at most 4 times before it must be replaced. The data gathered from the differential pressure test (Fig 3.3.2.2) also gives insight to the resistances of the filter and its cake. As previously stated, clear indication of irreversible fouling was determined. The filter tests also confirm that the efficiency of filtration decreases with increasing backwash number. This is shown best in Figure 3.3.2.2 where the steady state differential pressure decreases at increasing backwash number.

When determining the upper control limits (UCL) of turbidity and differential pressure, it is important to consider the condition of the bioreactor. Because the goal of the filtration system is to remove as much solid as it can before needing a backwash, the UCL of turbidity should be not much higher than the turbidity of fresh, clean water. During operation, it will be expected that the differential pressure of the system will reach its upper control limit before any significant change in turbidity will be detected. For the UCL of differential pressure (Fig. 3.3.2.3), the highest steady state value can be used. The UCL  of differential pressure will decrease over multiple backwashes due to the diminishing efficiency of the filter caused by irreversible fouling. Two exceptions are the steady state values for backwashes 1 and 2. This increase is attributed to the increase in initial differential pressure due to irreversible fouling. For backwashes 1 and 2, using the UCL of a fresh filter is adequate for controls.

Figures 3.3.2.4 and 3.3.2.5 shows how significant a variable flow rate affects the performance of the filter. At a constant flow rate, the differential pressure within the filter is significantly higher compared to a system with a variable flow rate. The inverse is true for the total resistance applied by the filter. As the solids deposit into the filter, the differential pressure increases. This increase in differential pressure will start affecting the flow rate due to backpressure, decreasing the flow through the system. As this flow further decreases, the total resistance increases much more rapidly.

In Figure 3.3.3.1, it is clear that more than one layer of solids is deposited to the filter via particles polarizing to each other. Because Equation 2.5.2.4.1.25 was derived from the mass balance on the available surface sites and does not contain a multi-layer case, the results from the verification of the theoretical model proved that, at the very least, the filter qualitatively trends with the theory, but that the particulate film is quite thick, consistent with the ~4 g per backwash data.

3.6 – Mass Balance Closure

Blah

3.7 – Energy Balance Closure

Blah

3.8 – Validation of Safety Responses to Expected Step Disturbances

Blah

3.9 – Validation of Safety Responses to Emergency Situations

Blah

Chapter 4 – Evaluation of Tissue Engineering Test Bed

Residence time distribution (RTD) testing around one bioreactor in this thesis was done as a prototype for the entire array of 36 bioreactors. For phase 1 of the tissue engineering test bed testing and validation, omitting the instrumentation of the bioreactor entirely, effectively makes the bioreactor and its accompanying external flow loop a “black box”. Additionally, omitting the feedstocks and recycle systems isolates the plumbing. This leaves a system referred to in this thesis as a bioreactor tracer testing apparatus that will be discussed in detail in Section 4.1.

Since a primary objective is to demonstrate that no mass adsorbs onto the walls or accumulates in the system, tracer studies with dyes and glucose were conducted to prove conservation of mass and no, or at least negligible, adsorption onto the walls of the plumbing. Testing the plumbing with and without a hydrophilic coating to minimize cell growth determined whether coating the plumbing was necessary, and if so, sufficient. In this manner, conducting all testing in Phase I around the inlet, effluent, and bioreactor “black box”, i.e. the bioreactor tracer testing apparatus functions as a non-ideal reactor.

To demonstrate that no mass adsorbs into the walls or even accumulates in the system, tracer study testing was conducted. Stage 1 tests conducted were on naked Tygon and silicone plumbing and then Stage 2 tests were coated plumbing. 
These coated tracer studies had polyethylene glycol (PEG) coated to the plumbing so that tubing and process vessels resist protein and glucose adhesion and any kind of accumulation or adsorption the walls. Afterwards, ultrapure water was purged through the system to ensure the ion concentration was as close to zero as possible to avoid any interactions that could lead to accumulation. Once the coated plumbing was ready for testing, tracer studies were again conducted.

The reactor data of each tracer study produced a concertation curve Ct from measurements in the effluent stream. This data was manipulated to produce several important values, including the mass of tracer in the effluent stream. The value m, mass of dye in the effluent, was compared to the initial mass injected, m0. This test determines the Tygon and silicone tubing efficacy for tissue engineering test bed applications. The Ct also determines the residence time distribution E(t). This function maps quantitatively how much time different fluid elements spend in the reactor (Fogler 2016).  Thereafter, Et was manipulated to produce values for the mean residence time tm, variance, 𝜎2, cumulative distribution function, Ft, and dispersion. These reactor properties model the flow through the bioreactor testing apparatus.

The concentration of a pulse of known mass m0 was calculated prior to injection. This was done using a known concentration of tracer C0, which was either dye or glucose, and known volume V0 (Equation 4.1 below).

|  | m0= C0* V0 | (4.1) |
| --- | --- | --- |

Likewise, the concentration was measured at the outlet of the system. Figure 4.1 below shows a schematic of the tracer study through a reactor. The pulses were dyes first, then glucose.

Figure 4.1: Pulse injection tracer study schematic (Fogler 2016)

The outlet measurements were taken incrementally, utilizing a concentration analyzer. The data produced a C(t) curve. Further, verifying that no mass accumulated within the plumbing, Equation 4.2 shows the relationship to calculate mass from the effluent concentration.

|  | dm=Q*C(t)dt | (4.2) |
| --- | --- | --- |

dm is the mass exiting the system in time increment dt, Q is volumetric flow rate, and C(t) is the function produced by the tracer study data. Equation 5 represents the integrated mass m of the tracer:

|  | m= Q*0∞Ctdt | (4.3) |
| --- | --- | --- |

The ratio of masses under the concentration vs. time and the area of the pulses at constant flowrate can be seen in Equation 4.4.

| Mass of Injection PulseMass of Effluent Pulse =Area Under Injection Pulse CurveArea Under Effluent Pulse Curve | (4.4) |
| --- | --- |

The injection pulse mass, m0, was calculated using Equation 4.1; therefore, it is known, and no inlet concentration function integration is necessary. Figure 4.2 shows the relationship between the injection and effluent concentration curves visually. Ideally, the mass of the exit pulse should equal the mass of the injection pulse. If it does, it proved that no mass accumulated in the system.

Figure 4.2: Theoretical expectation of tracer study result for a non-ideal reactor (Fogler 2016)

It is expected that the mass in the effluent should be the same mass as the input pulse. Ultimately, the testing aimed to prove Equation 4.5.

|  | m0=m | (4.5) |
| --- | --- | --- |

In this experimentation, the initial mass, volume, and concentration are all known. The primary methodology for proving mass is conserved will be through reactor properties derived from C(t) curve calculations. These calculations will yield the residence time distribution function, E(t), the first moment of E(t) known as mean residence time, tm, the second moment of E(t) known as variance, 𝜎2, the cumulative distribution curve, F(t) and the dispersion . These quantitative elements comprise the behavior of the reactor system and will be useful tools in proving the efficacy of Tygon and silicone plumbing for the tissue engineering test bed.

The residence time distribution function, Et, is defined in Equation 4.6 below.

|  | Et=C(t)0∞Ctdt | (4.6) |
| --- | --- | --- |

Integrating Ctdt from 0 to ∞ yields the entire area under the C(t) curve. Mathematically, this value is constant. Therefore, E(t) could be rewritten in Equation 4.7, below, with Area representing the area under the entire curve.

|  | Et=C(t)Area | (4.7) |
| --- | --- | --- |

|  | 0∞Etdt= 0∞Ctdt0∞Ctdt=1 | (4.8) |
| --- | --- | --- |

By taking the integral of the E(t) curve on the bounds t1 to t2 , the function can be interpreted as the fraction of material leaving the reactor that has resided in the reactor during the bounded time (Fogler 2016). For example, integrating from time 0 to 3 min in the E(t) function can be seen visually in Figure 4.3 below.  The shaded region represents the fraction of material that spends 3 minutes or less in the reactor.

Figure 4.3: E(t) vs. t integration schematic showing bounds from 0 to 3 minutes (Fogler 2016)

Additionally, taking the integral of E(t)dt on the bounds of 0 to ∞ yields a result of 1.  This makes sense intuitively because, by definition, at ∞, all the subject material leaving the reactor will have resided in the reactor from 0 to ∞. From the residence time distribution, the mean residence, tm, time can be calculated as shown in Equation 4.9. This is the first moment of the residence time distribution.

|  | tm=0∞t*Etdt= 0∞t*Etdt0∞Etdt=0∞t*Etdt | (4.9) |
| --- | --- | --- |

Since the mean residence time is known and constant, the second moment of the residence time distribution, the variance, σ2, can be calculated in equation 4.10.

|  | σ2=0∞(t-tm)2*Etdt | (4.10) |
| --- | --- | --- |

The next component to calculate for reactor analysis is the cumulative distribution function, F(t), defined in Equation 4.11 below. This distribution maps the function of the fraction of effluent that has been in the reactor for a time less than t.

|  | 0tEtdt=F(t) | (4.11) |
| --- | --- | --- |

Figure 4.4 shows the cumulative distribution function curve, F(t). At completion, the fraction of material in the reaction will be 1 (Equation 4.8) and the F(t) function approaches this bound.

Figure 4.4: Cumulative time distribution schematic (Fogler 2016)

The last component to the reactor analysis is dispersion, which is a parameter used to model reactor flow pattern. Figure 4.5 shows how dispersion affects the concentration exiting the reactor. A nearly ideal plug flow reactor (PFR) has a dispersion that approaches 0. The molecules travel through the system much faster than they diffuse backwards or forwards to lower a concentrated fluid. Conversely, a nearly ideal continuous stirred tank reactor (CSTR) has a dispersion that approaches ∞; the molecules diffuse much faster rate than they travel through the system. Dispersion is calculated using the relationship in Equation 4.12.

Figure 4.5: Effects of dispersion on effluent tracer concentration (Fogler 2016)

|  | 1Pe= Rate of transport by diffusionRate of transport by advection=DU*l=Dispersion | (4.12) |
| --- | --- | --- |

Pe is the dimensionless Peclet number. l is the characteristic length term, which is the reactor length. Uis the local flow velocity. Dis the dispersion coefficient. Figure 4.5 (Fogler 2016).

For residence time distribution data in a closed-closed system, the Peclet number can be calculated and therefore the dispersion can be calculated (Fogler 2016). A closed-closed system is defined as having boundary conditions of no dispersion to the left on the entrance or right of the exit of the reactor. Boundary conditions are shown in Equation 4.13.

|  | z= 0- no dispersion z= 0+ and z= l- dispersion z= l+ no dispersion | (4.13) |
| --- | --- | --- |

If these boundary conditions are met, the Peclet number for the reactor can be found experimentally by determining mean residence time tm and variance σ2 from the RTD data and then solving Equation 4.14 below.

|  | σ2tm2= 2Pe - 2Pe2*(1-e-Pe)) | (4.14) |
| --- | --- | --- |

Since the concentration analyzer is under development by other students, it was not part of the Phase I testing in the tissue engineering test bed development. To obtain the concentration measurements in the tracer dye study, absorbance readings were recorded using a UV/VIS spectrometer. Utilizing Beer’s Law, Equation 4.15, a conversion from absorbance at a certain wavelength to a concentration was computed for each data point (Tro 2014). A is the amount of light absorbed by the sample for a given wavelength, ε is the molar absorptivity, L is the distance that the light reveals through the solution, and C is the concentration of the absorbing species per unit volume.

|  | A= ε*C*L | (4.15) |
| --- | --- | --- |

C can also be rearranged as Equation 4.16.

|  | C=Aε*L | (4.16) |
| --- | --- | --- |

First, a wavelength was selected based on the color of the dye. Since the dye is yellow/green, light is readily absorbed in the UV region, and a wavelength of 380.7 nm was used. Then a calibration curve was plotted using known concentrations of the tracer dye and the corresponding absorbance measurements. Since ε is initially unknown, rearranging Beer’s Law gives Equation 4.17.

|  | ε=A/(C*L) | (4.17) |
| --- | --- | --- |

This allows for the calculation of molar absorptivity, 𝜀. The slope of the linear trendline developed from this absorbance vs. concentration data, α (Equation 4.18). divided by the path length L represents the molar absorptivity, 𝜀 in Lmol*cm (Equation 4.19).

|  | AC= α | (4.18) |
| --- | --- | --- |

|  | ε= αL | (4.19) |
| --- | --- | --- |

Now that 𝜀 is calculated, the data collected for absorbance and be converted to concentration. It is important to note that the concentrations used for the calibration curve needed to correspond to absorbance values less than 0.8 to avoid deviation from Beer’s Law (Tro 2014).  From there, the conservation of mass, the residence time distribution, the mean residence time, variance in mean residence time, the cumulative distribution function, and dispersion are all calculated.

4.1 – Dye Tracer Testing with “Blank” Reactor

The first component built for this thesis experimentation is the assembly of the reactor analysis apparatus. Tygon and silicone plumbing were used to verify its application in the tissue engineering test bed. The feedstocks and syringe pumps will be omitted for these studies as well. The tests were conducted through several sections of Tygon and silicone tubing attached to two diaphragm pumps. The bioreactor tracer testing apparatus is pictured in Figure 4.1.1. The components were assembled as follows: 38 cm of 4 mm diameter Tygon tubing (Appendix A2.5.4.11.2) flows from to a 4 L feed tank to a 7.5 cm flow reducer. This first flow reducer attaches to a 5 cm section of silicone tubing (Appendix A2.5.4.11.1) that also attaches to the inlet of the first diaphragm pump. The outlet of the first diaphragm pump attaches a 5 cm section of silicone tubing that is further attached to a second 7.5 cm flow reducer. This second flow reducer connects an 18 cm section of Tygon tubing to a flowmeter. The first flowmeter connects a 30 cm section of Tygon tubing into the 4 L bioreactor. Another 30 cm section of Tygon tubing connects the bioreactor to the third flow reducer. This third flow reducer attaches to a 5 cm section of silicone tubing which also attaches to the inlet of the second diaphragm pump. The outlet of the second diaphragm pump attaches a 5 cm section of silicone tubing that is further attached to a fourth 7.5 cm flow reducer. This fourth flow reducer connects a 18 cm section of Tygon tubing to a flowmeter. The second flowmeter connects a 38 cm section of Tygon tubing into the 4 L effluent tank. The flow meters are connected to an Arduino Uno (Appendix A2.5.4.7.1), which also connects to a computer.  An Arduino code outputs the measurements of the flowrate between the feed tank to the bioreactor and, likewise, the bioreactor to the effluent tank to monitor constant flowrate through the bioreactor.

Figure 4.1.1: Bioreactor tracer testing apparatus

The feed tank was filled and refilled with DI water before and during testing. The bioreactor was filled to 0.5 L with DI water. The effluent tank was filled and emptied throughout testing. Both diaphragm pumps were connected in parallel. Alligator clips attached the connection points of each pump to two stripped 12 V DC power supply, respectively. A diode was attached between the terminals of both pumps to ensure the direction of current is in one direction. Figure 4.1.2 shows the power connections. By switching on the power, fluid flow reached steady state quickly. Effectively, this simulated the recirculation loop around the bioreactor, tested input and output flow, and created a sufficient enough basis for testing the plumbing.

Figure 4.1.2: Power connection of bioreactor testing apparatus

Once the apparatus was set up, a calibration curve of absorbance vs concentration was generated to find the molar absorptivity of the tracer dye, 𝜀. This allowed for the conversion of the absorbance measurements into concentration values.

Generating this calibration curve started by creating dilutions of the tracer with DI water. The tracer dye used is a concentration of 1 molar (M) green food coloring and DI water. The mixture is prepared by adding enough dye so the absorbance of diluted samples of food coloring and DI water solution, used for the calibration curve, were less than 0.8 absorbance and above 0.1 to most accurately obey the linear relationship of Beer’s Law (Tro 2014). 0 M represents pure DI water, while 1 M represents pure tracer dye solution. Several samples of various tracer dye are prepared from 0 M to 0.5 M for calibration testing.

Each one of these absorbances was tested with a cuvette and a Vernier LabQuest data collector and analyzer linked with a Vernier SpectroVis Plus spectrophotometer (Figure A2.5.1.9.3.1) to create the absorbance vs. concentration plot described previously and seen in Figure A4.1.2.1. Wiring for this system is shown is Figure A2.5.1.9.3.2. This procedure establishes the molar absorptivity 𝜀 and ultimately generates the concentration values for each absorbance measurement during testing. From there, the C(t) curve was generated, and reactor analysis was conducted.  Sample calculations for all of this are in Appendix A4.1.1.

Five trials were run for each type of test: uncoated dye, uncoated glucose, coated dye, and coated glucose. The first reactor analysis was conducted with tracer dye through uncoated Tygon and silicone plumbing. The known mass m0 was injected into the injection mix point, the top of the bioreactor, of the bioreactor testing apparatus as feed is flowing through at constant flowrate Q. Simultaneously, data collection began. Data points are the concentration measurements taken at incremental time measurements starting at 0 seconds, and every 3 seconds until 15 seconds then every 5 seconds from 15 seconds up to 60 seconds, then from 60 to 90 in 10 second increments, then from 90 to 330 in 30 second increments, and two final measurements at 400 seconds and 480 seconds. Data collection was done by filling vessels with the effluent stream contents for 1 second.

After the run was over, absorbances at each time interval was measured. An absorbance of 0.000 ± 0.003 was the baseline value. Since the calibration curve determined α, absorbance per concentration, the molar absorptivity 𝜀 was calculated. Each data sample can be measured in absorbance and converted to a concentration. From here, reactor analysis C(t), N, E(t),  tm, σ2, F(t), and dispersion could all be calculated.  Sample calculations for all of this are in Appendix A4.1.1 and Appendix A4.1.2.

The second part of stage 1 testing was the glucose tracer study. The glucose solution was prepared using 100% pure dextrose powder and DI water to make a solution of 1000 mg/dL, which is normal human blood glucose concentration multiplied by a factor of 10. The glucose solution was mixed thoroughly, and dye was added until the absorbance was approximately 1.000.

The reason glucose was measured this way is because glucose concentration measuring is quite difficult without the proper instruments. During this work, several attempts were made to measure glucose directly. The first attempt was measuring glucose with a ReliOn diabetic blood glucometer (ReliOn Prime Bllod Glucose Monitoring System 2019) The ReliOn glucometer yielded nonsensical results. Testing failed because this style of glucometer can only measure concentrations of glucose in blood. Since most blood glucose meters use the either oxygen electrode method or a colorimetric assay, the hypothesis of the failed readings was that the DI water and glucose solution does not have the density, ionic concentration, or color, suitable for this glucometer. The next attempt was made by using a GluCell glucometer (Chemglass Life Sciences 2019). This glucometer is designed to measure concentrations of Dulbecco’s Modified Eagle Medium (DMEM) which is the standard glucose media for cell culture. Despite somewhat accurate measurements of glucose concentration in manufacturer prepared DMEM, this glucometer would not be feasible for measuring dilute concentrations of the DMEM because each prepared solution requires buffering to maintain physiological pH. That is difficult to achieve while using DMEM as a tracer in a tracer study with a constantly changing solution dilution. Such a concentration measurement device is the focus of a Ph.D. dissertation under the development by Guochang Ye in Florida Tech’s Biomedical Engineering Program.

Therefore, glucose was measured colorimetrically to determine concentration. After preparing the solution, the test was conducted exactly the same way as the tracer dye tests.

Stage 2 testing was the same as for the tracer studies conducted for Stage 1, but run with a coated bioreactor testing apparatus. This required coating of the bioreactor tracer testing apparatus. This was done by PEGylating the plumbing of the system with polyethylene glycol (PEG) 3350 (Figure 4.1.3), using the commercial form of MiraLax. PEG was chosen because it is a polymer with high hydrophilicity and electric neutrality ideal for resisting material adsorption and protein adhesion (Xiao, Jiang and Zhou 2013). The hydrophilicity of the PEG coating keeps highly polar water attracted to the walls of the Tygon and silicone tubing and keep the less polar dye and glucose in solution, making it an ideal coating.

Figure 4.1.3:  Molecular structure of PEG

A solution of 68 g/L of PEG, consistent with that of MiraLax, and DI water was mixed thoroughly. The bioreactor tracer testing apparatus was purged of all contents with ultrapure water for 5 min and then drained and dried. 3.5 L of the PEG solution was then dispersed through the system with 0.5 L directly added to the bioreactor and the other 3 L added to the feed tank. From stage 1 results, it takes approximately 300 seconds to pass all the tracer molecules through the bioreactor. Therefore, by applying a scaling factor of 4, the PEGylating was run for 20 minutes (1200 seconds), recirculating the fluid from effluent tank to feed tank to ensure every PEG molecule had passed through the entire system multiple times. After 20 min, the PEG was no longer recirculated, and the bioreactor was drained. Afterwards, the system was purged with DI water to prepare the system for stage 2 testing.

For Objective 2, determining the feasibility of Tygon and silicone tubing for this tissue engineering test bed, a series of tracer studies was conducted for tracer dye and tracer glucose to show conservation of mass and the net adsorption of the Tygon and silicone tubing walls is zero. All the tracer study data collection contained measurement error in both concentration (through absorbance) and time. Since there are many data points collected in every trial, the total partial derivative calculus error was higher than ideal. In general, the percent random error was substantially less than the percent error from partial derivative calculus The results were as follows:

The tracer studies for dye flowing through the non-instrumented bioreactor and recirculation loop prior to PEG coating yielded satisfactory results. The full reactor analysis results are shown in Table A4.1.3.1.1 in Appendix A4.1.3.1. These tracer studies contained measurement error in both concentration (through absorbance) and time, 3.0*10-3 mol/L and 0.5 seconds, respectively, for trials 1-2 and 2.5*10-3 mol/L and 0.5 seconds, respectively, for trials 3-5. The total partial derivative calculus error is between 19% - 21%. However, the average percent random error is 8 %, which is substantially less. The concentration curve is shown in Figure 4.1.4. There is some variance between the trials, but they follow the same general flow pattern; each has a sharp peak around 9-15 seconds with a downward cubic polynomial slope with completion at 300 seconds. Each trial’s Ct curve shows a small deviation from the trendline somewhere between 20-45 seconds. This is likely due to eddies located in the bioreactor or the fittings causing areas of mixing taking place. The average mean residence time and variance for trials 1-5 are 79 s and 3,900 s2, respectively. The residence time distribution Et for these trials is shown in Figure 4.1.5. The cumulative distribution function Ft for these trials is shown in Figure 4.1.6.  More detail is found in Appendix A4.1.3.1. The dispersion ranges from 0.50 – 0.75 with an average of 0.61.

Figure 4.1.4: Ct curves for tracer dye studies, trials 1-5, conducted prior to coating the reactor system with PEG

Figure 4.1.5: Et tracer dye studies, trials 1-5, conducted prior to coating the reactor system with PEG

Figure 4.1.6: Ft curves for tracer dye studies, trials 1-5, conducted prior to coating the reactor system with PEG

The tracer studies for tracer glucose flowing through the non-instrumented bioreactor and recirculation loop prior to PEG coating yielded satisfactory results. The full reactor analysis results are shown in Table A4.1.3.2.1 in Appendix A4.1.3.2. These tracer studies contained measurement error in both concentration (through absorbance) and time, 2.7 mg/dl and 0.5 seconds, respectively, for trials 1-5. The total partial derivative calculus error is between 19% - 22%. However, the average percent random error is 6 %, which is substantially less. The concentration vs. time curve is shown in Figure 4.1.7. Trials 2-5 are nearly identical while trial 1 has a higher peak and larger dispersion. All these trials follow the same general flow pattern; each has a sharp peak around 9-15 seconds with a downward cubic polynomial slope with completion at 300 seconds. Similar to the uncoated tracer dye study, each trial’s Ct curve shows a small deviation from the trendline somewhere between 20-45 seconds Again, this is likely due to eddies located in the bioreactor or the fittings causing areas of mixing taking place. The average mean residence time and variance for trials 1-5 are 74 s and 3,600 s2, respectively. The residence time distribution Et for these trials is shown in Figure 4.1.8. The cumulative distribution function Ft for these trials is shown in Figure 4.1.9. The dispersion ranges from 0.62 – 0.96 with an average of 0.73.

Figure 4.1.7: Ct curves for tracer glucose studies, trials 1-5, conducted prior to coating the reactor system with PEG

Figure 4.1.8: Et curves for tracer glucose studies trials 1-5 conducted prior to coating the reactor system with PEG

Figure 4.1.9: Ft curves for tracer glucose studies, trials 1-5, conducted prior to coating the reactor system with PEG

For Objective 3, determining whether coating the tubing of the tissue engineering test bed is necessary and, if so, sufficient, a series of tracer studies were conducted for tracer dye and tracer glucose to show conservation of mass and the net adsorption of the Tygon and silicone tubing walls is 0. All the tracer study data collection contained measurement error in both concentration (through absorbance) and time. Since there are many data points collected in every trial, the total partial derivative calculus error is higher than ideal. In general, the percent random error is substantially less than the percent error from partial derivative calculus. The results are as follows:

The tracer studies for tracer glucose flowing through the non-instrumented bioreactor and recirculation loop after PEG coating yielded satisfactory results. The full reactor analysis results are shown in Table A4.1.3.3.1 in Appendix A4.1.3.3. These tracer studies contained measurement error in both concentration (through absorbance) and time, 2.7*10-3 mol/L and 0.5 seconds, respectively, for trials 1-2 and 2.6*10-3 mol/L and 0.5 seconds, respectively, for trials 3-5. The total partial derivative calculus error is between 19% - 21%. However, the average percent random error is 6%, which is substantially less. The concentration curve is shown in Figure 4.1.10. There is some variance between the trials, but they follow the same general flow pattern; each has a sharp peak around 9-15 seconds with a downward cubic polynomial slope with completion at 300 seconds. Similar to the uncoated tracer dye study and uncoated tracer glucose study, each trial’s Ct curve shows a small deviation from the trendline somewhere between 20-45 seconds Again, this is likely due to eddies located in the bioreactor or the fittings causing areas of mixing taking place. The average mean residence time and average variance for trials 1-5 are 80 s and 3,700 s2, respectively. The residence time distribution Et for these trials is shown in Figure 4.1.11. The cumulative distribution function Ft for these trials is shown in Figure 4.1.12. The dispersion ranges from 0.49 – 0.68 with an average of 0.53.

Figure 4.1.10: Ct curves for tracer dye studies, trials 1-5, conducted after coating the reactor system with PEG

Figure 4.1.11: Et for tracer dye studies trials, 1-5, conducted after coating the reactor system with PEG

Figure 4.1.12: Ft curves for tracer dye studies, trials 1-5, conducted after coating the reactor system with PEG

The tracer studies for tracer glucose flowing through the non-instrumented bioreactor and recirculation loop prior to PEG coating yielded satisfactory results. The full reactor analysis results are shown in Table A4.1.3.4.1 in Appendix A4.1.3.4. These tracer studies contained measurement error in both concentration (through absorbance) and time, 2.7 mg/dl and 0.5 seconds, respectively. for trials 1-5. The total partial derivative calculus error is between 20% - 21%. However, the average percent random error is 5 %, which is substantially less. The concentration curve is shown in Figure 4.1.13. There is variance present, but all the trials have nearly identical shape to their curves with different peak values. They follow the same general flow pattern; each has a sharp peak around 15-20 seconds with a downward cubic polynomial slope with completion at 300 seconds. Unlike the previous three studies, each trial’s Ct curve shows a larger deviation, much like a plateau, from the trendline somewhere between 20-45 seconds. Trial 3 has this deviation very exaggerated and has a sharp drop from 30–50 seconds before flattening out and producing similar data from trials 1-4 from 90 seconds onward. In addition to eddies located in the bioreactor or the fittings causing areas of mixing taking place, the location of the injection point moved slightly farther away from the inlet and effluent streams. This most likely allowed for more mixing prior to exiting the reactor. However, results were satisfactory. The average mean residence time and variance for trials 1-5 are 78 s and 3,800 s2, respectively. The residence time distribution Et for these trials is shown in Figure 4.1.14. The cumulative distribution function Ft for these trials is shown in Figure 4.1.15. The dispersion ranges from 0.50 – 0.69 with an average of 0.62.

Figure 4.1.13: Ct curves for tracer glucose studies, trials 1-5, conducted after coating the reactor system with PEG

Figure 4.1.14: Et curves for tracer glucose studies trials, 1-5, conducted after coating the reactor system with PEG

Figure 4.1.15: Ft curves for tracer glucose studies trials, 1-5, conducted after coating the reactor system with PEG

## 4.2 Discussion

In the absence of protein, there was no need for PEGylation.  Protein adhesion has thus far not been evaluated.  The pigment of the dye used in the tracer studies is quite robust, and it should be obvious by visual detection where mass accumulated in the bioreactor testing apparatus.

4.3 – Dye Tracer Testing of Bioreactor Only

In order to determine the type of bioreactor, a tracer curve study is performed first using food coloring. The setup for experimentation is as follows, the bioreactor is mounted with fluid flow in (feed stream) at a constant flow rate () and fluid flow out (exit stream) at a constant rate of (). The feed and exit stream flow rate are measured with a flow meter as shown in the image below (). The entire flow system is brought to equilibrium before the food coloring is injected into the inlet stream of the bioreactor. That is, pulsing the reactor with food coloring tracer. As the food coloring is introduced into the inlet stream of the bioreactor, samples of the fluid from the exit stream are collected at time increments of 5 to 30 seconds as shown in Table 4.3.2. The collected samples at each time interval are put in a cuvette to measure absorbance using a Vernier spectrometer. The Vernier spectrometer is first calibrated with known concentrations of dye to create a standard curve of concentration (independent variable) versus absorbance (dependent variable) which are the x and y axes respectively. This standard curve will be obtained by measuring different concentrations and absorbances of food coloring and DI water made as shown in Table 4.3.1.

Table 4.3.1: Data set for creating standard curve

| % Food coloring | Concentration (g/L) | Absorbance at (xx nm) |
| --- | --- | --- |
| 0 | 0.00 |  |
| 5 | 0.05 |  |
| 15 | 0.15 |  |
| 25 | 0.35 |  |
| 35 | 0.35 |  |
| 45 | 0.45 |  |
| 50 | 0.50 |  |
| 55 | 0.55 |  |
| 60 | 0.60 |  |
| 65 | 0.65 |  |
| 70 | 0.70 |  |
| 75 | 0.75 |  |
| 80 | 0.80 |  |
| 85 | 0.85 |  |
| 90 | 0.90 |  |
| 95 | 0.95 |  |
| 100 | 1.00 |  |

Table 4.3.2:  Bench scale tracer application parameter

| Time (t, sec) | Absorbance | Tracer output concentration (g/L) |
| --- | --- | --- |
| 0 |  |  |
| 5 |  |  |
| 10 |  |  |
| 15 |  |  |
| 20 |  |  |
| 25 |  |  |
| 30 |  |  |
| 35 |  |  |
| 40 |  |  |
| 50 |  |  |
| 60 |  |  |
| 70 |  |  |
| 80 |  |  |
| 110 |  |  |
| 140 |  |  |
| 170 |  |  |

#### Mean Residence time

Blah

4.4 – Glucose Tracer Testing of Bioreactor Only

Blah

4.5 – Dye Tracer Testing of Full System

Blah

4.6 – Glucose Tracer Testing of Full System

Blah

4.7 - Filtration System

4.7.1 - Filtration Background

Of the different types of cell death, necrosis is the one requires the most attention because it affects the filtration system of the tissue engineering test bed the most. Cells undergoing necrosis swell, then rupture, releasing their contents out to the environment (Kroemer, et al., 2009). It is important to remove this dead cell debris from the media because of the inflammatory response it stimulates on nearby healthy cells (Rock & Kono, 2008). Inflammation of healthy cells can be a detriment by impairing the natural healing process of the tissue or, in some cases, killing healthy host cells (Martin & Leibovich, 2005).

This nanosized cellular debris can permeate through most of a particular film built upon a porous fibrous mesh before completely blocking the pore. This phenomena, called biofouling, can cause major problems to the filtration system by reducing membrane flux through the filter (Abd El Aleem, Al-Sugair, & Alahmad, 1998) or increasing the pressure drop across the system, affecting filtration performance (Herzberg & Elimelech, 2007). Another significant contributor of biofouling in the filter membrane is the adhesion of extracellular polymeric substances (EPS) such as polysaccharides, proteins, nucleic acids, and lipids (Nguyen, Roddick, & Fan, 2012).

In order to fully understand the big picture of filtration, a closer look on the components and mechanisms involved within the filter while in operation is necessary. At a constant rate of flux, as solid particles get trapped by the filter membrane, the pressure drop across the filter and the osmotic pressure applied by any biosolids will start to increase (Chen, Fane, Madaeni, & Wenten, 1997). There are three distinct phases in the pressure drop progression that are observed during operation (Elmoe, Tricoli, Grunwaldt, & Pratsinis, 2009). The first phase is when particles are adsorbed into the pores, slightly increasing pressure drop. Once there are enough particles on the filter to cover the entire surface area, a “polarization” phase will take over during which particles start sticking to other particles, further increasing the differential pressure within the filter. Finally, at a critical thickness of solid cake formed, the smallest particles that would usually pass through the filter will get trapped in the cake, significantly increasing the pressure drop as illustrated by the Ergun equation (Erdim, Akgiray, & Demir, 2015):

-ΔPL=150µ1-ε2ε3Vdp2+1.75ρ1-ε2ε3Vdp  			 (4.7.1.1)

The Kozeny-Carman equation (Erdim, Akgiray, & Demir, 2015; Tien & Ramarao, 2013) is viewed as an improvement on the Ergun equation:

-ΔPL=180µΦs2dp21-ε2ε3V 		 		(4.7.1.2)

where ε is the porosity of the bed, V is the velocity based on the empty cross-section of the bed, dp is the particle diameter, and Φs is the sphericity of the particles in the bed.

A comprehensive list of correlations relating pressure drop to the properties of the fibrous mesh filters (Fotovati, Hosseini, Tafreshi, & Pourdeyhimi, 2011; Benet, Badran, Pellegrino, & Vernerey, 2017) and/or the size distribution, the zeta potential, and the mechanical properties of the filtered solids (Iritani, Katagiri, Tsukamoto, & Hwang, 2014; Osterroth, Preston, Markicevic, Iliev, & Hurwitz, 2016; Erdim, Akgiray, & Demir, 2015) is found in Erdim, Akgiray, and Demir (2015). In particular, that paper shows that wall effect corrections to the Ergun and Kozeny-Carman equations are necessary, particularly at Re > 500, and are better represented by Equation 4.7.1.3:

fv=160+2.81Re0.904				 (4.7.1.3)

Whereas the aforementioned references are for particulate films growing on fibrous meshes, the viscous permeability, kr2, for molecules permeating through fibrous porous media has been best defined by Tomadakis where εp is defined as the percolation threshold (Tomadakis & Sotirchos, 1993; Tomadakis & Robertson, 2005):

kr2=ε8ln2εε-εpα+21-εpαα+1ε-εp2 				(4.7.1.4)

Chapter 5 – Conclusions and Future Work

## 5.1 Conclusion

The hypothesis of Adrien Hosking's thesis (ref.), that a tissue engineering test bed can be designed and tested in which no mass of glucose accumulates on the plumbing, was confirmed.

Objective 1: The tissue engineering test bed was schematically designed. A P&ID of the entire system was constructed. The appropriate stream numbering of the test bed was also completed. Additionally, a complete mass balance list was generated. These elements along with the hazardous operability study laid the foundation for coding the data acquisition and control for the tissue engineering test by subsequent students.

Objective 2: Tygon and silicone tubing for tissue engineering test bed applications were proven satisfactory. The tracer studies of both dye and glucose conducted in this thesis yielded mathematical and physical results that mass did not accumulate in the system or adsorb on the walls. These results determined the net adsorption rate of the walls of Tygon and silicon tubing was negligible, if not zero, and Tygon and silicone tubing are both suitable for tissue engineering test bed applications.

Objective 3: Coating the tubing was likely not necessary to eliminate adsorption of mass onto Tygon and silicone tubing walls. PEGylating the bioreactor tracer testing apparatus had no quantitative effect on reducing adsorption on the walls. This is a direct result of no detectable adsorption on the walls of the tubing present in Objective 2, Stage 1 tracer study testing. Though PEG might be sufficient for reducing adsorption in other areas of the tissue engineering test bed, perhaps the fully instrumented bioreactor, but it is unnecessary to do so for the plumbing.

Lastly, automation of the VICI 10-way, a decisive part of the success of the tissue engineering test bed construction, was proven feasible. Diagnostic testing ultimately derived the wiring framework that allows NI MAX to send signals to control valve positions on the VICI 10-way valve. This leads directly to full automation of the over 20 VICI 10-way valves and verifies the plumbing design for the tissue engineering test bed.

A working prototype filtration system was successfully created for the tissue engineering test bed project by Jonah Melegrito (ref.). The sensors were also able to show the capability of the filters to be used in the system. The 5-micron capsule filter was found to be able to completely filter out 10 g/L of diatomaceous earth after 4-5 backwashes. Because of the declining filtration rate efficiency of the filters after each backwash, the upper control limits of differential pressure decreases from >115 kPa with a fresh filter to 88 kPa after 4 backwashes.

## 5.2 - Future Considerations

After completing Adrien Hosking's thesis (ref.), there are several avenues to explore. Firstly, reactor analysis should be conducted on a fully instrumented bioreactor. This is the location within the tissue engineering test bed design likely to accumulate mass because of all the unique instrumentation that will be submersed in the fluid volume of the reactor. If no net adsorption of the mass within the bioreactor can be achieved, the entire system will most assuredly be able to resist mass accumulation, and the mass balance of glucose will be complete.

The reactor analysis conducted in Mr. Hosking's thesis (ref.) should be completed for other common cell media that will be used in the tissue engineering test bed. This will verify if the silicone and Tygon tubing are able to resist protein adhesion of growth factors, for example. Likewise, the fully instrumented bioreactor can be tested in the same manner to ensure that protein adhesion will not cause adsorption within the test bed.

A second consideration pertains to developing the code for operation of the entire tissue engineering test bed. Implementing proper data acquisition and control for the sensors and instruments of the test bed system can be achieved by building from the foundation of the code provided by this thesis. Building from the schematics and mass balance accounting provided in this thesis, along with a complete hazardous operability study, will fulfill necessary automation requirements for the test bed.

Now that a wiring framework has been established for communicating between NI equipment and software and VICI equipment, physical wiring of all VICI 10-way can be undertaken. By wiring all of the VICI 10-way valves, the installation of the plumbing for the entire tissue engineering test bad may begin. Moreover, testing flow through the entire system can be conducted along with the coding required to automate these valves.

It is highly recommended that a different pump is installed instead of a diaphragm pump. A pump that is unaffected by backpressure, such as a rotary vane pump, will allow water to flow at a constant rate regardless of the increase in differential pressure. Based on the data acquired throughout Jonah Melegrito's thesis, it is also recommended that a better, more stable flow meter should be implemented in order to accurately track the flow rate in the system.

If the capability of the filters is not enough, switching the capsule filter with a membrane filter is a step towards the right direction. With a membrane filter, backwashing becomes more effective, allowing for more usage of each filter. The theory presented in this thesis will be more accurate, if replaced, because it is based on the modeling of a membrane filter.

# References

Abd El Aleem, F., Al-Sugair, K., & Alahmad, M. (1998). Biofouling problems in membrane processes for water desalination and reuse in Saudi Arabia. International Biodeterioration & Biodegradation (41)1, 19-23.

Anirban Roy, S. D. (2015). Resistance-in-series model for flux decline and optimal conditions of Stevia extract during ultrafiltration using novel CAP-PAN blend membranes. Food and Bioproducts Processing, 94, 489-499.

Benet, E., Badran, A., Pellegrino, J., & Vernerey, F. (2017). The porous media's effect on the permeation of elastic (soft) particles. Journal of Membrane Science, (535), 10-19.

Chen, V., Fane, A., Madaeni, S., & Wenten, I. (1997). Particle deposition during membrane filtration of colloids: transition between concentration polarization and cake formation. Journal of Membrane Science, (125), 109-122.

DFRobot. (2019). Retrieved from DFRobot: https://wiki.dfrobot.com/Turbidity_sensor_SKU__SEN0189

DFRobot. (2019). Retrieved from DFRobot: https://wiki.dfrobot.com/Water_Flow_Sensor_-_1_8__SKU__SEN0216

Dragan Petrovic, C. M. (2011). On the particle size distributions of diatomaceous earth and perlite granulations. Strojniski vestnik, 843-850.

EBay capsule filter. (2019). Retrieved from EBay: https://www.ebay.com/itm/5-micron-Capsule-Ink-Filter-for-LIYU-JHF-Myjet-Allwin-DGI-Printers/172475309276?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

EBay diaphragm pump. (2019). Retrieved from EBay: https://www.ebay.com/itm/Peristaltic-Pump-Diaphram-Pump-Submersible-for-Arduino-Raspberry-Pi-Projects/183034848690?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

Elmoe, T., Tricoli, A., Grunwaldt, J., & Pratsinis, S. (2009). Filtration of nanoparticles: Evolution of cake structure and pressure-drop. Journal of Aerosol Science, (40), 965-981.

Erdim, E., Akgiray, O., & Demir, I. (2015). A revisit of pressure drop-flow rate correlations for packed bed of spheres. Powder Technology, (283), 488-504.

Fotovati, S., Hosseini, S., Tafreshi, H., & Pourdeyhimi, B. (2011). Modeling instantaneous pressure drop of pleated thin filter media during dust loading. Chemical Engineering Science, (66), 4036-4046.

Freescale Semiconductor. (2009, September). Retrieved from NXP: https://www.nxp.com/docs/en/data-sheet/MPX5500.pdf

Herzberg, M., & Elimelech, M. (2007). Biofouling of reverse osmosis membranes: Role of biofilm-enhanced osmotic pressure. Journal of Membrane Science, (295)1-2, 11-20.

Iritani, E., Katagiri, N., Tsukamoto, M., & Hwang, K. (2014). Determination of cake properties in ultrafiltration of nano-colloids based on single step-up pressure filtration test. AIChE Journal, (60)1, 289-299.

Kroemer, G., Galluzzi, L., Vandanabeele, P., Abrams, J., Alnemri, E., Baehrecke, E., . . . Piacentini, M. (2009). Classification of cell death: recommendations of the nomenclature committee on cell death 2009. Cell Death and Differentiation, (16), 3-11.

LabSmith. (2018). "Basic Microfluidic Education Kit". Retrieved from http://labsmith.com/education/demo/

Martin, P., & Leibovich, J. (2005). Inflammatory cells during wound repair: the good, the bad and the ugly. Trends in Cell Biology, (15)11, 599-607.

Nguyen, T., Roddick, F., & Fan, L. (2012). Biofouling of water treatment membranes: a review of the underlying causes, monitoring techniques and control measures. Membranes, (2), 804-840.

Osterroth, S., Preston, C., Markicevic, B., Iliev, O., & Hurwitz, M. (2016). The permeability prediction of beds of poly-disperse spheres with applicability to the cake filtration. Separation and Purfication Technology, (165), 114-122.

Rock, K., & Kono, H. (2008). The inflammatory response to cell death. Annu Rev Pathol,(3), 99-126.

Tien, C., & Ramarao, B. (2013). Can filter cake porosity be estimated based on the Kozeny-Carman equation? Powder Technology, (237), 233-240.

Tomadakis, M., & Robertson, T. (2005). Viscous permeability of random fiber structures: comparison of electrical and diffusional estimates with experimental and analytical results. Journal of Composite Materials, (39)2, 163-187.

Tomadakis, M., & Sotirchos, S. (1993). Ordinary and transition regime diffusion in random fiber structures. AIChE Journal, (39)3, 397-412.

Wikswo, J., Block, F., Cliffel, D., Goodwin, C., Marasco, C., Markov, D., . . . Sherrod, S. (2013). Engineering challenges for instrumenting and controlling integrated organ-on-a-chip systems. IEEE Transactions on Biomedical Engineering, (60)3, 682-690.

# References

- "26 Pin 26 Way F/F Connector IDC Flat Rainbow Ribbon Cable." March 23, 2019. https://www.ebay.com/itm/1-4ft-26-Pin-26-Way-F-F-Connector-IDC-Flat-Rainbow-Ribbon-Cable/312535453305?epid=1567752965&hash=item48c4908e79:g:YVQAAOSwqctcloAx (accessed April 11, 2019).

- Aizenberg, J., J. Weaver, M. Thanawala, V. Sundar, D. Morse, and P Fratzl. "Skeleton of euplectella sp.: Structural hierarchy from the nanoscale to the macroscale." Science, 2005: 309:5732, 275-278.

- Antoni, D., H. Burckel, E. Josset, and G. Noel. "Three-dimensional cell culture: a breakthrough in vivo." The International Journal of Molecular Sciences, 2015: 16:1, 5517-5527.

- Bashur, C.A., and M. Shojaee. "Compositions including synthetic and natural blends for integration and structural integrity: engineered for different vascular graft applications." Advanced Healthcare Materials, 2017: 6(12).

- Bashur, C.A., L.A. Dahlgren, and and A.S. Goldstein. "Effect of fiber diameter and orientation on fibroblast morphology and proliferation on electrospun poly(D,L-lactic-co-glycolic acid) meshes." (Biomaterials) 2006: 27(33): p. 5681-5688.

- Basic Microfluidic Education Kit. LabSmith News. 2018. http://labsmith.com/education/demo/ (accessed January 2019).

- Baudoin, R., Griscom, L., Prot, J.M., Legallais, C., and Leclerc, E. "Behavior of HepG2/C3A cell cultures in a microfluidic bioreactor." Biochem, 2011: 53, 172-181.

- Birthare, K., M. Shojaee, C.G. Jones, J.R. Brenner, and C.A Bashur. "Collagen incorporation within electrospun conduits reduces lipid oxidation and impacts conduit mechanics." Biomedical Materials, 2016: 11(2): 025019.

- Burgess, W.H., and T. Maciag. "The heparin-binding (fibroblast) growth factor family of proteins." Annual Review of Biochemistry, 1989: 58:1, 576-606.

- "Cereal and Dry Food Storage Container." April 11, 2019. https://www.ebay.com/itm/Set-of-4-Cereal-Dry-Food-Storage-Container-16-9-Cup-135-2oz-FREE-Chalkb-/153216576441 (accessed April 11, 2019).

- Chemglass Life Sciences. "GluCell Glucose Monitoring System." April 22, 2019. https://chemglass.com/gluccell-glucose-monitoring-system?AspxAutoDetectCookieSupport=1 (accessed April 2019, 2019).

- Chen, F.-M., and X. Liu. "Advancing biomaterials of human origin for tissue engineering." Polymer Science, 2016: 53:1, 86-168.

- Chen, K. G., B. S. Mallon, R. D. G McKay, and P. G. Robey. "Human pluripotent stem cell culture: considerations for maintenance, expansion, and therapeutics." Cell Stem Cell, 2014: 14:1, 13-26.

- "DAQ SC-205X Series User Manual ." 1998. http://www.ni.com/pdf/manuals/371176a.pdf (accessed 4 11, 2019).

- Davidson, M. Cell Structure. November 13, 2015. https://micro.magnet.fsu.edu/cells/index.html (accessed April 24, 2019).

- "Diaphragm Pump for Arduino Projects." March 15, 2019. https://www.ebay.com/itm/Peristaltic-Pump-Diaphram-Pump-Submersible-for-Arduino-Raspberry-Pi-Projects/183034848690?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649 (accessed April 11, 2019).

- Engelka, K.A., Maciag, T. Angiogenesis and Cardovascular Disease. New York: Oxford Univeristy Press, 1999.

- "Flow Sensor Switch Meter." March 7, 2019. https://www.ebay.com/itm/6mm-Water-Flow-Sensor-Switch-Meter-Flowmeter-0-15-1-5L-min-with-340mm-Cable/163236231684?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649 (accessed April 11, 2019).

- Fogler, S.H. Elements of Chemical Reaction Engineering. 5th ed. Englewood Cliffs, NJ: Pearson Education, 2016.

- "Full Tested PCI-DIO-96." 2019. https://www.ebay.com/itm/Full-Tested-PCI-DIO-96-60-Day-Warranty-vpu/323653381446?hash=item4b5b3ead46:g:NyQAAOSw8PJcQF~J (accessed April 11, 2019).

- Halldorsson, S., E. Lucumi, R Gómez-Sjöberg, and R. Fleming. "Advantages and challenges of microfluidic cell culture in polydimethylsiloxane devices." Biosensors and Bioelectronics, 2015: 63: 218-231.

- Hinderer, S., S. Lee-Layland, and K. Schenke-Layland. "ECM and ECM-like materials — Biomaterials for applications in regenerative medicine and cancer therapy." Advanced Drug Delivery Reviews, 2016: 97:1, 260-269.

- IORodeo. IO Rodeo Smart Lab Technology. 2019. https://iorodeo.com/.

- Kimball, J.W. "Biology." Creative Commons Attribution. 2017. https://bio.libretexts.org/Bookshelves/Introductory_and_General_Biology/Book%3A_Biology_(Kimball)/Unit_03%3A_The_Cellular_Basis_of_Life/3.15%3A_Junctions_between_Cells (accessed April 9, 2019).

- "Kloehn Syringe Pump V8 Multi-Channel Syringe Pump." February 27, 2019. https://www.ebay.com/itm/KLOEHN-SYRINGE-PUMP-V8-Multi-Channel-Syringe-Pump-p-n-30488/273685108040?hash=item3fb8e75548:g:jLMAAOSwkV5ab-D6 (accessed April 16, 2019).

- Martin, I., D. Wendit, and M. Heberer. "The role of bioreactors in tissue engineering." TRENDS in Biotechnology, 2004: 22:2, 80-86.

- McCoy, R. J., and F.J. O'Brien. "Influence of shear stress in perfusion bioreactor cultures for the development of three-dimensional bone tissue constructs: a review." Tissue Engineering Part B: Reviews, 2010: 16:6.

- McKenzie, J.W., A.C. Cognata, A.N. Davis, and J.P. Wikswo. "Real-time monitoring of cellular bioenergetics with a multianalyte screen-printed electrode." Anal. Chem., 2015: 87, 7857-7864.

- Mencía C., Irene, C.M. Curtin, G. Shawd, J.M. Murphy, G. P Duffy, and F. J. O'Brien. "A novel collagen-nanohydroxyapatite microRNA-activated scaffold for tissue engineering applications capable of efficient delivery of both miR-mimics and antagomiRs to human mesenchymal stem cells." Journal of Controlled Release, 2015: 200:1, 42,51.

- "Multiposition Electric Actuator Instruction Manual." 2000. https://www.vici.com/support/manuals/mpact.pdf (accessed April 11, 2019).

- National Institute of Biomedical Imaging and Bioengineering. Tissue Engineering and Regenerative Medicine. 2018. https://www.nibib.nih.gov/science-education/science-topics/tissue-engineering-and-regenerative-medicine.

- "National Instruments R1005050 Ribbon Cable." September 12, 2018. https://www.ebay.com/itm/National-Instruments-R1005050-Ribbon-Cable-182762-02-2M/253685193690?epid=1423788303&hash=item3b10d0dbda:g:SOQAAOSwdm1bHydF&frcectupt=true (accessed April 11, 2019).

- "National Instruments SC-2054 Module." March 31, 2019. https://www.ebay.com/itm/National-Instruments-SC-2054-Module-Used/332695822243?epid=1955618879&hash=item4d76375fa3:g:JbEAAOSwlHJbLa~T (accessed April 11, 2019).

- Orntiz, D., J. Xu, J. S. Colvin, D.G. McEwen, C.A. MacArthur, and G. Coulier. "Receptor Specificity of the Fibroblast Growth Factor Family." Journal of Biological Chemistry, 1996: 271:25, 15292-15297.

- Puleo, C.M., H. Yeh, and T.H Wang. "Applciations of MEMS technologies in tissue engineering." Tissue Engineering, 2007: 13:12, 2839-2854.

- Radisic, M., A. Marsano, R. Maidhof, Y. Wang, and G. Vunjak-Novakovic. "Cardiac tissue engineering using perfusion bioreactor systems." Nature Protocols, 2008: 3:1, 719-738.

- Reis, R.L., S. Pina, and J.M. Oliveira. "Natural based nanocomposites for bone tissue engineering and regenerative medicine: a review." Advanced Materials, 2015: 27:7, 1143-1169.

- "ReliOn Prime Bllod Glucose Monitoring System." April 22, 2019. https://www.walmart.com/ip/ReliOn-Prime-Blood-Glucose-Monitoring-System-Red/20752267?wmlspartner=wlpa&selectedSellerId=0&wl13=974&adid=22222222227015223232&wl0=&wl1=g&wl2=c&wl3=40754015672&wl4=pla-78606320192&wl5=9011817&wl6=&wl7=&wl8=&wl9=pla&wl10=817503 (accessed April 22, 2019).

- Ruan, J., L. Wang, M. Xu, D. Cui, X. Zhou, and D. Liu. "Fabrication of a microfluidic chip containing dam, weirs and gradient generator for studying cellular response to chemical modulation." Materials Science and Engineering C, 2009: 29:3, 674-679.

- Schügerl, K., and Kretzmer, G. "Response of mammalian cells to shear stress." Applied Microbiology and Biotechnology, 1991: Volume 34, Issue 5, pp 613–616.

- Shojaee, M., K.B. Wood, L.K. Moore, and C.A Bashur. "Peritoneal pre-conditioning reduces macrophage marker expression in collagen-containing engineered vascular grafts." Acta Biomaterialia, 2017: 64:80.

- "Silicone Tubing Clear Food Pharmaceautical Grade." March 27, 2019. https://www.ebay.com/itm/Silicone-Tubing-Clear-Food-Pharmaceutical-Grade-Peristaltic-Pump-Hose-All-Sizes/113411717461?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649 (accessed April 11, 2019).

- Sprio, S., A. Ruffini, F. Valentini, T. D’Alessandro, M. Sandri, and S. Panseri. "Biomimesis and biomorphic transformations: new concepts applied to bone regeneration." Journal of Biotechnology, 2011: 156:1, 347-355.

- Tro, N. J. Chemistry: A Molecular Approach. 3rd ed., London: Pearson, 2014.

- "Tygon Tubing." April 3, 2019. https://www.ebay.com/itm/TYGON-Tubing-4mm-I-D-49-ft-Clear-Flexible-ACF1S1518/253727657982?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649 (accessed April 11, 2019).

- "Union Hose Barb Fitting Connector." April 8, 2019. https://www.ebay.com/itm/1-8-to-1-4-4mm-6mm-Plastic-Straight-Union-Hose-Barb-Fitting-Connector-Reducer/323483672449?hash=item4b51211f81&enc=AQADAAADIFjVrDbVsZ8oH%2F8PNHtt9VX4%2Fw7FZcmMuqsX8uaFEduVcwN%2BwivWRaw4bNHA4BApm59cqPfao%2F4ts9ZZJ84NGViw9TNokSUcxKT (accessed April 13, 2019).

- "UNO R3 Board with USB Cable for Arduino." March 10, 2019. https://www.ebay.com/itm/New-ATmega328P-CH340G-UNO-R3-Board-USB-Cable-Compatible-with-Arduino/381350929204?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649 (accessed April 11, 2019).

- "Valco VICI Microelectric Valve Actuator." April 4, 2019. https://www.ebay.com/itm/Lot-of-4-Valco-VICI-EMHMA-CE-Microelectric-Valve-Actuator-W-CHEMINERT-10-Port/273778250061?hash=item3fbe74914d:g:zYsAAOSwImRYdsCJ (accessed April 11, 2019).

- Vernier. "LabQuest 2 Hardware." 2019. https://www.vernier.com/products/interfaces/labq2/hardware/ (accessed April 11, 2019).

- Vernier. "SpectroVis Plus Spectrophotometer." 2019. https://www.vernier.com/products/sensors/spectrometers/visible-range/svis-pl/ (accessed April 11, 2019).

- Wagner, R.C., and F.E. Hosler. "Blood vessels - endothelial tight junction." 1998. http://udspace.udel.edu/handle/19716/1640 (accessed April 2019).

- Webber, M.J., Khan, O.F., S.A. Sydlik, B.C. Tang, and R. Langer. "A perspective on the clinical translation of scaffolds for tissue engineering." Annals of Biomedical Engineering, 2015: 43:3, 641-656.

- Wikswo, J.P., J.W. McKenzie, and D.E. Cliffel. "Engineering challenges for instrumenting and controlling integrated organ-on-a-chip systems,." IEEE Trans. Biomed Eng., 2013: 60, 682-690, 2013.

- Xiao, X.-F., X.-Q. Jiang, and L.-J. Zhou. "Surface modification of poly ethylene glycol to resist nonspecific adsorption of proteins." Chinese Journal of Analytical Chemistry, 2013: 41:3, 445-453.

- Yeatts, A.B., Fisher, J.P. "Bone tissue enineering bioreactors: dynamic culture and the influence of shear stress." Bone, 2011: 48:1, 171-181.

- Yoshimotoa, H., Y.M. Shina, H. Teraia, and J.P. Vacanti. "A biodegradable nanofiber scaffold by electrospinning and its potential for bone tissue engineering." Biomaterials, 2003: 24:12, 2077-2082.

Appendixes

Appendix 2.1 -  Questions and Issues Sheet

Technical & Engineering

- Will the valves fail? (ie. burst randomly, open and close involuntarily)

- Will the pump sequence fail?

- Will the dye leak from its reservoir?

- Does the liquid in the oxygen tank have to be periodically replaced?

- How will the microscopes be placed in order to take the images?

- Buy strain gages that can be put under each bioreactor.

- Isolation valves are needed to cut flow while weighing.

- How will trays be wired and plumbed?

- Will all equipment be able to withstand salt water?

- Will there be leakages of water or PBS?

- What material will the outer aquarium of the bioreactor be made of in order to properly function as a heating jacket?

- What material will the inner aquarium be made of to withstand any corrosion from PBS?

- Mass measurement  ($12 method)

  - How sturdy is the glass tubing that is to be used?

  - Is there a possibility for cells to accumulate in the glass tubing thus hindering the mass measurement process?

  - Will the peristaltic pump malfunction?

  - How often does the sensor have to be calibrated?

Biological

- Will salt crystals be deposited in the valves?

- If one bioreactor is falling behind, will all of them have to be stopped to fixed the spoiled one?

- Is there a quantitative way to measure the growth of cells without being invasive?

- If there is a malfunction with the valves to remove the toxic waste and that waste interacts with the cells, what is our preventative measure?

- What is the test we will be using to determine the reusability of the recycled materials?

- Will reusing the materials slow down the cells growth?

Safety

- Will there be a way to detect leakages in the aquarium?

- Will there be a manual failsafe in case leakages cannot be detected by sensors?

- Will there be a double casing around the reservoirs of the dyes to prevent leaking of the dyes?

Quality

Legal

Regulatory

- How will maintenance be performed on the system?

- How often will maintenance be performed on the system?

- Do all bioreactors have to be maintained at the same time or can some still be running?

Social Impact

- Will this be more time efficient than the traditional tissue engineering?

- How will this project affect the way tissue engineering is looked at?

- Will this system improve current research practices?

Health

- How to minimize the chemical exposure of dye to the tissue?

- How long will the expected life of each bioreactor be?

Environmental

- Will there be any biohazardous waste produced?

- What can be recycled?

Economic

- How much will it cost to pay an external agent to get rid of the biohazardous waste?

Appendix 2.3 - Fiber Optic LED Sensor Used for Cellular Imaging

Appendix 2.4 – Complete List of Mass and Energy Balances

Synthesis 10-way valve (Mstream#-component)

- M1-ppX1-pp = M4-ppX4-pp

- M2-bpX2-bp = M4-bpX4-bp

- M3-crossX3-cross = M4-crossX4-cross

Input 10-way valve (Mstream#-component)

- M4-jX4-j = M11-jX11-j (where j = pp,bp,cross)

- M5-drugX5-drug = M11-drugX11-drug

- M6-amX6-am = M11-amX11-am

- M7-glucX7-gluc = M11-glucX11-gluc

- M8-dyeAX8-dyeA = M11-dyeAX11-dyeA

- M9-dyeBX9-dyeB = M11-dyeBX11-dyeB

- M10-waterX10-water = M11-waterX11-water

Pre-Bubble Trap mix

- M11-waterX11-water + M12-oxX12-ox = M13-13X13-13

Pre-Bioreactor 10-way valve

- M14-jX14-j = M15-jX15-j (where j=pp,bp,cross,drug,am,gluc,dyeA,dyeB,water)

- M14-jX14-j = M16-jX16-j

- M14-jX14-j = M17-jX17-j

- M14-jX14-j = M18-jX18-j

- M14-jX14-j = M19-jX19-j

- M14-jX14-j = M20-jX20-j

- M14-jX14-j = M21-jX21-j

Pre-Bioreactor recycle

- M15-jX15-j + MR1XR1 = M22X22

- M16-jX16-j + MR2XR2 = M26X26

- M17-jX17-j + MR3XR3 = M30X30

- M18-jX18-j + MR4XR4 = M34X34

- M19-jX19-j + MR5XR5 = M38X38

- M20-jX20-j + MR6XR6 = M42X42

- M21-jX21-j + MR7XR7 = M46X46

Pre-Bioreactor return

- M22X22 + M25X25 = M23X23

- M26X26 + M29X29 = M27X27

- M30X30 + M33X33 = M31X31

- M34X34 + M37X37 = M35X35

- M38X38 + M41X41 = M39X39

- M42X42 + M45X45 = M43X43

- M46X46 + M49X49 = M47X47

Post-Bioreactor 10-way valve

- MA1-A1XA1-A1 = M50-A1X50-A1

- MA2-A2XA2-A2 = M50-A2X50-A2

- MA3-A3XA3-A3 = M50-A3X50-A3

- MA4-A4XA4-A4 = M50-A4X50-A4

- MA5-A5XA5-A5 = M50-A5X50-A5

- MA6-A6XA6-A6 = M50-A6X50-A6

- MA7-A7XA7-A7 = M50-A7X50-A7

Purifier

- M50-AnX50-An = M51-51X51-51 + M52-52X52-52 (where n = 1 to 7)

Recycle Stream 10-way valve

- M52-52X52-52 = MR1XR1

- M52-52X52-52 = MR2XR2

- M52-52X52-52 = MR3XR3

- M52-52X52-52 = MR4XR4

- M52-52X52-52 = MR5XR5

- M52-52X52-52 = MR6XR6

- M52-52X52-52 = MR7XR7

Appendix A2.5 - Instrumentation Specifications for Tissue Engineering Test Bed

All http://my.fit.edu/~jbrenner/ web sites have 
name = fltech and password = brenner

Download the appropriate CP210x driver for your operating system from

https://www.silabs.com/products/development-tools/software/usb-to-uart-bridge-vcp-drivers

Programs to download

1. Arduino

A. Software download - https://www.arduino.cc/en/Main/Donate
	B. Training

https://www.colorado.edu/physics/phys3330/phys3330_sp18/resources/guide-lab-10.pdf

downloaded as intro-to-arduino-guide-lab-10.pdf to
	http://my.fit.edu/~jbrenner/bashur/3dprinting/Arduino/

Notes from a 3-hour course taught by Dr. Julius Chatterjee of both Florida 	Tech 	& Project-Based Learning of Palm Bay, FL to Prof. Jim Brenner of Florida Tech
	http://my.fit.edu/~jbrenner/bashur/3dprinting/Arduino/
	intro_to_arduino_rev4.pptx

2. Repetier Host - Software download

3. Slic3r
	A. Software download
	B. New default settings for GBotz 3D printer
		http://my.fit.edu/~jbrenner/bryangoldstein/GBotzconfig.ini

4. Marlin
	A. Software download

Appendix A2.5.1 - Within Bioreactors

Appendix A2.5.1.1 - Temperature sensor for inside reactors

A. Sensor - DS18B20

Figure A2.5.1.1.1 - Temperature sensor

B. Arduino shield (seen in above figure)

C. Arduino code to download or write
	http://static.cactus.io/downloads/code/ds18b20/DS18B20_Sketch.ino		http://static.cactus.io/downloads/library/ds18b20/cactus_io_DS18B20.zip

D. Sensor documentation
 		https://datasheets.maximintegrated.com/en/ds/DS18B20.pdf

E. Wiring

Figure A2.5.1.1.2 - Wiring diagram for temperature sensor

F. Testing with Arduino/computer
G. Web site for purchase

https://www.ebay.com/itm/DS18B20-Waterproof-Digital-Temperature-Sensor-With-Adapter-Module-for-Arduino-TW/131899447876

H. Cost per item: $4.29 + $0 S/H including shield

I. Dimensions:

Appendix A2.5.1.2 - Liquid level measured with 5 kg load cell

A. Sensor

Figure A2.5.1.2.1 - Load cell sensor used to measure reactor mass

B. Arduino shield (included in above figure)

C. Arduino code to download or write
 	https://gist.github.com/matt448/14d118e2fc5b6217da11

D. Sensor documentation
https://halckemy.s3.amazonaws.com/uploads/attachments/392653/hx711_datasheet_english.pdf

https://www.instructables.com/id/How-to-Interface-HX711-Balance-Module-With-Load-Ce/

E. Wiring

Figure A2.5.1.2.2 - Wiring diagram for load cell sensor used to measure reactor mass

Figure A2.5.1.2.3 - Load cell wiring

F. Testing with Arduino/computer
 G. Web site for purchase

https://www.ebay.com/itm/Scale-Load-Cell-Weight-Weighing-Sensor-HX711-AD-Module-Metal-Shied-2-5-10-20KG/272817516318?ssPageName=STRK%3AMEBIDX%3AIT&var=571961640473&_trksid=p2057872.m2749.l2649

https://i.ebayimg.com/images/g/jpEAAOSw1blZnYBW/s-l1600.jpg

H. Cost per item: $4.99 + $1.88 S/H

I. Dimensions:

Figure A2.5.1.2.4 - Dimensions of load cell sensor

https://i.ebayimg.com/images/g/jpEAAOSw1blZnYBW/s-l1600.jpg

Appendix A2.5.1.3 - pH

Appendix A2.5.1.3.1 - pH Sensor and Shield

A. Sensor

Figure A2.5.1.3.1.1 - pH sensor and shield

B. Arduino shield (included in above figure)

C. Arduino code to download or write
	https://www.atlas-scientific.com/_files/code/Arduino-Uno-pH-sample-code.pdf

D. Sensor documentation
	https://www.atlas-scientific.com/_files/_datasheets/_probe/pH_probe.pdf

E. Wiring
	Shield: https://www.atlas-scientific.com/_files/instructions/ezo-ph-wiringdiagram.pdf

F. Testing with Arduino/computer
G. Web site for purchase

https://www.ebay.com/itm/Liquid-PH0-14-Value-Detect-Sensor-Module-PH-Electrode-Probe-BNC-for-Arduino/172462790633?var=471310656103

H. Cost per item: $14.54 + $1.77 S/H incl. shield

I. Dimensions: 42 x 32 x 20 mm

Figure A2.5.1.3.1.3 - Dimensions for Atlas Scientific pH sensor

https://www.atlas-scientific.com/_files/_datasheets/_probe/PH_probe.pdf

Appendix A2.5.1.3.2 - pH/ORP Electrode Storage Solution

A. Item - Biopharm pH/ORP Electrode Storage Solution 8 oz (250 mL)

Figure A2.5.1.3.2.1 - pH/ORP electrode storage solution https://i.ebayimg.com/images/g/83cAAOSwPN9bIsvQ/s-l1600.jpg

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Biopharm-pH-ORP-Electrode-Storage-Solution-8-oz-250-mL/182697284389?epid=2154319355&hash=item2a899b5325%3Ag%3A83cAAOSwPN9bIsvQ&_sacat=0&_nkw=Biopharm+pH%2FORP+Electrode+Storage+Solution+8+oz+%28250+mL%29&_from=R40&rt=nc&_trksid=m570.l1313

H. Cost per item: $11.95 + $0 S/H

I. Dimensions: 250 ml (8 oz) Poly Bottle

J. SDS: https://images-na.ssl-images-amazon.com/images/I/81dld4MjoUL.pdf

Appendix A2.5.1.3.3 - pH = 4, 7, and 10 Calibration Solutions

A. Item - "Biopharm Buffer Calibration Solution Kit 3-Pack of 250 ml (8oz) each pH 4, 7 and"

Figure A2.5.1.3.3.1 - pH = 4, 7, and 10 calibration solutions

F. Testing with Arduino/computer

G. Web site for purchase

H. Cost per item: $22.95 + $0 S/H

I. Dimensions:

J. SDS: https://images-na.ssl-images-amazon.com/images/I/81xaRo05-2L.pdf

Appendix A2.5.1.4 - Electrical Conductivity
Appendix A2.5.1.4.1 - Generic Electrical Conductivity Sensor

A. Sensor

https://www.ebay.com/itm/Arduino-Conductivity-Sensor/132438394283?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

Figure A2.5.1.4.1.1 - Electrical conductivity sensor and shield

B. Arduino shield

https://www.atlas-scientific.com/_files/_datasheets/_circuit/EC_EZO_Datasheet.pdf

C. Arduino code to download or write

https://www.atlas-scientific.com/_files/code/Arduino-Uno-EC-sample-code.pdf

D. Sensor documentation

https://www.atlas-scientific.com/_files/_datasheets/_probe/EC_K_1.0_probe.pdf

E. Wiring

Figure A2.5.1.4.1.2 - Wiring diagram for generic electrical conductivity sensor & shield

https://www.ebay.com/itm/Arduino-Conductivity-Sensor/132438394283?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.atlas-scientific.com/_files/instructions/ezo-ec-wiringdiagram.pdf

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Arduino-Conductivity-Sensor/132438394283?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $80 + $0 S/H (includes shield)

I. Dimensions:

Figure A2.5.1.4.1.3 - Dimensions of Atlas Scientific electrical conductivity probe

https://www.atlas-scientific.com/_files/_datasheets/_probe/EC_probe.pdf https://www.atlas-scientific.com/_files/_datasheets/_probe/EC_K_1.0_probe.pdf

Appendix A2.5.1.4.2 - Vernier Electrical Conductivity Probe

A. Sensor:  Vernier Software Conductivity Probe

Figure A2.5.1.4.2.1 - Vernier electrical conductivity probe

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

H. Cost per item: 8 used for $111.60

I. Dimensions:

Figure A2.5.1.4.2.1 - Dimensions of Atlas Scientific electrical conductivity probe

https://www.atlas-scientific.com/_files/_datasheets/_probe/EC_probe.pdf

Appendix A2.5.1.4.3 - 10x Phosphate Buffered Saline

A. Item - 10x Phosphate Buffered Saline

Figure A2.5.1.4.3.1 - 10x phosphate buffered saline (PBS)

https://i.ebayimg.com/images/g/Al8AAOSw~xFZjkj1/s-l1600.jpg

G. Web site for purchase

https://www.ebay.com/itm/10x-Phosphate-buffered-saline-10xPBS-buffer-1L-10L/162627991283?hash=item25dd6236f3%3Am%3AmQlmBUIvIVZ6XAvoEgS38dA&var=461627257353&_sacat=0&_nkw=10x+Phosphate+buffered+saline+%2810xPBS+buffer%2C+1L+-+10L%29&_from=R40&rt=nc&_trksid=m570.l1313

https://www.ebay.com/itm/10x-Phosphate-buffered-saline-10xPBS-buffer-1L-10L/162627991283?ssPageName=STRK%3AMEBIDX%3AIT&var=461627257354&_trksid=p2057872.m2749.l2649

H. Cost per item: $30 for 1 L + $0 S/H + $75 for 4 L

I. Dimensions: 1 L

J. SDS: https://www.bioland-sci.com/PDF/Buffer_MSDS/10x%20PBS%20Buffer.pdf

Appendix A2.5.1.4.4 - Electrical Conductivity Standards Pack #1

A. Item - "Conductivity Standard 3-Pack 500 ml each 1000 uS, 10,000 uS and 100,000 uS"

Figure A2.5.1.4.4.1 - Electrical conductivity standards pack #1

https://i.ebayimg.com/images/g/IZAAAOSwZLhY27-Z/s-l1600.jpg

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Conductivity-Standard-3-Pack-500-ml-each-1000-uS-10-000-uS-and-100-000-uS/182507917359?hash=item2a7e51d02f%3Ag%3AIZAAAOSwZLhY27-Z&_sacat=0&_nkw=Conductivity+Standard+3-Pack+500+ml+each+1000+uS%2C+10%2C000+uS+and+100%2C000+uS&_from=R40&rt=nc&LH_TitleDesc=0

H. Cost per item: $44.95 + $0 S/H

I. Dimensions: 500 mL

J. SDS: https://images-na.ssl-images-amazon.com/images/I/719pQ7NioEL.pdf

Appendix A2.5.1.4.5 - Electrical Conductivity Standards Pack #2

A. Item - "Conductivity 2-point Calibration Standard 2-Pack 8oz each 100 uS and 3000 uS"

Figure A2.5.1.4.5.1 - Electrical conductivity standards pack #2

https://i.ebayimg.com/images/g/j3kAAOSwc-tY27zx/s-l1600.jpg

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Conductivity-2-point-Calibration-Standard-2-Pack-8oz-each-100-uS-and-3000-uS/182507906324?hash=item2a7e51a514%3Ag%3Aj3kAAOSwc-tY27zx&_sacat=0&_nkw=Conductivity+2-point+Calibration+Standard+2-Pack+8oz+each+100+uS+and+3000+uS&_from=R40&rt=nc&_trksid=m570.l1313

H. Cost per item: $22.95 + $0 S/H

I. Dimensions: 250 mL

J. SDS: https://images-na.ssl-images-amazon.com/images/I/719pQ7NioEL.pdf

Appendix A2.5.1.4.6 - Vernier LabQuest 2

A. Data collection device

Figure 2.5.1.4.6.1: Vernier LabQuest 2 (Vernier 2019)

B. Arduino shield: N/A

C. Arduino code to download or write: N/A

D. Item Specifications:

Figure 2.5.1.4.6.2: Vernier LabQuest 2 front panel (Vernier 2019)

Figure 2.5.1.4.6.3: Vernier LabQuest 2 ports I (Vernier 2019)

Figure A2.5.1.4.6.4: Vernier LabQuest 2 ports II (Vernier 2019)

Table A2.5.1.4.6.1: Item specifications for Vernier LabQuest2

| LabQuest2 Specifications |  |  |  |
| --- | --- | --- | --- |
| Size | 9.8 cm x 15.4 cm x 2.5 cm | Storage | 200 MB |
| Weight | 350 g | Ports | 5 sensor channels |
| Processor | 800 MHz application processor | Data acquisition | 100,000 samples per second |

E. Wiring:

G. Website for Purchase: https://www.vernier.com/products/interfaces/labq2/hardware/

H. Cost per item: $329

Appendix A2.5.1.5 - Dissolved Oxygen Sensors
A. Sensor:  Generic dissolved oxygen sensor

Figure A2.5.1.5.1 - Dissolved oxygen sensor

Vernier Software ~ Dissolved Oxygen Probe ~ DO-DIN ~ RH89

Figure A2.5.1.5.2 - Vernier dissolved oxygen probe and adapter

B. Arduino shield

https://www.atlas-scientific.com/_files/_datasheets/_circuit/DO_EZO_Datasheet.pdf?

C. Arduino code to download or write

https://www.atlas-scientific.com/_files/code/Arduino-Uno-DO-sample-code.pdf

D. Sensor documentation

Features:

1. 30s quick measurements of Dissolved Oxygen and Temperature in water.

2.Large measuring range from 0.00-20.00mg/L.

3.Large and clear LCD with backlight display, easy to read at dark.

4.Measure precisely,high accuracy, auto temperature compensation.

5.Made of premium plastic shell, durable and sturdy.

Descriptions:

1.Lightweight and portable, easy to carry.

2.Use ergonomics. It is easy and fast for user to use with simple key operation mode and comfortable hand feeling.

3.Apply in water sources, aquarium, hydroponics, spas, swimming pools and other water systems.

Specifications:

Brand:SMART SENSOR

Model:AR8010

Color: White & blue

Battery Type:4 x LR44 button type batteries

Dissolved Oxygen measuring range:0.00-20.00 mg/L

Accuracy: ± 0.4mg/L

Respond time: < 30s

Temperature measuring range:0-40℃

Solvement temperature compensation range:0-40℃

Data hold:yes

Backlight:yes

Calibration:yes

Size:140 x 40 x 40 mm

Weight:132 g

Package Informations:

Package Size: 245 x 90 x 52 mm/9.65 x 3.54 x 2.05 in

Package Weight:28 0g/9.89 oz

Package Included:

1 x Dissolved Oxygen Meter

4 x LR44 Batteries

3 x Spare Parts of Protection Cover

10 x A Type Reagent

10 x B Type Reagent

1 x Bag

1 x User Manual

https://www.atlas-scientific.com/product_pages/kits/do_kit.html

https://www.atlas-scientific.com/_files/_datasheets/_probe/DO_probe.pdf

E. Wiring

https://www.atlas-scientific.com/_files/instructions/ezo-do-Wiringdiagram.pdf

F. Testing with Arduino/computer
G. Web site for purchase

https://www.ebay.com/itm/SMART-SENSOR-Digital-Dissolved-Oxygen-Detector-Dissolved-Oxygen-Meter-G2P0/352685214352?hash=item521dad4a90:g:uOYAAOSwTFRc-OaL

https://www.ebay.com/itm/SMART-SENSOR-Dissolved-Oxygen-Meter-Water-Quality-Tester-Monitor-for-Aquarium-KZ/254225168670?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/Vernier-Software-Dissolved-Oxygen-Probe-DO-DIN-RH89-/312007425022?_ul=AR

https://www.atlas-scientific.com/product_pages/kits/do_kit.html

H. Cost per item: $86.26 excl. accessories;

$106.39 incl. accessories for new generic sensors

Cost per item: 7 for $279.93 for used Vernier sensors

Cost per item: $283/kit + $0 S/H for Atlas Scientific sensors

Cost per shield: $46 + $0 S/H (included with kit) for Atlas Scientific sensors

I. Dimensions:

Figure A2.5.1.5.3 - Dimensions of Atlas Scientific dissolved oxygen probe

https://www.atlas-scientific.com/_files/_datasheets/_probe/DO_probe.pdf

Appendix A2.5.1.6 - ORP

A. Sensor
	Figure A2.5.1.6.1 - Oxidation reduction potential (ORP) sensor

B. Arduino shield

Figure A2.5.1.6.2 - Atlas Scientific shield for oxidation reduction potential (ORP) sensor

https://www.atlas-scientific.com/_files/_datasheets/_circuit/ORP_EZO_datasheet.pdf?

C. Arduino code to download or write
	https://www.atlas-scientific.com/_files/code/Arduino-Uno-ORP-sample-code.pdf

D. Sensor documentation
	https://www.atlas-scientific.com/_files/_datasheets/_probe/orp_probe.pdf

E. Wiring

https://www.atlas-scientific.com/_files/instructions/ezo-orp-wiringdiagram.pdf

F. Testing with Arduino/computer
G. Web site for purchase

https://www.ebay.com/itm/ORP-Oxidation-Reduction-Potentiometer-ORP-Electrode-Measuring-Redox-Potentia/121940943525?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/ORP-Electrode-Probe-Oxidation-Reduction-Potential-Test-BNC-Q9-Connector-M0E4/323343421394 (only $17.79 each!)

H. Cost per item: $25 + $0 S/H (only 5 purchased from 1st site; remainder to be 	purchased from 2nd site); Cost per shield: $40 + $0 S/H

I. Dimensions: https://www.atlas-scientific.com/_files/dxf_files/orp-footprint.zip

Figure A2.5.1.6.3 - Oxidation-reduction potential dimensions

https://www.atlas-scientific.com/_files/_datasheets/_probe/orp_probe.pdf

Appendix A2.5.1.7 - Glucose concentration & potentiostat
Appendix A2.5.1.7.1 - Glucose concentration

A. Sensor

Figure A2.5.1.7.1.1:  Pine Research three-electrode chip used for sensing single analytes

https://pineresearch.com/shop/products/electrodes/screen-printed-electrodes/carbon-spes/

Screen-Printed Carbon Electrodes, 100-pack, 4 x 5 mm working electrode

P/N RRPE1002C-100

B. Arduino shield

C. Arduino code to download or write
D. Sensor documentation
E. Wiring
F. Testing with Arduino/computer
G. Web site for purchase

https://pineresearch.com/shop/products/electrodes/screen-printed-electrodes/carbon-spes/

H. Cost per item: $241.20 for 100 + $10.89 S/H

I. Dimensions:

Appendix A2.5.1.7.2 - IORodeo Multiplexed RodeoStat Potentiostat

A. Item: IORodeo Multiplexed RodeoStat Potentiostat

Figure A2.5.1.7.2.1 - Multiplexed RodeoStat potentiostat

B. Arduino shield

C. Arduino code to download or write:  https://bitbucket.org/iorodeo/potentiostat/src/default/

D. Sensor documentation: https://sites.google.com/iorodeo.com/potentiostat
E. Wiring
F. Testing with Arduino/computer
G. Web site for purchase

https://iorodeo.com/collections/cheapstat-open-source-potentiostat/products/rodeostat-plus

H. Cost per item: $330 + $20 S/H

I. Dimensions:

Appendix A2.5.1.7.3 - Ten pin card edge connector for three electrode electrochemical sensors

A. Item

Figure A2.5.1.7.3.1 - Ten pin card edge connector for three electrode electrochemical sensors

https://i.ebayimg.com/images/g/840AAOSwv-NWYxiQ/s-l1600.jpg
G. Web site for purchase

https://www.ebay.com/itm/10pc-IDC-Cable-type-Card-Edge-Slot-Socket-Connector-5x2P-10P-2-54mm-0-1-1A/141845554699?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $5.30 + $0.00 S/H

I. Dimensions:

# Appendix A2.5.1.7.4 - Twenty pin card edge connector

A. Sensor:  T.I. 20 Pin Card Edge Connector - NOS

# Figure A2.5.1.7.4.1 - Twenty pin card edge connector

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/T-I-20-Pin-Card-Edge-Connector-NOS-/320612119326

H. Cost per item: 40 for $71.60 + 3.99 S/H

I. Dimensions:

# Appendix A2.5.1.7.5 - Twenty pin ribbon cable sets

# A. Item - Flat Cable 20 Pin Wires IDC Ribbon 12 Ft. Long 25 mm wide 10 sets connectors

# Figure A2.5.1.7.5.1 - Twenty pin ribbon cable sets

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase	https://www.ebay.com/i/272291220385?chn=ps

H. Cost per item: 4 for $31.80 + 4.95 S/H

I. Dimensions:

Appendix A2.5.1.7.6 - Potassium Ferricyanide for Potentiostat Calibration

A. Item - "NC-0738 Potassium Ferricyanide, 100 g, Cyanotype, Photo chemical, crystal growing"

Figure A2.5.1.7.6.1 - Potassium ferricyanide for potentiostat calibration

D. Item documentation - 99.0% purity

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/NC-0738-Potassium-Ferricyanide-100g-Cyanotype-Photo-chemical-crystal-growing/272285221996?hash=item3f6576c46c%3Ag%3AVTsAAOSwuehbM9KI&_sacat=0&_nkw=NC-0738+Potassium+Ferricyanide%2C+100g%2C+Cyanotype%2C+Photo+chemical%2C+crystal+growing&_from=R40&rt=nc&_trksid=m570.l1313

https://i.ebayimg.com/images/i/272285221996-0-0/s-l140/p.jpg

H. Cost per item: $10.95 + $3.50 S/H

J. SDS: https://www.sciencecompany.com/msds/Potassium_Ferricyanide_SDS.pdf

Appendix A2.5.1.8 - Lactate concentration
A. Sensor

Figure A2.5.1.8.1.1:  Pine Research three-electrode chip used for sensing single analytes

https://pineresearch.com/shop/products/electrodes/screen-printed-electrodes/carbon-spes/

Screen-Printed Carbon Electrodes, 100-pack, 4 x 5 mm working electrode

P/N RRPE1002C-100

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring
F. Testing with Arduino/computer
G. Web site for purchase

https://pineresearch.com/shop/products/electrodes/screen-printed-electrodes/carbon-spes/

H. Cost per item: $241.20 for 100 + $10.89 S/H

I. Dimensions:

Appendix A2.5.1.9 - LED microscope camera + holder
Appendix A2.5.1.9.1 - Generic LED IP68 Endoscope

A. Sensor = LED microscope camera

Figure A2.5.1.9.1.1 - Microscope

Appendix A2.5.1.9.2 - Andonstar microscope

Andonstar Camera Stand 500X 2MP USB Digital Microscope Video Webcam Magnifier

Figure A2.5.1.9.2.1 - Andonstar microscope

B. Arduino shield - none

C. Arduino code to download or write
https://www.atlas-scientific.com/_files/code/Arduino-Uno-RGB-sample-code.pdf

D. Sensor documentation

https://www.atlas-scientific.com/_files/_datasheets/_probe/EZO_RGB_Datasheet.pdf

E. Wiring
http://www.molex.com/webdocs/datasheets/pdf/en-us/0022012057_CRIMP_HOUSINGS.pdf

F. Testing with Arduino/computer
G. Web site for purchase

https://www.ebay.com/itm/USB-Microscopes-Wireles-Endoscope-Camera-OVEHEL-8mm-HD-1200P-Waterproof-IP68-5M/163007510914?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/Andonstar-Camera-Stand-500X-2MP-USB-Digital-Microscope-Video-webcam-Magnifier/352183284439?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $46.99 + $0 S/H (2 purchased); $35.71 + $0 S/H for stands (2

purchased); $41.56 (1 purchased)

I. Dimensions:

Figure A2.5.1.9.2.2 - Fiber optic LED sensor used for cellular imaging

Appendix A2.5.1.9.3 - Vernier SpectroVis Plus Spectrophotometer

A. Spectrophotometer

Figure A2.5.1.9.3.1: Vernier SpectroVis Plus Spectophotometer (Vernier 2019)

B. Arduino shield: N/A

C. Arduino code to download or write: N/A

D. Item Specifications:

Table A2.5.1.9.3.1: Item specifications for spectrophotometer

| Spectrophotometer Specifications |  |
| --- | --- |
| Size | 15 cm x 9 cm x 4cm |
| Wavelength range | 380 nm - 950 nm |
| Light source | Incandescent white bulb, approximately 8000 hour lifetime, one step calibration |
| Wavelength accuracy: | +/- 3.0 nm (at 650 nm) and +/- 7.0 nm (at 450 nm |
| Photometric accuracy: | +/- 13.0% (as determined with nickel sulfate standards from 0.1 to 1.0 A.U.) |

E. Wiring:

Figure A2.5.1.9.3.2: Wiring diagram for absorbance measurement.

The Vernier SpectroVis Plus spectrophotometer in Figure A2.5.1.9.3.2 connects via USB to the Vernier LabQuest 2 data collection device which is powered through a standard wall socket outlet connection connected to a 12 VDC adapter.

G. Website for purchase:

https://www.vernier.com/products/sensors/spectrometers/visible-range/svis-pl/

H. Cost per item: $399

Appendix A2.5.1.9.4 - Plastic Holder for Two Glass Slides

A. Item:  Plastic Holder for Two Glass Slides
10 Pcs Microscope Glass Slide Box Case Clear White for 2 Slides

Figure A2.5.1.9.4.1 - Glass microscope slide holders

B. Arduino shield
C. Arduino code to download or write
D. Sensor documentation
E. Wiring
F. Testing with Arduino/computer
G. Web site for purchase

https://www.ebay.com/itm/10-Pcs-Plastic-Rectangle-Microscope-Glass-Holder-Slide-Box-for-5-Slides-/231247659805

H. Cost per item: 3 for $23.46 + 2 for $20

I. Dimensions:

Appendix A2.5.1.9.5 - Microscope Stand

A. Item:

USB Microscope Endoscope Aluminum Alloy Digital Magnifier Camera Bracket Table

Figure A2.5.1.9.5.1 - Endoscope Stand

B. Shield: N/A

C: Arduino code to download or write: N/A

D. Item documentation:

E. Wiring: N/A

F. Testing with Arduino/computer: N/A

G. Web Site for Purchase: https://www.amazon.com/Harris-Diatomaceous-Powder-Duster-Included/dp/B0728JL9SQ/ref=sr_1_1_sspa?keywords=diatomaceous+earth&qid=1562270832&s=gateway&sr=8-1-spons&psc=1

H. Cost Per Item: $12.95

I. Dimensions:

Appendix A2.5.1.10 - Overhead mixer for each reactor and impeller speed control

Appendix A2.5.1.10.1 - Overhead mixer

A. Sensor/instrumentation -

Electric overhead stirrer mixer corrosion resistance laboratory STEEL shaft

Figure A2.5.1.10.1.1 - Impeller speed controller

https://i.ebayimg.com/images/g/kAYAAOSw2cZbRzK0/s-l1600.jpg

B. Arduino shield - none yet

C. Arduino code - https://github.com/carrant/overheadstirrer.git

D. Sensor documentation

Power Supply: 110V, 60Hz

Stir Speed: 0-3000 rpm adjustable

Motor Power: 100 W

Rated Torque: 5.2 kg*cm

Timer Range: 0-120 min adjustable

Operating Noise: ≤ 40 dB

Total Height: 60 cm (23.6")

Stir Bar Length: 20 cm (7.9")

Stir Bar Span Width: 5.5cm (2.2")

E. Wiring - 12VDC Molex connector to 6-pin header to breadboard?

http://www.molex.com/webdocs/datasheets/pdf/en-us/0022012057_CRIMP_HOUSINGS.pdf

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Electric-overhead-stirrer-mixer-corrosion-resistance-laboratory-STEEL-shaft/183321449865

H. Cost per item: $77 + $0 S/H

I. Dimensions: Total Height: 60 cm (23.6")

Stir Bar Length: 20 cm (7.9")

Stir Bar Span Width: 5.5 cm (2.2")

0 - 3000 rpm

Appendix A2.5.1.10.2 - Rotating Teflon shaft impeller for each bioreactor

A. Item -

"PTFE Overhead Mixer Stirrer, Length 250 mm, Diameter 7 mm, Paddle width 40 mm"

Figure A2.5.1.10.2.1 - Rotating shaft impeller

https://i.ebayimg.com/images/g/OfkAAMXQTghRKG9A/s-l400.jpg

G. Web site for purchase

https://www.ebay.com/itm/PTFE-Overhead-Mixer-Stirrer-Length-250mm-Diameter-7mm-Paddle-width-40mm/262812895314?hash=item3d30de8852%3Ag%3AOfkAAMXQTghRKG9A&_sop=15&_sacat=0&_nkw=7+mm+PTFE+stirrer&_from=R40&rt=nc

https://www.ebay.com/itm/8x7mm-Chuck-Collet-For-Electric-Overhead-Stirrer-PTFE-Shafts-Use-Equipment/132699353550?hash=item1ee57f71ce%3Ag%3AYykAAOSwSCZbRd1y&_sacat=0&_nkw=7+mm+PTFE+stirrer&_from=R40&rt=nc&_trksid=m570.l1313

PTFE Stirring Rod For Electric Overhead Stirrer Mixer Shaft L 250mm x W 40mm 7mm

https://www.ebay.com/itm/PTFE-Stirring-Rod-For-Electric-Overhead-Stirrer-Mixer-Shaft-L-250mm-x-W-40mm-7mm/122771247279?epid=2289342710&hash=item1c95bc90af%3Ag%3AQbkAAOSwAC1Z7vxs&_sop=15&_sacat=0&_nkw=7+mm+PTFE+stirrer&_from=R40&rt=nc  $6.68

PTFE overhead mixer stirrer 8mm shaft, 500mm long (20")

https://www.ebay.com/itm/PTFE-overhead-mixer-stirrer-8mm-shaft-500mm-long-20/111167239929?hash=item19e21576f9%3Ag%3AudgAAOxyoA1RI261&_sop=15&_sacat=0&_nkw=8+mm+PTFE+stirrer&_from=R40&rt=nc&LH_TitleDesc=0  $19.99!

https://www.ebay.com/itm/Length-250mm-500mm-PTFE-Stir-Paddle-Oar-Teflon-Stirrer-Stirring-Blade-Laboratory/263868908888?hash=item3d6fd00558:m:mIGmABEo5RWb4dN0Mk_2QTg&var=563245734563

H. Cost per item: $6.40 + $1 S/H x 40 for 300 mm long, 7 mm diameter rod, 40 mm paddle width

Cost per item: $7.99 + $0 S/H for 250 mm long rods

I. Dimensions: 250 mm long, 7 mm diameter rod, 40 mm paddle width; 300 mm long, 7 mm diameter rod, 40 mm paddle width is preferred

Appendix A2.5.1.10.3 - Chuck Collet for Stirrers

A. Item
 - "8x7mm Chuck Collet For Electric Overhead Stirrer PTFE Shafts Use Equipment"

Figure A2.5.1.10.3.1 - Chuck collet for stirrers

https://i.ebayimg.com/images/g/YykAAOSwSCZbRd1y/s-l1600.jpg

G. Web site for purchase

https://www.ebay.com/itm/8x7mm-Chuck-Collet-For-Electric-Overhead-Stirrer-PTFE-Shafts-Use-Equipment/132699353550?hash=item1ee57f71ce%3Ag%3AYykAAOSwSCZbRd1y&_sop=15&_sacat=0&_nkw=7+mm+PTFE+stirrer&_from=R40&rt=nc

H. Cost per item: $5.88 + $0 S/H

I. Dimensions: Converts 8 mm shaft to 7 mm

Appendix A2.5.1.10.4 - Tachometer for stirrers

A. Sensor
https://www.ebay.com/itm/Precise-LCD-Display-Tachometer-RPM-Measure-Tool-Car-Motorcycle-Speed-Timer/173187481235?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

B. Arduino shield

https://www.ebay.com/itm/3pcs-IR-Optical-Limit-Switch-Endstop-Tachometer-Speed-Sensor-Module-for-Arduino/292168545216?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

Figure A2.5.1.10.4.1 - Tachometer for stirrers

C. Arduino code to download or write
https://github.com/CrossTheRoadElec/Phoenix-Documentation.git

D. Sensor documentation
Material: ABS

Color: Black

Resolution: 0.1 hrs. / 10 RPMs

Reading: 9999.9 hrs. / 99999 RPMs

Accuracy: 1 minute or 10 RPM, Quartz

Operating temperature: Standard -30oC-85oC

Power supply: 1 * CR2450 battery

Shield: This "sensor" triggers when something comes between the IR emitter and receptor. Generally used as non-contact limit switches/endstops/proximity sensors. Also can be used to build a speed sensor (tachometer).

E. Wiring

F. Testing with Arduino/computer
G. Web site for purchase

H. Cost per item: 	Sensor:  $5.36 + $3.09 S/H	Shield: 3 for $6.99 + $0 S/H

I. Dimensions:		Sensor Size:7 3.5*47*20 mm	Shield Size:

Appendix A2.5.1.10.5 - Teflon adapters for rotating shafts

Figure A2.5.1.10.5.1 - Teflon adapters for rotating shafts

https://i.ebayimg.com/images/g/NHIAAOSwC9VZhThq/s-l400.jpg

G. Web site for purchase

https://www.ebay.com/itm/PTFE-Standard-Stopper-Teflon-Stirrer-Bearing-Adapter-Stopper-Mixing-Plug/292167696489?ssPageName=STRK%3AMEBIDX%3AIT&var=591107057739&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/PTFE-Standard-Stopper-Teflon-Stirrer-Bearing-Adapter-Stopper-Mixing-Plug-U/323261620747?ssPageName=STRK%3AMEBIDX%3AIT&var=512328836768&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/14-PTFE-Standard-Stopper-14-20-Stirrer-bearing-adapter-7MM-Lab-PTFE-ware/151269582134?hash=item23385ea536%3Ag%3AQKgAAOxyUgtTPD5u&_sacat=0&_nkw=PTFE+Standard+Stopper+Teflon+Stirrer&_from=R40&rt=nc&_trksid=m570.l1313

H. Cost per item: $6.40 + $4 S/H for 1st site; $6.90 + $3 S/H for 2nd site; $7.71 + $0 S/H for 3rd site (3rd site is for 7 mm; others are for 8 mm)

I. Dimensions: For 14/20 glass joints

Appendix A2.5.1.11.1 - 1 kg load cell for measuring cell mass + scaffold

A. Sensor

Figure A2.5.1.11.1.1 - Load cell sensor used to measure reactor mass

B. Arduino shield (included in above figure)

C. Arduino code to download or write
 	https://gist.github.com/matt448/14d118e2fc5b6217da11

D. Sensor documentation
https://halckemy.s3.amazonaws.com/uploads/attachments/392653/hx711_datasheet_english.pdf

https://www.instructables.com/id/How-to-Interface-HX711-Balance-Module-With-Load-Ce/

https://www.hackster.io/MOHAN_CHANDALURU/hx711-load-cell-amplifier-interface-with-arduino-fa47f3

E. Wiring

Figure A2.5.1.11.1.2 - Wiring diagram for load cell sensor used to measure reactor mass

Figure A2.5.1.11.1.3 - Load cell wiring

F. Testing with Arduino/computer
G. Web site for purchase

https://www.ebay.com/itm/Scale-Load-Cell-Weight-Weighing-Sensor-HX711-AD-Module-Metal-Shied-2-5-10-20KG/272817516318?ssPageName=STRK%3AMEBIDX%3AIT&var=571961640473&_trksid=p2057872.m2749.l2649

H. Cost per item: $4.99 + $1.88 S/H

I. Dimensions:

Figure A2.5.1.11.1.4 - Dimensions of 1 kg load cell for measuring cell mass + scaffold

https://i.ebayimg.com/images/g/jpEAAOSw1blZnYBW/s-l1600.jpg

Appendix A2.5.1.11.2 - Vibration Isolation Granite

A. Item: 1x1 Absolute Black Granite Polished Mosaic Tile

B. Shield: N/A

C: Arduino code to download or write: N/A

D. Item documentation:

E. Wiring: N/A

F. Testing with Arduino/computer: N/A

G. Web Site for Purchase:

https://mosaictiledirect.net/1x1-absolute-black-granite-polished-mosaic-tile.html

H. Cost Per Item: $16.00

I. Dimensions: 1 foot x 1 foot of 1 inch x 1 inch squares

Appendix A2.5.1.11.3 - Vibration Sensor

A. Item:  KEYES Analog Piezoelectric Ceramic Piezo Vibration Sensor For Arduino

Figure A2.5.1.11.3.1 - Vibration Sensor

B. Arduino shield (attached to vibration sensor)

C. Arduino code to download or write
D. Sensor documentation

Introduction:

The analog vibration sensor based on the piezoelectric ceramic piece is an inverse transformation process that uses the piezoelectric ceramic to generate vibration. When the piezoelectric ceramic piece vibrates, an electrical signal is generated, which is combined with the Arduino special sensor expansion board, and the Arduino analog port is used. It can sense weak vibration electric signals and realize interactive works related to vibration, such as electronic drum interactive works. Connect the analog voltage ceramic vibration sensor to the analog port A0 of the Arduino UNO controller according to the program, and observe the output value of the serial port when the vibration level is different, which can realize the interactive work related to vibration.

Parameter:

1. Working voltage: 3.3 V or 5 V

2. Working current: < 1 mA

3. Operating temperature range: -10°C ~ +70°C

4. Interface type: analog signal output

5. Pin definition: S signal output, + power supply (VCC), - ground (GND)
E. Wiring
F. Testing with Arduino/computer
G. Web site for purchase

https://www.ebay.com/itm/KEYES-Analog-Piezoelectric-Ceramic-Piezo-Vibration-Sensor-For-Arduino/133096501836?hash=item1efd2b724c:g:B0wAAOSwsyhdFGpg

H. Cost per item: $1.20 + $1.09 S/H

I. Dimensions:

Figure A2.5.1.11.3.2 - Vibration Sensor and Shield Dimensions

Appendix A2.5.1.12 - Miscellaneous Items for Bioreactor

Appendix A2.5.1.12.1 - Cereal container used for bioreactor

A. Item - Set of 4 Cereal & Dry Food Storage Container (16.9 Cup/135.2oz) + FREE

Figure A2.5.1.12.1.1 - Cereal container used for bioreactor

Figure A2.5.1.12.1.2: Bioreactor vessel (Cereal and Dry Food Storage Container 2019)

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Set-of-4-Cereal-Dry-Food-Storage-Container-16-9-Cup-135-2oz-FREE-Chalkb-/153216576441

https://www.ebay.com/itm/Set-of-4-Cereal-Dry-Food-Storage-Container-16-9-Cup-135-2oz-FREE-Chalkb-/153216576441

H. Cost per item: $31.47 for 4 x 9 sets

I. Dimensions:

Table A2.5.1.12.1.1: Item specifications for bioreactor vessel

| Bioreactor Vessel Specifications |  |
| --- | --- |
| Length | 8 inches |
| Width | 3.5 inches |
| Height | 9.5 inches |
| Volume | 135.2 ounces |

Appendix A2.5.1.12.2 - 10 K Ohm Potentiometers

A) Item: 20PCS 10K ohm 3296W Trim Pot Trimmer Potentiometer 3296W-103 For Arduino

Figure A2.5.1.12.2.1 - 10K ohm potentiometers

B) Shield: N/A

C) Code (if any): None yet

D) Documentation:

20pcs 10K ohm 3296W Trim Pot Trimmer Potentiometer 3296W-103

Resistor Tolerance: +/-10%

Value: 10K

Very Low temperature excursion

E) Wiring Diagram: None yet

F) Testing: None yet

G) Web Site for Purchase:

https://www.ebay.com/itm/20PCS-10K-ohm-3296W-Trim-Pot-Trimmer-Potentiometer-3296W-103-For-Arduino/382245186458?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H): $2.96 for pack of 20

I) Dimensions: None yet

Appendix A2.5.1.12.3 - Waterproof USB Bulkhead Connector

A. Item - "USB Bulkhead Connector, IP-67, Type A Females *32355 PL"

Figure A2.5.1.12.3.1 - Waterproof USB Bulkhead Connector

https://i.ebayimg.com/images/g/th0AAOSwVoFavQPd/s-l1600.jpg

G. Web site for purchase

https://www.ebay.com/itm/USB-Bulkhead-Connector-IP-67-Type-A-Females-32355-PL/223017087001?hash=item33ecdab019%3Ag%3Ath0AAOSwVoFavQPd&_sacat=0&_nkw=USB+Bulkhead+Connector%2C+IP-67%2C+Type+A+Fe&_from=R40&rt=nc&LH_TitleDesc=0

https://www.ebay.com/itm/USB-Female-Socket-Plug-Panel-Mount-Adapter-USB-3-0-Waterproof-Connector-IP67/113201047236?ssPageName=STRK%3AMEBIDX%3AIT&var=413500490069&_trksid=p2057872.m2749.l2649

H. Cost per item: $11.95 + $0 S/H for 1st site; 41 for $129.15 (USB 2.0 on 2nd site); 36 for $179.64 (USB 3.0 on 2nd site)

Appendix A2.5.1.12.4 - BNC adapters to breadboard

A. Sensor - None, but used with pH, EC, DO, and ORP

B. Arduino shield

Figure A2.5.1.12.4.1 - BNC adapters to breadboard

C. Arduino code to download or write

D. Sensor documentation

Heating voltage: 5 ± 0.2V (AC -• DC)
Working current: 5-10mA
The detection concentration range: pH 0-14
The detection range of temperature: 0-80 centigrade
The response time: ≤ 5 s
Stability time: ≤ 60 s
Power consumption: ≤ 0.5W
The working temperature: -10~50 centigrade (the nominal temperature 20 centigrade)
Working humidity: 95% RH (nominal humidity 65% RH)
Service life: 3 years
Size: 42 mm x 32 mm x 20 mm
The output: analog voltage signal output

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Liquid-PH-Value-Detection-detect-Sensor-Module-Monitoring-Control-For-arduino/400926133627

H. Cost per item: $7.99 + $1.85 S/H

I. Dimensions:  42 mm x 32 mm x 20 mm

Appendix A2.5.2 - Bioreactor External Recirculation Loop

Appendix A2.5.2.1 - Flow Totalizers
Appendix A2.5.2.1.1 - 1/2" NPT Hall Effect Flow Totalizer

A. Sensor

Figure A2.5.2.1.1.1 - 1/2" NPT Hall Sensor-Based Flowmeter

https://www.ebay.com/itm/1-2-Water-Flow-Sensor-Control-Effect-Flowmeter-Hall-1-30L-min-For-Arduino-US/312183053141?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649 ?

B. Arduino shield
https://www.atlas-scientific.com/_files/_datasheets/_circuit/flow_EZO_Datasheet.pdf

C. Arduino code to download or write

https://www.atlas-scientific.com/_files/code/arduino_flow_sample_code.pdf

D. Sensor documentation
https://www.atlas-scientific.com/_files/_datasheets/_probe/1-4_flow_datasheet.pdf

Mainly used for water testing, water cooling system

Application: Water heaters, credit card machines, water vending machine, flow measurement device

High amplitude: ≥ 4.6V

Low amplitude: ≤ 0.5V

Electric strength: 1250 V/min

Insulation resistance: ≥ 100 MΩ

Hydrostatic pressure testing: ≤ 2.0 MPa

Specification:

The lowest rated working voltage: DC4.5 5V-24V

Maximum operating current: 15 mA (DC 5V)

Working voltage range: DC 5~18V

Load capacity: ≤ 10mA (DC 5V)

Use temperature: ≤ 80°C

Operating humidity range: 35%~90%RH (no frost)

Allowing pressure: pressure 2.0 MPa

Temperature: -25 ~ +80 °C

External threads: 1/2"

Outer diameter: 20 mm

Intake diameter: 9 mm

Outlet diameter: 12 mm

Package included: 1 pc x Water Flow Sensor

Note: Please allow 1-5 mm difference due to manual measurement.

E. Wiring
https://www.atlas-scientific.com/_files/instructions/ezo-do-Wiringdiagram.pdf

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/1-2-Water-Flow-Sensor-Control-Effect-Flowmeter-Hall-1-30L-min-For-Arduino-New/122418815078?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $3.03 + $0.19 S/H

I. Dimensions:

Appendix A2.5.2.1.2 - 1/4" NPT Hall Sensor-Based Flowmeter

A. Sensor - G1/4inch 0.3-3L/min Water Coffee Flow Hall Sensor Switch Meter Flowmeter Co D6A1

Figure A2.5.2.1.2.1 - 1/4" NPT Hall Sensor-Based Flowmeter

B. Arduino shield
C. Arduino code to download or write

D. Sensor documentation

Figure A2.5.2.1.2.2 – Flowmeter schematics (DFRobot, 2019)

Table A2.5.2.1.2.1 – Flowmeter specifications (DFRobot, 2019)

|  | Value | Units |
| --- | --- | --- |
| Inner Diameter | 4 | mm |
| Outer Diameter | 7 | mm |
| Proof Water Pressure | < 0.8 | MPa |
| Water Flow Range | 0.3 ~ 6 | L/min |
| Voltage Range | 5 ~ 12 | V |
| Operating Current | 15 | mA (DC 5V) |
| Insulation Resistance | > 100 | MΩ |
| Accuracy | ± 5% |  |
| Output Pulse High Level | >4.5 | VDC |
| Output Pulse Low Level | <0.5 | VDC |
| Output Pulse Duty Ratio | 50% ± 10% |  |
| Water-flow Formula | 1L = 5880 square waves |  |
| Working Humidity Range | 35% ~ 90% | RH |
| Dimensions | 58 x 35 x 26 | mm |
|  | 2.28 x 1.37 x 1.02 | inches |
| Weight | 30 | g |

E. Wiring
F. Testing with Arduino/computer
G. Web site for purchase

https://www.ebay.com/itm/G1-4inch-0-3-3L-min-Water-Coffee-Flow-Hall-Sensor-Switch-Meter-Flowmeter-Co-D6A1/173471736221?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/G1-4inch-0-3-3L-min-Water-Coffee-Flow-Hall-Sensor-Switch-Meter-Flowmeter-Co-D6A1/173471736221?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $3.13 ea. (Purchased 10 + 36)

I. Dimensions:

Appendix A2.5.2.2 – Differential Pressure Transducer

Sensor:

Figure A2.5.2.2.1 - Differential Pressure Transducer

Figure A2.5.2.2.2 – Pressure transducer operating characteristics (Freescale Semiconductor, 2009)

Figure A2.5.2.2.3 – Voltage output vs. differential pressure (Freescale Semiconductor, 2009)

Figure A2.5.2.2.4 – Pressure transducer schematics (Freescale Semiconductor, 2009)

Appendix A2.5.2.3 - Turbidity Sensor

A. Sensor
Turbidity Transducer Water Module Mixed Water Detection Sensor Liquid For Arduino

Figure A2.5.2.3.1 - Turbidity Transducer and Shield

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

Table A2.5.2.3.1 – Turbidity sensor specifications (DFRobot, 2019)

|  | Value | Units |
| --- | --- | --- |
| Operating Voltage | 5 | V; DC |
| Operating Current | 40 | mA; MAX |
| Response Time | < 500 | ms |
| Insulation Resistance | 100 | MΩ; Min |
| Output Method: |  |  |
| Analog Output | 0-4.5 | V |
| Digital Output | High/Low level signal |  |
| Operating Temperature | 5 ~ 90 | oC |
| Storage Temperature | -10 ~ 90 | oC |
| Adapter Dimensions | 38 x 28 x 10 | mm |
|  | 1.5 x 1.1 x 0.4 | inches |
| Weight | 30 | g |

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Turbidity-Transducer-Water-Module-Mixed-Water-Detection-Sensor-Liquid-F-Arduino/292638550372?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $11.99 ea. (15 + 22)

I. Dimensions:

Figure A2.5.2.3.2 - Turbidity sensor schematics (DFRobot, 2019)

Appendix A2.5.2.4.2 - Five Micron Capsule Filter

A. Sensor:  5 micron Capsule Ink Filter for LIYU / JHF / Myjet / Allwin / DGI Printers

Figure A2.5.2.4.2.1 - Five Micron Capsule Filter

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/5-micron-Capsule-Ink-Filter-for-LIYU-JHF-Myjet-Allwin-DGI-Printers/172475309276?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $12.50 ea. (36 for $387.36 + 30 for $375 + 6 for $75)

I. Dimensions:

Appendix A2.5.2.5 - Diaphragm Pumps

A. Diaphragm Pump

Figure A2.5.2.5.1: Diaphragm pump (Diaphragm Pump for Arduino Projects 2019)

B. Arduino shield: N/A

C. Arduino code to download or write: N/A

D. Item Specifications:

Figure A2.5.2.5.2: Diaphragm pump schematic (mm scale) (Diaphragm Pump for Arduino Projects 2019)

Figure A2.5.2.5.3: Diaphragm pump schematic for inlet and outlet (mm scale) (Diaphragm Pump for Arduino Projects 2019)

Table A2.5.2.5.1: Item specifications for diaphragm pump

| Diaphragm Pump Specifications |  |
| --- | --- |
| Type | submersible pump |
| Diameter air inlet/ outlet | 10 mm |
| Power | 12V (1A) |
| Weight | 0.1 kg |

E. Wiring

Figure A2.5.2.5.4: Wiring diagram for diaphragm pump. Alligator clips connect a 12 V power supply to the terminals on the diaphragm pump.

F. Web site for purchase:

https://www.ebay.com/itm/Peristaltic-Pump-Diaphram-Pump-Submersible-for-Arduino-Raspberry-Pi-Projects/183034848690?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

G. Cost per item: $4.16 + $2.30 S/H

3.6 mm Water Flow Sensor Switch Meter Flowmeter 0.15-1.5 L/min with 340 mm Cable

Appendix A2.5.2.6 - Diatomaceous Earth

A. Item: Harris Diatomaceous Earth Food Grade, 2 lb with Powder Duster

B. Shield: N/A

C: Arduino code to download or write: N/A

D. Item documentation:

E. Wiring: N/A

F. Testing with Arduino/computer: N/A

G. Web Site for Purchase: https://www.amazon.com/Harris-Diatomaceous-Powder-Duster-Included/dp/B0728JL9SQ/ref=sr_1_1_sspa?keywords=diatomaceous+earth&qid=1562270832&s=gateway&sr=8-1-spons&psc=1

H. Cost Per Item: $12.95

I. Dimensions: 2 pounds

Appendix A2.5.2.7 - Glucose

A. Sensor - Maltodextrin [D-Glucose] 100% Food Grade Powder 6 Oz in a Space-Saver Bottle USA

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Maltodextrin-D-Glucose-100-Food-Grade-Powder-6-Oz-in-a-Space-Saver-Bottle-USA/322006494777?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $16.99

I. Dimensions:

Appendix A2.5.2.8 - Polyethylene Glycol (PEG)

A. Item: MiraLAX

B. Shield: N/A

C: Arduino code to download or write: N/A

D. Item documentation: https://www.miralax.com

E. Wiring: N/A

F. Testing with Arduino/computer: N/A

G. Web Site for Purchase: Amazon.com

H. Cost Per Item: $16.99

I. Dimensions: 1 pound

Appendix A2.5.2.9 - Protein Adhesion

A. Item: Bovine serum albumin

B. Shield: N/A

C: Arduino code to download or write: N/A

D. Item documentation:

E. Wiring: N/A

F. Testing with Arduino/computer: N/A

G. Web Site for Purchase: https://www.ebay.com/itm/10-grams-bovine-serum-albumin-BSA/273316108409?hash=item3fa2e8d879:g:mggAAOxy2LJRCFQ8

H. Cost Per Item: $45.00

I. Dimensions: 10 g

Appendix A2.5.3 - Feed Tanks

Appendix A2.5.3.1 - Diaphragm Pumps

Appendix A2.5.3.1.1 - Better Diaphragm Pump

A. Diaphragm Pump

Figure A2.5.3.1.1.1: Diaphragm pump (Diaphragm Pump for Arduino Projects 2019)

B. Arduino shield: N/A

C. Arduino code to download or write: N/A

D. Item Specifications:

Figure A2.5.3.1.1.2: Diaphragm pump schematic (mm scale) (Diaphragm Pump for Arduino Projects 2019)

Figure A2.5.3.1.1.3: Diaphragm pump schematic for inlet and outlet (mm scale) (Diaphragm Pump for Arduino Projects 2019)

Table A2.5.3.1.1.1: Item specifications for diaphragm pump

| Diaphragm Pump Specifications |  |
| --- | --- |
| Type | submersible pump |
| Diameter air inlet/ outlet | 10 mm |
| Power | 12V (1A) |
| Weight | 0.1 kg |

E. Wiring

Figure A2.5.3.1.1.4: Wiring diagram for diaphragm pump. Alligator clips connect a 12 V power supply to the terminals on the diaphragm pump.

F. Web site for purchase:

https://www.ebay.com/itm/Peristaltic-Pump-Diaphram-Pump-Submersible-for-Arduino-Raspberry-Pi-Projects/183034848690?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

G. Cost per item: $4.16 + $2.30 S/H

3.6 mm Water Flow Sensor Switch Meter Flowmeter 0.15-1.5 L/min with 340 mm Cable

Appendix A2.5.3.1.2 - Alternate Diaphragm Pump

A. Item:
Peristaltic Pump / Diaphragm Pump Submersible for Arduino/ Raspberry Pi Projects

Figure A2.5.3.1.2.1 - Alternate diaphragm pump

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Peristaltic-Pump-Diaphram-Pump-Submersible-for-Arduino-Raspberry-Pi-Projects/183034848690?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $4.16 + $2.30 S/H

I. Dimensions:

Appendix A2.5.3.1.3 - Peristaltic pump
A. Item

Figure A2.5.3.1.3.1 - Peristaltic pump

https://i.ebayimg.com/images/g/uToAAOSwBY5a8DUB/s-l1600.jpg

B. Arduino shield (None)

C. Arduino code to download or write
https://www.atlas-scientific.com/_files/code/Arduino-Uno-PMP-sample-code.pdf

D. Equipment documentation
https://www.atlas-scientific.com/_files/_datasheets/_peristaltic/EZO_PMP_Datasheet.pdf

E. Wiring - 12VDC Molex connector to 6-pin header to breadboard

http://www.molex.com/webdocs/datasheets/pdf/en-us/0022012057_CRIMP_HOUSINGS.pdf

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/DC-12V-D3-Peristaltic-Dosing-Pump-3x5mm-DIY-Head-Tube-For-Lab-Analytical-Liquid/222969387787?hash=item33ea02db0b%3Am%3Ampn1MMbrhYm-vHHgJRU9Q-A&var=521774685491&_sop=15&_sacat=0&_nkw=peristaltic+pump&_from=R40&rt=nc

H. Cost per item: $5.99 + $0 S/H

I. Dimensions: for 4 mm tubing

J. Support Stand:  http://www.thingiverse.com/thing:2279394

Appendix A2.5.3.2 - Syringe Pumps

Appendix A2.5.3.2.1 - Eight-Channel Kloehn Syringe Pumps

Figure A2.5.3.2.1.1 - Kloehn Syringe Pump With Multi-Channel Input

B. Arduino shield

C. Arduino code to download or write

D. Item documentation:  http://cdn.norgren.com/pdf/IMI-PE_PDS-0009.pdf

E. Wiring

Figure A2.5.3.2.1.2 - Kloehn Syringe Pump With Multi-Channel Input Pinout

www.firsttenangstroms.com/pdfdocs/Versa3.pdf

F. Testing with Arduino/computer

G. Web site for purchase - same seller as

https://www.ebay.com/itm/KLOEHN-SYRINGE-PUMP-V8-Multi-Channel-Syringe-Pump-250ul-p-n-20480/273913687277?hash=item3fc6872ced:g:nD0AAOSw5UZY9po-

For cable connection to syringe pump

https://www.ebay.com/itm/Centronics-36-Pin-IDC-Type-Female-CN36-Crimp-Connector-for-Ribbon-Cable/392062879912?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $200 each used x 3

I. Dimensions: p. 4 of http://cdn.norgren.com/pdf/IMI-PE_PDS-0009.pdf

Appendix A2.5.3.2.2 - Newer Kloehn Syringe Pump With Multi-Channel Input

A. Item  KLOEHN SYRINGE PUMP WITH DRRIVE ASSEMBLY P/N # 23254

Figure A2.5.3.2.2.1 - Newer Kloehn Syringe Pump With Multi-Channel Input

B. Arduino shield

C. Arduino code to download or write

D. Item documentation:  http://cdn.norgren.com/pdf/IMI-PE_PDS-0009.pdf

E. Wiring

Figure A2.5.3.2.2.2 - Newer Kloehn Syringe Pump With Multi-Channel Input Pinout

www.firsttenangstroms.com/pdfdocs/Versa3.pdf

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/KLOEHN-SYRINGE-PUMP-WITH-DRRIVE-ASSEMBLY-P-N-23254/302512001140?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $99.50 + $14.50 S/H

I. Dimensions: p. 4 of

http://cdn.norgren.com/pdf/IMI-PE_PDS-0009.pdf

Appendix A2.5.3.3 - Feed Tanks

A. Item - Whatman Zapcap CR Filters, 0.2 m, case of 12

Figure A2.5.3.3.1 - Feed Tanks = Whatman Zapcap CR 0.2 m Filters with Shutoff Valves

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Whatman-Zapcap-CR-Filters-0-2um-case-of-12/172003226074?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $98 per dozen x 2

I. Dimensions:

Appendix A2.5.3.4 - 1/16" PEEK Fittings

A. Item
PEEK 1/16 inch Tubing Finger Tight PEEK Fittings, Pack of (20)- FAST SHIPPING!

Figure A2.5.3.4.1 - PEEK Fittings

B. Arduino shield

C. Arduino code to download or write
D. Sensor documentation
E. Wiring
F. Testing with Arduino/computer
G. Web site for purchase

https://www.ebay.com/itm/PEEK-1-16-inch-Tubing-Finger-Tight-Peek-Fittings-Pack-of-20-FAST-SHIPPING/282700657400?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $35.99 + $6.49 S/H x 2

I. Dimensions:

Appendix A2.5.3.5 - 1/16" PEEK Tubing
A. Item

NEW IDEX 1533 PEEK Tubing Green 1/16 x 0.030 in x 5 20 100 feet FreeShip HPLC

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/NEW-IDEX-1533-PEEK-Tubing-Green-1-16-x-0-030-in-x-5-20-100-feet-FreeShip-HPLC/264094306052?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $19.00

I. Dimensions:

Appendix A2.5.3.6 - Manually Operated Valves for Shutoff Around Feed Tanks

A. Item
10X Air Oxygen Valve Pipe Hose Aquarium Fish Tank Pool Fitting Tube Connectors

Figure A2.5.3.6.1 - Manually Operated Valves For Shutoff Around Feed Tanks

B. Arduino shield
C. Arduino code to download or write
D. Sensor documentation
E. Wiring
F. Testing with Arduino/computer
G. Web site for purchase

https://www.ebay.com/itm/1-10X-Air-Oxygen-Valve-Pipe-Hose-Aquarium-Fish-Tank-Pool-Fitting-Tube-Connectors/132912992298?ssPageName=STRK%3AMEBIDX%3AIT&var=432212355948&_trksid=p2057872.m2749.l2649

H. Cost per item: 10 for $1.18 x 10

I. Dimensions:

Appendix A2.5.4 - Aquarium Level

Appendix A2.5.4.1 - Colorimetric Imaging of Unwanted Biofilm Growth

Appendix A2.5.4.2 - UV LED's and UV LED Detectors

Appendix A2.5.4.2.1 - 470-475 nm LED Strip

A) Sensor/Item:  5M DC 12V 24V Waterproof Blue 470-475 nm 5050 SMD 150LED / 300LED Flexible Strip

Actual Item: 0.5 m (1.6 ft.); 30 LED's, 12 V, IP67 waterproofing; 470-475 nm

Figure A2.5.4.2.1.1 - 470-475 nm LED Strip

B) Shield: None

C) Code (if any): None

D) Documentation:

LED Type: 5050 SMD xugeled Flexible Strip

LED Color: Blue, Superbright

LED Quantity: 150/300LEDs/ 5 Meter (30/60LEDs/M)

FPC Background: White FPCB

Protection Rate: Waterproof (IP65):LED strip by a silicone, but not in a plastic tube.

Self-adhesive back with 3M double side adhesive tape.

Size: L5000 mm x W10 mm x T2.4 mm

Luminous Flux: 1200-1320 Lumens/Meter.

Wavelength: 470-475 nm

Power Consumption: 60W / 5 Meter

Working Input Voltage: DC 12V / 24V

Working Current: 1A / Meter

Cut every 3-LED: Eligible

Beam Angle:120°

Viewing Angle: adjustable by mounting position

Working Temperature: -20° to 50°

Drive Mode: Constant Voltage

Ra: >80

Package Content: Brang New in static bag, 5 Meters/Reel

eBay  xugeled®  Global  Online  Shop

Features:

Low power consumption, Super-bright but running with low temperature.

Flexible Ribbon For Curving Around Bends.

Completely Smooth And Even Light Spread, Solving The Uneven Luminous Problem.

Every 3 LEDs cuttable without damaging the rest of the strips, according to your requirement.

Self-adhesive back with adhesive tape for secure and easy application.

LED Type: High Quality SMD xugeled, high intensity and reliability

Long lifespan >50,000 hours.

E) Wiring Diagram: None

F) Testing: None

G) Web Site for Purchase:

https://www.ebay.com/itm/5M-DC-12V-24V-Waterproof-Blue-470-475nm-5050-SMD-150LED-300LED-Flexible-Strip/151408048211?ssPageName=STRK%3AMEBIDX%3AIT&var=453080982109&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H): $3.99 ($ 0 S/H)

I) Dimensions: See A)

Appendix A2.5.4.2.2 - 265 nm LED

A) Sensor:

UVC LED Diode 265 nm SMD3535 Chip Ultra Violet LED beads for UVC LED Lamp

Figure A2.5.4.2.2.1 - 265 nm LED

Figure A2.5.4.2.2.2 - 265 nm LED, 0.2-0.3 mW @ 40 mA

B) Shield: None

C) Code (if any): None

D) Documentation:

Features:

1. High performance: Kill 99.99% of harmful bacteria with UV light in seconds.

2. DIY Water Sanitizer solution: Clean water by installing a UVC LED module.

3. Environmental friendly: Chemical-free ultroviolet lighting source.

4. Long serving life:  Nearly 10000 hours of working life.

5. Third party tested: CE, RoHS certificates.

6. Instant start.

Parameters:

Brand:	Dr. Finsen	Model:	UL-UVC-002

LED Type:	SMD3535	Wattage:	0.2-0.3 mW

Voltage:	6V		Current:	40 mA

Wavelength:	265 nm	Lifespan:	10000 Hours

Warranty:	1 year

Storage:

The LEDs shall be hermetically sealed; the package shall be kept at 10℃ to 55℃ and 30% to 65% Relative Humidity (RH).

Operation:

The workbench shall be grounded. It is recommended to wear gloves or finger stalls when handling the LEDs. It is also recommended to seal the package promptly after opening it, or it may result in lead foot oxidation. After opening the package, use tweezers to hold the LED’s body and avoid touching its obverse side with bare hands.

Installation:

Equipment and machinery handling LED chips must be properly grounded.

Anti-static wrist strap must be worn during installation; check if the metal part makes close contact with skin.

It is strongly recommended to wear anti-static gloves or finger stalls during installation.

Put anti-static tape on the workbench; the tape shall be interconnected and grounded.

The LEDs shall be soldered within 24 hours after opening the package, or it may lead to electrode oxidation and rustiness.

Reflow soldering is recommended, and the temperature curve is shown below:

Preheat : 1.0 to 3.0℃/s

Maximum time: 90 seconds

Soak : 110 to 130℃

Last time : 90 or 100 s

Reflow :

Maximum temperature : 170 - 180℃

Cooling maximum time : 50 - 80 s above 138℃

Cooling rate < 4℃/s

The LEDs’ lifetime will decrease as current and temperature increase, which accelerates deterioration. It is recommended to include a grounded circuit when designing the PCB. To avoid circuit breakdown, the LEDs shall be operated at -30℃ to 55℃ and 30% to 65% RH, or it may result in device failure.

Cautions:

DUV LEDs emits ultraviolet light, which is potentially harmful to skin and eyes. Avoid direct or reflected exposure to UV light when LED is in operation.

Special precautions must be taken to avoid direct exposure to UV light.

Special precautions must be taken to avoid looking directly at UV light without the use of UV light protective glasses.

Always wear eye protection, gloves and protective garment during DUV LED operation.

Applications:

Disinfection closet

Food and beverage processing or packaging equipment

Food products factory, etc.

E) Wiring Diagram: None

F) Testing: None

G) Web Site for Purchase:

https://www.ebay.com/itm/UVC-LED-Diode-265nm-SMD3535-Chip-Ultra-Violet-LED-beads-for-UVC-LED-Lamp/143002051043?ssPageName=STRK%3AMEBIDX%3AIT&var=441967544817&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H): $49.99 ($ 0 S/H) for a pack of 5

I) Dimensions: on figure are in mm

Figure A2.5.4.2.2.3 - 265 nm LED dimensions

Appendix A2.5.4.2.3 - Aluminum Printed Circuit Board for UVC LED's

A) Sensor/Item: 6 pc 90 mm x 20 mm Aluminum PCB Circuit Board for 18 pcs SMD3535 UVC LEDs @40 mA 6V

Figure A2.5.4.2.3.1 - Aluminum Printed Circuit Board for UVC LED's

B) Shield: None yet

C) Code (if any): None yet

D) Documentation:

Aluminum PCB Circuit Board Series circuit for UVC LEDs with the following parameters:

Voltage: 6V

Current:@40 mA

E) Wiring Diagram: None yet

F) Testing: None yet

G) Web Site for Purchase:

https://www.ebay.com/itm/6pc-90mmx20mm-Aluminium-PCB-Circuit-Board-for-18pcs-SMD3535-UVC-LEDs-40mA-6V/143069316434?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H): $17.99 + $0 S/H

I) Dimensions: None yet

Appendix A2.5.4.2.4 - 365-370 and 400-410 nm LED's

A) Sensor/Item: 9W UV Cyan Red Purple Blue Cold Warm White LED Lamp Light + Driver + PCB 260 x 33 mm

365-370 nm, 90 degree lens, + Waterproof DC driver (purchased 2)

400-410 nm, 90 degree lens, + Waterproof DC driver (purchased 2)

Figure A2.5.4.2.4.1 - 365-370 and 400-410 nm LED's

Figure A2.5.4.2.4.2 - 365-370 and 400-410 nm LED's

B) Shield: PCB included in A)

C) Code (if any): None

D) Documentation: Item Description

9W Full Spectrum UV LED Lamp Light + Waterproof Lens + PCB 138 mm + Driver

LED Features:

Light Color:

UV (365-370 nm)

UV (400~410 nm)

Purple (415~420 nm)

Royal Blue (450~455 nm)

Ice Blue (470~475 nm)

Cyan (495~500 nm)

Green (520~525 nm)

Yellow (585~590 nm)

Red (625~630 nm)

Deep Red (660 nm)

IR (850 nm)

Warm White (3000~3500 K)

Neutral White (4000~4500 K)

Pure White (6000~6500 K)

Cold White (10000~15000 K)

Full Spectrum (400~840 nm)

Luminous flux: 18 Lm - 1300 Lm

AC Driver + Shell Features:

AC Input Voltage: 85V~265V

Size: (L) 59 x (W) 32 x (H) 23mm

Waterproof AC Driver Features:

AC Input Voltage: 85V~265V

Size: (L) 67 x (W) 25 x (H) 20 mm

Waterproof AC/DC Driver Features:

AC/DC Input Voltage: 12-24 VDC

Size: (L) 48 x (W) 23 x (H) 15 mm

Aluminium PCB Features:

Size: (L) 260mm x (W) 33 mm x (T) 1.5 mm

E) Wiring Diagram: None yet

F) Testing:

G) Web Site for Purchase:

https://www.ebay.com/itm/9W-UV-Cyan-Red-Purple-Blue-Cold-Warm-White-LED-Lamp-Light-Driver-PCB-260x33mm/392147452737?ssPageName=STRK%3AMEBIDX%3AIT&var=661088865808&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H): $21.98 (+$ 0 S/H) for 365-370 nm; $13.98 for 400-410 nm

I) Dimensions: 260 x 33 nm

Appendix A2.5.4.2.5 - 270-285 nm UVC LED

A) Sensor/Item: UVC LED Lamp DC12V Water Sterilizer DUV LED Light with Cable XH2.54 2P Terminal

Figure A2.5.4.2.5.1 - 270-285 nm UVC LED

Figure A2.5.4.2.5.2 - 270-285 nm UVC LED in Bioreactor

Figure A2.5.4.2.5.3 - 270-285 nm UVC LED

B) Shield: None

C) Code (if any): None

D) Documentation:

Figure A2.5.4.2.5.4 - 270-285 nm UVC LED irradiation of bacteria

Figure A2.5.4.2.5.5 - 270-285 nm UVC LED assembly diagram

Parameters:

Voltage: 12VDC

Current: 40 ± 2 mA

Environment temperature: ≤ 55oC

UVC LED Luminous Power; 2.0-3.0 mW

Insulation and voltage resistance: AC 1500V, 50 Hz / 1 min

Lifespan: >10,000 hr

Dominant Wavelength: 270 nm - 285 nm

Table A2.5.4.2.5.1 - Optical and Electrical Characteristics @TJ = 25 oC

Item			Symbol	Condition	Min	Average	Max	Unit

Angle			2θ1/2		12 VDC	---	120		---	deg

Power Output		P		12 VDC	---	≥2		---	mW

Half-wave width 	Δλ		12 VDC	8	11.5		12	nm

Peak Wavelength 	λP		12 VDC	265	275		285	nm

Features

1. High performance: Kill 99.99% of harmful bacteria with UV light in seconds.

2. DIY Water Sanitizer solution: Clean water by installing a UVC LED module.

3. Environmentally friendly: Chemical-free ultroviolet lighting source.

4. Long serving life:  Nearly 10000 hours of working life.

5. Third party tested: CE, RoHS certificates.

6. Instant start.

Test Method

1. Turn on the UVC LED device.

2. Make a bacteria smear of 0.1 ml colon bacillus liquid with the density of 3000 cfu/ml.

3. Expose the smear under the UVC LED device for 5 minutes with a distance of 1 cm.

4. Get the test result and calculate the disinfection rate.

Applications: Water, food, & toy sanitizer

E) Wiring Diagram: None yet

F) Testing: None yet

G) Web Site for Purchase:

https://www.ebay.com/itm/UVC-LED-Lamp-DC12V-Water-sterilizer-DUV-LED-Light-with-Cable-XH2-54-2P-Terminal/143048721816?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H):

I) Dimensions: None yet

Appendix A2.5.4.2.6 - 370 nm UV LED

A) Sensor: 5 mm Round Head LED Violet Light Beads UV LED 370 nm for Money Detector Lamp

Figure A2.5.4.2.6.1 - 370 nm UV LED

B) Shield: None

C) Code (if any): None

D) Documentation:

Description:

30/50/100/200Pcs 5mm Round Head Purple UV Lamp LED Emitting Diodes Sterilization

Material: LED

Color: Purple

Size: 5 mm

Description of electrical parameters:

Voltage: 3.6 to 4.0 V

Current: 20 mAh

Brightness: 2-3 mW

Wavelength: 365-375 nm

Reverse leakage: less than 5UA(reverse 9V impact)

Service life: 80-100,000 hours. Colloid color: transparent

E) Wiring Diagram: None

F) Testing: None

G) Web Site for Purchase: :

https://www.ebay.com/itm/5mm-Round-Head-LED-Violet-Light-Beads-UV-LED-370nm-for-Money-Detector-Lamp/332892356411?ssPageName=STRK%3AMEBIDX%3AIT&var=542005617336&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H): Pack of 30 for $4.66 (+ $0 S/H)

I) Dimensions: 5 mm tall

Appendix A2.5.4.2.7 - 475 nm LED

A) Sensor/Item: 10 pc 3W High Power Blue 475 nm LED Emitter 45 mil Chip For DIY

Figure A2.5.4.2.7.1 - 475 nm LED's

B) Shield: None yet

C) Code (if any): None yet

D) Documentation:Specifications:

LED Emitter: 3W Blue LED

Output Lumens: 120 Lumens

DC Forward Voltage (VF) : 3.0-3.4 VDC

DC Forward Current (IF) : 700 mA

Wavelength: 475 nm

Beam Angle: 120 degrees

LifeSpan Time : > 50,000 hours

Dimension: Diameter 20 mm

E) Wiring Diagram: None yet

F) Testing: None yet

G) Web Site for Purchase:

https://www.ebay.com/itm/10pc-3W-High-Power-Blue-475nm-LED-Emitter-45mil-Chip-For-DIY/221609354321?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H): $1.83 for 10 pieces

I) Dimensions: See part D)

Appendix A2.5.4.2.8 - 253.7 nm UV Lamp

A) Sensor/Item: UV-9W Germicidal Lamp LED Home Lighting Bulbs UV-C 253.7 nm 254 nm

Figure A2.5.4.2.8.1 - 253.7 nm UV Lamp

B) Shield: None yet

C) Code (if any): None yet

D) Documentation:

Item Description:

Length: 145 mm (without pins), 160 mm (overall length)

Lamp Base: G23, 2 pins

Working Voltage: 60V

Life: 8000 hours

UVC Output: 23 W/cm

Features:

- UVC radiation.

- Ozone-free.

- Made with quartz glass.

- Quartz sleeves and ballast available upon request.

- Ozone-generating type available upon request.

- Lamp contains mercury, manage in accord with disposal laws.

- Ultraviolet radiation, follow professional instructions, avoid expose eye and skin to the uv.

- This lamp is not intend to general lighting service.

Package Weight: 42.0 g / 1.48 Ounces

Package Dimensions (L*W*H): 100*20*20 MM / 3.94*0.79*0.79 Inches

E) Wiring Diagram:v None yet

F) Testing: None yet

G) Web Site for Purchase:

https://www.ebay.com/itm/UV-9W-Germicidal-Lamp-LED-Home-Lighting-Bulbs-UV-C-253-7nm-254nm/263047598191?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H): $4.89 + $1.90 S/H

I) Dimensions: See part D) + Figure 2.5.4.2.8.2 below

Figure A2.5.4.2.8.2 - 253.7 nm UV lamp dimensions

Appendix A2.5.4.2.9 - 365-370 nm UV LED + PCB Board

A) Sensor/Item: 3W High Power LED Chip Light Beads White Warm red green UV IR emitting diode (purchased 365-370 nm + PCB board)

Figure A2.5.4.2.9.1 - 365-370 nm UV LED + PCB Board

B) Shield: PCB included

C) Code (if any): None yet

D) Documentation:

E) Wiring Diagram: None yet

F) Testing: None yet

G) Web Site for Purchase:

https://www.ebay.com/itm/3W-High-Power-LED-Chip-Light-Beads-White-Warm-red-green-UV-IR-emitting-diode/142416640027?ssPageName=STRK%3AMEBIDX%3AIT&var=441423484865&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H): $5.20 for 2 pieces

I) Dimensions:

Figure A2.5.4.2.9.2 - 365-370 nm UV LED + PCB Board

Appendix A2.5.4.2.10 - 253.7 nm AlGaN UV Detector

A) Sensor/Item: KPDU27HQ2 PHOTO DIODE AlGaN UV Detector KYOSEMI

Figure A2.5.4.2.10.1 - 253.7 nm AlGaN UV Detector

B) Shield: None yet

C) Code (if any): None yet

D) Documentation:

AlGaN UV Detector

• Sensitivity covering UV-C radiations

• Large detectable area size 0.86 mm sq.

• Hermetic seal type with quartz flat window for high reliability uses

Applications

• Suitable for monitoring optical intensity of sterilizing ray of Hg lamp (253.7 nm)

• UV-C lasers and UV-C LEDs

• UV-C source monitors

• UV-C sensors

• UV-C exposure meters

• Medical use

E) Wiring Diagram: None yet

F) Testing: None yet

G) Web Site for Purchase:

https://www.ebay.com/itm/KPDU27HQ2-PHOTO-DIODE-AlGaN-UV-Detector-KYOSEMI/263556429210?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H): $29.99 + $5.00 S/H

I) Dimensions: None yet

Appendix A2.5.4.2.11 - 254 nm UV Lamp

A) Sensor/Item:  UV-5W Germicidal Lamp LED Home Water / Air Lighting Bulbs 254 nm UVC H-Shape Tube

B) Shield: None yet

C) Code (if any): None yet

D) Documentation:

Item Description:

Length: 135mm

Lamp Base: G23 / 2G7

Working Voltage: 34V

Life: 13000 hours

UVC Output: 12 W / 1.1c㎡

Feature:

- UVC radiation.

- Ozone-fre.

- Made with quartz glass.

- Quartz sleeves and ballast available upon request.

Note:

- Ozone-generating type available upon request.

- Lamp contains mercury, manage in accord with disposal laws.

- Ultraviolet radiation, follow professional instructions, avoid expose eye and skin to the uv.

- This lamp is not intend to general lighting service.

Package Weight: 30.0 g / 1.06 ounces

Package Dimensions (L*W*H): 140*38*25 mm / 5.51*1.50*0.98 inches

Retail Packaging: Yes, Colored Box

Package Including:

1 x UV-5W Germicidal Lamp

E) Wiring Diagram: None yet

F) Testing: None yet

G) Web Site for Purchase:

https://www.ebay.com/itm/UV-5W-Germicidal-Lamp-LED-Home-Water-Air-Lighting-Bulbs-254nm-UVC-H-Shape-Tube/392127615326?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H): $4.39 + $0 S/H

I) Dimensions: See D) above

Appendix A2.5.4.2.12 - 440 nm LED Strip

A) Sensor/Item: Salt Water Aquarium Reef 440 nm Actinic BLUE Light LED Strip 250 Lumens/Ft Moon

Figure A2.5.4.2.12.1 - 440 nm LED strip

B) Shield: None yet

C) Code (if any): None yet

D) Documentation:

This listing includes the Actinic Blue (250 lumens per foot) moon strip length that you selected form the drop box above as well as the appropriate POWER SUPPLY which plugs directly into a wall outlet. Power Supplies are different for different lengths. The longer the strip, the larger the power supply. Once you get longer than 40 ft there will be a large power supply that will require you to go both directions from the power to get the full length ( ---------LEDs----------power-----------LEDs----------- ).  This means you can go both ways from the power and meet in the middle on the other side of the pool or anything else you are lighting.

Overview

Create your own custom lighting with LED tape strips. Pre-applied 3M adhesive attaches to almost any surface and the sealed design is impact resistant. This single color Blue LED strip can be cut to custom lengths. Simply cut and go.

Once applying, you just plug in the stripand you are ready to go.

** Strips can be connected to each other if you need longer.

Highlights

Waterproof... See waterproofing details below.

Longer life (50,000 hours) and much lower power consumption compared to rope lights or any other lighting

Cut lines on circuit board for creating miniature segments

Impact resistant to use anywhere with 3M adhesive backing

FOR WATERPROOF STRIPS

Waterproof Strips are silicone filled to protect the LEDs from water all along the top and sides of the strip. To fully protect the strip long term you will need to run a bead of silicone along both sides of the strip and all around any connection, plug and both ends of the strip. If you do not do this there is risk of the LEDs shorting out. Being a 12V light, there is no safety issue. This suggestion is simply to protect your LEDs long term.

Why buy MY LEDs??

1. The board color.

Instead of "yellow board" my LEDs are on a much nicer and more durable white board with an oil brushed surface.

The wire running through this is 100% copper instead cheap short lasting iron as most LED strips are.

2. The board material quality.

"A" is my LED strip with two layer board completely resistant to heat and protected from short circuiting. (Very Common for cheap LED strips)

3. Silicone Filled NOT Epoxy (for Waterproof Only)

Waterproof Strips are filled with real silicone instead of epoxy glue that becomes yellow and faded over time.  The real silicone is also the only way ensure a consistent color temperature over time.

4. Chip Connection

My LEDs have gold thread running through to connect each LED and chip together rather than copper thread. This will ensure a long lasting connection for LED to chip.

E) Wiring Diagram: None yet

F) Testing: None yet

G) Web Site for Purchase:

https://www.ebay.com/itm/Salt-Water-Aquarium-Reef-440-nm-Actinic-BLUE-Light-LED-Strip-250-Lumens-Ft-Moon/280905564366?ssPageName=STRK%3AMEBIDX%3AIT&var=580196655806&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H): $12.95/foot + $ 0 S/H

I) Dimensions: 1 foot long

Appendix A2.5.4.2.13 - 440-450 nm LED

A) Sensor/Item: 1w 3w 5w 10w 20w 30w 50w 100w Royal Blue 440-450nm High Power LED SMD Chip Lamp

Figure A2.5.4.2.13.1 - 440-450 nm LED

B) Shield: included in A)

C) Code (if any): None yet

D) Documentation:

Condition:	New	Color Temperature:	>7000k

Royal Blue wavelength:	440-450 nm

About US: Professional LED factory	Forward Voltage Current 1W:  3.2-3.4V 350 mA

Fast Fulfillment:	YES		Forward Voltage Current 3W:  3.2-3.4V 700 mA

Type:	LED Chip Flood Light	Forward Voltage Current 5W:  6.0-7.0V 700 mA

MPN:	Does Not Apply		Forward Voltage Current 10W: 9.0-11.0V 1000 mA

Brand:	Ch				Forward Voltage Current 20-100W: 32-34V 700 mA

Model:	 1W/3W/5W/10W/20W/30W/50W/100W Chips LED

Extra Support:	Need wholesale, send E-mail, negotiate the price

Chips Brand:	Epistar 33*33mil

E) Wiring Diagram: None yet

F) Testing: None yet

G) Web Site for Purchase:

https://www.ebay.com/itm/1w-3w-5w-10w-20w-30w-50w-100w-Royal-Blue-440-450nm-High-Power-LED-SMD-Chip-Lamp/122022099000?ssPageName=STRK%3AMEBIDX%3AIT&var=421032119650&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H): $2.09

I) Dimensions: None yet

Appendix A2.5.4.2.14 - Power Supplies for UV LED's

A) Sensor/Item: High Power Supply 3W 10W 20W 30W 50W 100W Constant Current COB Chip LED Driver:	Input AC 85-277V Waterproof PCB Ceiling Lamp For Light

Figure A2.5.4.2.14.1 - Power Supplies for UV LED's

B) Shield:

C) Code (if any): None yet

D) Documentation:

E) Wiring Diagram: None yet

F) Testing: None yet

G) Web Site for Purchase:

https://www.ebay.com/itm/AC-85-277V-High-Power-Supply-LED-Driver-1-100W-Waterproof-Constant-Current-USA-/122480543106?

H) Cost Per Item (Including S/H): $6.59 for 5W to drive 6 1 W's;

I) Dimensions: None yet

Appendix A2.5.4.2.15 - 200-370 nm UV Detector

A. Item:

KEYES UV UVM-30A Ultraviolet Ray Detection Sensor Module 3-5V for Arduino HK

Figure A2.5.4.2.15.1 - 200-370 nm UV Detector

B. Arduino shield
C. Arduino code to download or write
D. Sensor documentation
E. Wiring
F. Testing with Arduino/computer
G. Web site for purchase

https://www.ebay.com/itm/KEYES-UV-UVM-30A-Ultraviolet-Ray-Detection-Sensor-Module-3-5V-for-Arduino-HK/133047144874?_trkparms=aid%3D333200%26algo%3DCOMP.MBE%26ao%3D1%26asc%3D59178%26meid%3D4ac0e8ed15c746d382036a2d0c0a6c2d%26pid%3D100008%26rk%3D4%26rkt%3D12%26sd%3D400594940658%26itm%3D133047144874%26pg%3D2047675&_trksid=p2047675.c100008.m2219

H. Cost per item: 3 for $23.46 + 2 for $20

I. Dimensions:

Appendix A2.5.4.3 - Temperature and Heat-Related Aquarium-Level Sensors/Controls

Appendix A2.5.4.3.1 - Aquarium Heaters

A. Item - 2X 25W/50W/100W/200W/300W/500W Aquarium Heater Adjustable Submersible Fish Tank

Figure A2.5.4.3.1.1 - Aquarium Heaters

https://i.ebayimg.com/images/g/kO0AAOSwAYtWMudz/s-l1600.jpg

B. Arduino shield - None

C. Arduino code to download or write
	heatercontrol.txt from https://forum.arduino.cc/index.php?topic=94301.0

D. Item documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/2X-25W-50W-100W-200W-300W-500W-Aquarium-Heater-Adjustable-Submersible-Fish-Tank/262117055497?hash=item3d0764dc09%3Am%3Am7Fg5N_UGcgmYY_ADgp30jw&var=560885236567&_sacat=0&_nkw=2X+25W%2F50W%2F100W%2F200W%2F300W%2F500W+Aquarium+Heater+Adjustable+Submersible+Fish+Tank&_from=R40&rt=nc&_trksid=m570.l1313

H. Cost per item: 2 for $10.99 + $0 S/H

I. Dimensions: 25 W

Appendix A2.5.4.3.2 - Temperature Sensor for Entire Aquarium

A. Sensor - DS18B20

Figure A2.5.4.3.2.1 - Temperature sensor

B. Arduino shield (seen in above figure)

C. Arduino code to download or write
	http://static.cactus.io/downloads/code/ds18b20/DS18B20_Sketch.ino		http://static.cactus.io/downloads/library/ds18b20/cactus_io_DS18B20.zip

D. Sensor documentation
	https://datasheets.maximintegrated.com/en/ds/DS18B20.pdf

E. Wiring

Figure A2.5.4.3.2.2 - Wiring Diagram for Temperature Sensor for Entire Aquarium

F. Testing with Arduino/computer
G. Web site for purchase

https://www.ebay.com/itm/DS18B20-Waterproof-Digital-Temperature-Sensor-With-Adapter-Module-for-Arduino-TW/131899447876

H. Cost per item: $4.29 + $0 S/H including shield

I. Dimensions:

Appendix A2.5.4.3.3 - Temperature and Humidity of Vapor Space above Aquarium

A. Sensor - DHT11 Temperature and Relative Humidity Sensor Module

Figure A2.5.4.3.3.1 - Temperature and Humidity of Vapor Space above Aquarium

B. Arduino shield
 - Sensor and shield integrated into one piece (see above figure)

C. Arduino code - http://www.inhaos.com/downcount.php?download_id=212

D. Sensor documentation

http://www.circuitbasics.com/wp-content/uploads/2015/11/DHT11-Datasheet.pdf

E. Wiring
 	http://www.circuitbasics.com/how-to-set-up-the-dht11-humidity-sensor-on-an-arduino/

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/New-DHT11-Temperature-and-Relative-Humidity-Sensor-Module-for-arduino/310670497097?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $1.09 + $1.85 S/H

I. Dimensions:

Appendix A2.5.4.4 - Load cell for entire aquarium

A. Sensor

Figure A2.5.4.4.1 - Load cell sensor

B. Arduino shield
C. Arduino code to download or write
D. Sensor documentation
E. Wiring

Figure A2.5.4.4.2 - Wiring diagram for load cell sensor used to measure cell/scaffold mass

Figure A2.5.4.4.3 - Load cell wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Scale-Load-Cell-Weight-Weighing-Sensor-HX711-AD-Module-Metal-Shied-2-5-10-20KG/272817516318?ssPageName=STRK%3AMEBIDX%3AIT&var=571961640473&_trksid=p2057872.m2749.l2649

H. Cost per item: $4.99 + $1.88 S/H

I. Dimensions:

Figure A2.5.4.4.4 - Dimensions of load cell sensor

https://i.ebayimg.com/images/g/jpEAAOSw1blZnYBW/s-l1600.jpg

Appendix A2.5.4.5 - Non-Temperature Environmental Monitoring for Aquarium

Appendix A2.5.4.5.1 - CO2 concentration for entire headspace

A. Sensor - Generic MG-811

MG811 Carbon Dioxide CO2 Gas Sensor Detector Analog Signal CCS811 CJMCU Module

Mq-7 Carbon Dioxide Sensor Module Gas Sensor Detection Alarm Module Components

Figure A2.5.4.5.1.1 - Generic MG811 Carbon Dioxide Gas Sensor and Shield

https://i.ebayimg.com/images/g/F0IAAOSwDk5UG96J/s-l400.jpg

Figure A2.5.4.5.1.2 - MQ-7 Carbon Dioxide Gas Sensor and Shield

https://www.ebay.com/itm/Mq-7-Carbon-Dioxide-Sensor-Module-Gas-Sensor-Detection-Alarm-Module-Components/183249727485?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

B. Arduino shield included with sensor

C. Arduino code to download or write
D. Sensor documentation
The MG-811 is highly sensitive to CO2 and less sensitive to alcohol and CO.

IC: LM393、MG811 Sensor

Work voltage：DC6V

Features：Instructions with the signal output; Analog + TTL output; TTL output signal is low effective; Analog output 30~50mV, (higher consistency, higher voltage)

http://vi.vipr.ebaydesc.com/ws/eBayISAPI.dll?ViewItemDescV4&item=301191399526&t=1509776346000&tid=10&category=175745&seller=trade_spotting&excSoj=1&excTrk=1&lsite=0&ittenable=false&domain=ebay.com&descgauge=1&cspheader=1&oneClk=1&secureDesc=0

E. Wiring
F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/MG811-Carbon-Dioxide-CO2-Gas-Sensor-Detector-Analog-Signal-CCS811-CJMCU-Module/173557245152?ssPageName=STRK%3AMEBIDX%3AIT&var=472239800532&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/Mq-7-Carbon-Dioxide-Sensor-Module-Gas-Sensor-Detection-Alarm-Module-Components/183249727485?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item:  $ 29.99 + 5.44; $40.99 + $4.99 S/H for Sain Smart

I. Dimensions:

Appendix A2.5.4.5.2 - OpenDrop for Concentration Monitoring Comparison

A. Sensor - Open Drop

Figure A2.5.4.5.2.1 - Open Drop for Concentration Monitoring (U. Gaudenz, GaudiLabs, http://www.gaudi.ch/OpenDrop/?p=17, 2019)

B. Arduino shield (included with sensor)
C. Arduino code to download or write
 	OpenDrop-master.zip from https://github.com/GaudiLabs/OpenDrop.git

http://microfluidics.utoronto.ca/microdrop

D. Sensor documentation
 	http://www.gaudi.ch/GaudiLabs/wp-content/uploads/OpenDrop_Instructions.pdf

https://github.com/sci-bots/microdrop/wiki/Install-MicroDrop

https://github.com/sci-bots/microdrop/wiki/Installing-Arduino-driver

https://github.com/sci-bots/microdrop/releases/download/v2.19/MicroDrop-2.19-Windows-x86.exe

E. Wiring (included with instructions)

F. Testing with Arduino/computer

G. Web site for purchase - https://buyzero.de/products/opendrop-v2

H. Cost per item: 546.22 euros + 35.71 euros

I. Dimensions:

Appendix A2.5.4.5.3 - Gas phase particle size analysis

A. Sensor - PMS5003

Figure A2.5.4.5.3.1 - Particle Size Analyzer for Gas Sampling Above Aquarium

https://i.ebayimg.com/images/g/zVcAAOSw63FaK32S/s-l1600.jpg

B. Arduino shield (included in above photo)

C. Arduino code - https://github.com/jbanaszczyk/pms5003.git

D. Sensor documentation

> Minimum distinguishable particle diameter : 0.3 micrometer

PMS5003 is a kind of digital and universal particle concentration sensor, which can be used to obtain the number of suspended particles in the air, i.e. the concentration of particles, and output them in the form of digital interface. This sensor can be inserted into variable instruments related to the concentration of suspended particles in the air or other environmental improvement equipments to provide correct concentration data in time.

Working principle

Laser scattering principle is used for such sensor, i.e. produce scattering by using laser to radiate suspending particles in the air, then collect scattering light in a certain degree, and finally obtain the curve of scattering light change with time. In the end, equivalent particle diameter and the number of particles with different diameter per unit volume can be calculated by microprocessor based on Mie theory. Please find the functional diagram of each part of the sensor from Figure A2.5.4.5.3.1.

Output result

Mainly output as the quality and number of each particles with different size per unit volume, the unit volume of particle number is 0.1L and the unit of mass concentration is μg/m³.

There are two options for digital output: passive and active. Default mode is active after power up. In this mode sensor would send serial data to the host automatically. The active mode is divided into two sub-modes: stable mode and fast mode. If the concentration change is small the sensor would run at stable mode with the real interval of 2.3 s. And if the change is big the sensor would be changed to fast mode automatically with the interval of 200~800 ms, the higher of the concentration, the shorter of the interval.

Circuit Attentions

1) DC 5V power supply is needed because the FAN should be driven by 5V. But the high level of data pin is 3.3V. Level conversion unit should be used if the power of host MCU is 5V.

2) The SET and RESET pins are pulled up inside so they should not be connected if without usage.

3) PIN7 and PIN8 should not be connected.

4) Stable data should be got obtained after at least 30 seconds after the sensor wakeup from the sleep mode because of the fan’s performance.

Installation Attentions

1) Metal shell is connected to the GND so be careful not to let it shorted with the other parts of circuit except GND.

2) The best way of install is making the plane of inset and outset closely to the plane of the host. Or some shield should be placed between inset and outset in order to prevent the air flow from inner loop.

3) The blowhole in the shell of the host should not be smaller than the inset.

4) The sensor should not be installed in the air flow way of the air cleaner or should be shielded by some structure.

5) The sensor should be installed at least 20 cm higher than the grand in order to prevent it from blocking by the floc dust.

6) Do not break up the sensor.

7) M2 self-tapping strew should be used to fix the sensor but it should not be deeper than 5 mm into the sensor.

Other Attentions

1) Only the consistency of all the PM sensors of PLANTOWER is promised and ensured. And the sensor should not be checked with any third party equipment.

2) The sensor is usually used in the common indoor environment. So some protection must be added if using in the conditions as followed:

a) The time of concentration ≥300μg/m³ is longer than 50% of the whole year or concentration≥500μg/m³ is longer than20% of the whole year.

b) Kitchen

c) Water mist condition such as bathroom or hot spring.

d) outdoor

E. Wiring (in prior section)

Figure A2.5.4.5.3.2 - Wiring Diagram for Particle Size Analyzer

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Digital-Particle-Concentration-Laser-Sensor-PMS5003-PM2-5-PM10-Cable-for-Arduino/302558986801

H. Cost per item: $28.99 + $0 S/H

I. Dimensions:

Appendix A2.5.4.5.4 - Barometric Pressure Sensors

A. Sensor - BMP180

Figure A2.5.4.5.4.1 - Barometric Pressure Sensors With Built-In Shield

B. Arduino shield - GY68 BMP180 Replace BMP085 Digital Barometric Pressure Sensor Board includes sensor

C. Arduino code to download or write

https://github.com/sparkfun/BMP180_Breakout_Arduino_Library/archive/master.zip

D. Sensor documentation
https://cdn-shop.adafruit.com/datasheets/BST-BMP180-DS000-09.pdf

https://randomnerdtutorials.com/guide-for-bmp180-barometric-sensor-with-arduino/

https://www.youtube.com/watch?v=i1LQaQmFutE

E. Wiring
 Vin	5V

GND	GND

SCL	A5

SDA	A4

Figure A2.5.4.5.4.2 - Barometric Pressure Sensors Wiring Diagram

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/GY68-BMP180-Replace-BMP085-Digital-Barometric-Pressure-Sensor-Board-Arduino/200915895472?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $1.09 + $1.85 S/H

I. Dimensions: 1mm x 1.1mm (0.039 in x 0.043 in).

Appendix A2.5.4.6 - Flow totalizers

Appendix A2.5.4.6.1 - Flow totalizer for entire aquarium
A. Sensor

Figure A2.5.4.6.1.1 - 1/2" NPT Hall Sensor-Based Flowmeter for Entire Aquarium

https://www.atlas-scientific.com/_files/_datasheets/_probe/1-4_flow_datasheet.pdf

B. Arduino shield

Figure A2.5.4.6.1.2 - Flowmeter Shield

https://www.atlas-scientific.com/_files/_datasheets/_circuit/flow_EZO_Datasheet.pdf

C. Arduino code to download or write
https://www.atlas-scientific.com/_files/code/arduino_flow_sample_code.pdf

D. Sensor documentation

https://www.atlas-scientific.com/_files/_datasheets/_probe/1-4_flow_datasheet.pdf

E. Wiring

https://www.atlas-scientific.com/_files/instructions/ezo-do-Wiringdiagram.pdf

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/1-2-Water-Flow-Sensor-Control-Effect-Flowmeter-Hall-1-30L-min-For-Arduino-US/312183053141?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.atlas-scientific.com/product_pages/kits/1-4_flow_meter.html

H. Cost per item: $175/kit + $0 S/H (includes $40 shield); $3.34 + $0 S/H excluding shield

I. Dimensions:

Appendix A2.5.4.6.2 - 0.15 - 1.50 L/min Flowmeter

A. Sensor
6 mm Water Flow Sensor Switch Meter Flowmeter 0.15-1.5 L/min with 340 mm Cable

Figure A2.5.4.6.2.1 - 0.15 - 1.50 L/min Flowmeter

B. Arduino shield

C. Arduino code to download or write
https://codebender.cc/sketch:304865#%5Barduinotutorial%5D%20Water%20Flow%20Sensor.ino

D. Sensor documentation
Table A2.5.4.6.2.1: Item specifications for flowmeter

| Diaphragm Pump Specifications |  |  |  |
| --- | --- | --- | --- |
| External threads | 6 mm | Working voltage range | DC 4.5~18V |
| Allowing pressure | ≤ 1.2 MPa | size | 86 x 42 x 20 mm |
| Rating voltage/current | DC 5V, ≤ 10 mA | Flow range | 0.3-4.0 L/min |
| Insulation resistance | > 100 MΩ | Cable length | 340 mm |
| Electric strength | AC 500V, 50Hz |

Figure A2.5.4.6.2.2: Wiring diagram for flowmeters (Flow Sensor Switch Meter 2019)

Measurements from the flowmeters are read on the Arduino code on the computer via USB connection with an Arduino Uno, shown in Figure A2.5.4.6.2.2.

G. Web site for purchase

https://www.ebay.com/itm/6mm-Water-Flow-Sensor-Switch-Meter-Flowmeter-0-15-1-5L-min-with-340mm-Cable/163236231684?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $4.19

I. Dimensions:

Appendix A2.5.4.6.3 - Stirrer for entire aquarium

A. Item -

"PTFE Overhead Mixer Stirrer, Length 250 mm, Diameter 7 mm, Paddle width 40 mm"

Figure A2.5.4.6.3.1 - Rotating shaft impeller

https://i.ebayimg.com/images/g/OfkAAMXQTghRKG9A/s-l400.jpg

G. Web site for purchase

https://www.ebay.com/itm/PTFE-Overhead-Mixer-Stirrer-Length-250mm-Diameter-7mm-Paddle-width-40mm/262812895314?hash=item3d30de8852%3Ag%3AOfkAAMXQTghRKG9A&_sop=15&_sacat=0&_nkw=7+mm+PTFE+stirrer&_from=R40&rt=nc

H. Cost per item: $7.99 + $0 S/H

I. Dimensions: 250 mm long, 7 mm diameter rod, 40 mm paddle width

Appendix A2.5.4.6.4 - Tachometer for stirrers

A. Sensor
https://www.ebay.com/itm/Precise-LCD-Display-Tachometer-RPM-Measure-Tool-Car-Motorcycle-Speed-Timer/173187481235?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

B. Arduino shield

https://www.ebay.com/itm/3pcs-IR-Optical-Limit-Switch-Endstop-Tachometer-Speed-Sensor-Module-for-Arduino/292168545216?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

Figure A2.5.4.6.4.1 - Tachometer for stirrers

C. Arduino code to download or write
https://github.com/CrossTheRoadElec/Phoenix-Documentation.git

D. Sensor documentation
Material: ABS

Color: Black

Resolution: 0.1 hrs. / 10 RPMs

Reading: 9999.9 hrs. / 99999 RPMs

Accuracy: 1 minute or 10 RPM, Quartz

Operating temperature: Standard -30oC-85oC

Power supply: 1 * CR2450 battery

Shield: This "sensor" triggers when something comes between the IR emitter and receptor. Generally used as non-contact limit switches/endstops/proximity sensors. Also can be used to build a speed sensor (tachometer).

E. Wiring

F. Testing with Arduino/computer
G. Web site for purchase

H. Cost per item: 	Sensor:  $5.36 + $3.09 S/H	Shield: 3 for $6.99 + $0 S/H

I. Dimensions:		Sensor Size:7 3.5*47*20 mm	Shield Size:

Appendix A2.5.4.7 - Arduino Data Acquisition Equipment

Appendix A2.5.4.7.1 - Arduino Uno

A. Sensor - Arduino Uno - ATmega328P CH340G UNO R3 Board + USB Cable

Figure A2.5.4.7.1.1 - Arduino UNO

D.  Sensor Specifications

Table A2.5.4.7.1.1: Item specifications for Arduino Uno

| Arduino Uno Specifications |  |
| --- | --- |
| Main chip | ATmega328P, CH340G |
| Support | both 3.3V & 5V Power Supply and can meet different I/O expansion requirements, including temperature humidity sensor, Xbee module, BT Bluetooth module, RF module, GPRS module, GPS module, LCD shield and so on |
| Length | 3.5 inches |
| Width | 2.5 inches |
| Weight | 1.76 ounces |

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/New-ATmega328P-CH340G-UNO-R3-Board-USB-Cable-Compatible-with-Arduino/381350929204?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $4.16 + $0 S/H

I. Dimensions:

Appendix A2.5.4.7.2 - Arduino ESP8266 WiFi Card

A. Sensor - ESP8266 standard

(OTA WeMos D1 CH340 WiFi Arduino UNO R3 Development Board ESP8266 	ESP-12E or ESP-12E ESP8266 UART WIFI Wireless Shield for Arduino UNO

R3); recognized as USB-to-Serial CH340 within Device Manager

B. Arduino shield

Figure A2.5.4.7.2.1 - Arduino ESP8266 WiFi Card

Figure A2.5.4.7.2.2 - Arduino ESP8266 WiFi Card

C. Arduino code to download or write
ESP8266-master.zip as https://github.com/esp8266/Arduino.git

https://www.exploreembedded.com/wiki/Arduino_Support_for_ESP8266_with_simple_test_code

D. Sensor documentation
https://arduino-esp8266.readthedocs.io/en/latest/installing.html#boards-manager

Start Arduino and open Preferences window.

Enter http://arduino.esp8266.com/stable/package_esp8266com_index.json into Additional Board Manager URLs field. You can add multiple URLs, separating them with commas.

Open Boards Manager from Tools > Board menu and find esp8266 platform.

E. Wiring

Figure A2.5.4.7.2.3 - Arduino ESP8266 WiFi Card Wiring Diagram

https://www.exploreembedded.com/wiki/images/5/59/619px-UART_with_ESP.png

https://www.exploreembedded.com/wiki/Arduino_Support_for_ESP8266_with_simple_test_code

F. Testing with Arduino/computer

G. Web site for purchase

WeMoS - https://www.ebay.com/itm/OTA-WeMos-D1-CH340-WiFi-Arduino-UNO-R3-Development-Board-ESP8266-ESP-12E/322001157723?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/ESP-12E-ESP8266-UART-WIFI-Wireless-Shield-for-Arduino-UNO-R3/171981792137?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $4.10 + $1.85 S/H for 30 WeMoS; $5.11 + $1.85 S/H for 6 ESP8266

I. Dimensions:

Appendix A2.5.4.7.3 - Multiplexers

A. Sensor - AS-6408

B. Arduino shield

Figure A.2.5.4.7.3.1 - AS-6408 multiplexer

C. Arduino code to download or write
http://www.inhaos.com/downcount.php?download_id=212

D. Sensor documentation
  Power supply

With UNO CORE:3 to 15V

Without UNO CORE:3 to 20V

ON Resistance:125ohm

Change in ON Resistance:50ohm

OFF channel leakage current:+ -0.01nA

Capacitance Input:30pF

INPUT Low:Max 1.5V

INPUT HIGH:Min 2.2V

Operation Temperature :0°C to 55°C

Operating Humidity :0 to 90% non-condensing

Storage Temperature :-20°C to 65°C

Propagation Delay Time:

Signal Input to Output:10nS

Address-to-Signal OUT:120nS

Cutoff (-3dB) Frequency :20MHz

THD(Total Harmonic Distortion):0.12%

Address-or-Inhibit-to-Signal Crosstalk:65 mV PEAK

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/AS-6408-64-to-8-or-1-Analog-Digital-Multiplexer-Switch-for-Arduino-STM32-MCU-DAQ/222827261696?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $8.99 + $0 S/H

I. Dimensions: 50 x 87 mm

Appendix A2.5.4.7.4 - Arduino Unos with Breadboards

B. Shield - "Arduino Prototyping Prototype Shield ProtoShield With Mini Breadboard"

Figure A2.5.4.7.4.1 - Arduino Uno with breadboard

D. Sensor documentation

The mini breadboard has 170 tie points.

The breadboard sticks on the prototype shield with its self-adhesive backing.

E. Wiring
F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Arduino-Prototyping-Prototype-Shield-ProtoShield-With-Mini-Breadboard/200985069840?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $1.64 + $0 S/H

I. Dimensions:  Prototype Size: 7 cm x 5.5 cm x 2 cm

Mini breadboard size: 4.4 * 3.4 * 1 cm

Appendix A2.5.4.7.5 - Electrical signal isolators for breadboards

A. Sensor

HW-399 4-channel Optocoupler Isolation Module for Arduino board blocks D5Q2

B. Arduino shield

Figure A.2.5.4.7.5.1 - Electrical signal isolator

https://www.ebay.com/itm/HW-399-4-channel-Optocoupler-Isolation-Module-for-Arduino-board-blocks-D5Q2/263586790587

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/HW-399-4-channel-Optocoupler-Isolation-Module-for-Arduino-board-blocks-D5Q2/263586790587 (for 100)

https://www.ebay.com/itm/HW-399-4-channel-Optocoupler-Isolation-Module-for-Arduino-high-and-low-leve-H3O6/273259128455 (for 40)

H. Cost per item: $1.45 + $0 S/H

I. Dimensions:

Appendix A2.5.4.7.6 - BNC adapters to breadboard

A. Sensor - None, but used with pH, EC, DO, and ORP

B. Arduino shield

Figure A2.5.4.7.6.1 - BNC adapters to breadboard

C. Arduino code to download or write

D. Sensor documentation

Heating voltage: 5 ± 0.2V (AC -• DC)
Working current: 5-10mA
The detection concentration range: pH 0-14
The detection range of temperature: 0-80 centigrade
The response time: ≤ 5 s
Stability time: ≤ 60 s
Power consumption: ≤ 0.5W
The working temperature: -10~50 centigrade (the nominal temperature 20 centigrade)
Working humidity: 95% RH (nominal humidity 65% RH)
Service life: 3 years
Size: 42 mm x 32 mm x 20 mm
The output: analog voltage signal output

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Liquid-PH-Value-Detection-detect-Sensor-Module-Monitoring-Control-For-arduino/400926133627

H. Cost per item: $7.99 + $1.85 S/H

I. Dimensions:  42 mm x 32 mm x 20 mm

Appendix A2.5.4.7.7 - Female USB to Breadboard Adapter for Endoscopes

A. Equipment

Figure A2.5.4.7.7.1 - Female USB to Breadboard Adapters for Endoscopic Microscopes

G. Web site for purchase

https://www.ebay.com/itm/10-20-30pcs-USB-2-0-Female-Type-A-Port-4-Pin-DIP-90-Degree-Jack-Socket-Connector/272843081715?var=571992850556

H. Cost per item: 20 for $1.59 + $0 S/H x 2 units

Appendix A2.5.4.7.8 - Solderless Breadboard Simple Type, 1560 Tie Point, 1 Bus Strip

A. Item - Solderless Breadboard Simple Type, 1560 Tie Point, 1 Bus Strip

Figure A2.5.4.7.8.1 - 1560 Tie Point Solderless Breadboard https://i.ebayimg.com/images/g/21QAAOxydB1SlhbA/s-l1600.jpg

G. Web site for purchase

https://www.ebay.com/itm/Solderless-Breadboard-Simple-Type-1560-Tie-Point-1-Bus-Strip/271267336371?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $13.49 + $0 S/H

I. Dimensions:

Appendix A2.5.4.7.9 - Arduino Kit

A) Item:  Adeept Electronic Starter Kit for Arduino Resistor Buzzer Breadboard LED cable

Figure A2.5.4.7.9.1 - Arduino Kit

B) Shield: N/A

C) Code (if any): None

D) Documentation: Package included：

10x Red LED

10x Green LED

10x Yellow LED

10x Blue LED

8x Button (small)

6x Button (large)

2x Button Cap (red)

2x Button Cap (white)

2x Button Cap (blue)

1x Active buzzer

1x Passive buzzer

1x 40 Pin header

1x Band Resistor Card

1x Potentiometer

2x  Light Sensor (Photoresistor)

2x  Analog Temperature Sensor(Thermistor)

1x 400 hole breadboard

1x Hookup Wire Set

2x  1N4148 Diode

2x  1N4001 Diode

10x  Capacitor (104)

10x  Capacitor (10uF)

2x  NPN Transistor (8050)

2x  PNP Transistor (8550)

16x Resistor (220Ω)

10x Resistor (1KΩ)

5x Resistor  (10KΩ )

5x Resistor  (100KΩ)

5x Resistor  (1MΩ)

5x Resistor  (5.1MΩ)

E) Wiring Diagram: None

F) Testing: None

G) Web Site for Purchase:

https://www.ebay.com/itm/Adeept-Electronic-Starter-Kit-for-Arduino-Resistor-Buzzer-Breadboard-LED-cable/232188571392?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H) Cost Per Item (Including S/H): $8.99 ($ 0 S/H)

I) Dimensions: N/A

Appendix A2.5.4.7.10 - Serial to USB Panel Mount Connector

A. Item:  5X USB 3.0 Female Type-A 9 Pin SMT SMD Panel Mount DIP USB Connector

Figure A2.5.4.7.10.1 - Serial to USB Panel Mount Connector

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/5X-USB-3-0-Female-Type-A-9-Pin-SMT-SMD-Panel-Mount-DIP-USB-Connector/191761480260?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: pack of 5 for $2.81

I. Dimensions:

Appendix A2.5.4.7.11 - 12 V, 2 A DC adapters for Arduino Uno

G. Web site for purchase

https://www.ebay.com/itm/Quality-12V-DC-2A-2-amp-24W-POWER-Supply-ADAPTER-Transformer-for-LED-Strip-CCTV/152586667824?var=452005224175

H. Cost per item: 50 for $148 + $0 S/H

Appendix A2.5.4.8 - Electrical Feedthroughs and Enclosures

Appendix A2.5.4.8.1 - Waterproof Cable Gland Connectors

A. Item - 10/20/50/100pcs PG7 To PG21 Waterproof Cable Gland Wire Connector Range 3.5 - 18 mm

Figure A2.5.4.8.1.1 - Waterproof Cable Gland (PG Gland) Connectors

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/10-20-50-100pcs-PG7-To-PG21-Waterproof-Cable-Gland-Wire-Connector-Range-3-5-18mm/172224185690?ssPageName=STRK%3AMEBIDX%3AIT&var=471050540958&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/Waterproof-Connector-Gland-Cable-PG7-9-11-13-5-16-19-21-25-29-36-42-48-63/322367761269?ssPageName=STRK%3AMEBIDX%3AIT&var=511297937542&_trksid=p2057872.m2749.l2649

H. Cost per item: 100 PG-13.5 glands for $35.99 (1st site); 100 PG-7 glands for $17.99 (1st site); 100 PG-19 glands for 49.98 (1st site);

I. Dimensions:  Table A2.5.4.8.1.1 - Cable Gland Standard Dimensions

No.	 D1(mm)  D2(mm)  L1(mm) L2(mm) L3(mm)  L4(mm)  B1(mm) Cable Range (mm)

PG7	 12.0	        8.1	9	5	  15	       14.0	6.4	      3.5-6

PG9	 16.0	      11.0	9	5	  15	       16.0	8.6	        4-8

PG11	 17.9	      13.0	9	5	  18	       18.0	10.7	       5-10

PG13.5 20.2	      15.0	9	5	  20	       19.0	13.3	       6-12

PG16	 22.0	      17.0	11	5	  20	       19.0	14.3	     10-14

PG19	 24.0	      19.0	11	5	  22	       21.0	15.5	     12-15

PG21	 26.9	      21.8	11	6	  24	       22.0	16.7	     13-18

PG24	 29.8	      24.0	13	7	  24	       24.0	19.2	     15-22

PG29	 36.0	      28.0	13	7	  25	       24.0	24.8	     18-25

PG36	 45.7	      39.0	15	8	  32	       27.0	30.7	     22-32

PG42	 54.0	      46.0	15	8	  33	       29.0	35.6	     30-38

PG48	 59.0	      51.0	16	8	  36	       30.0	41.4	     34-44

PG63	 71.0	      63.5	28	9	  45	       44.0	55	     42-54

Appendix A2.5.4.8.2 - Waterproof USB Bulkhead Connector

A. Item - "USB Bulkhead Connector, IP-67, Type A Females *32355 PL"

Figure A2.5.4.8.2.1 - Waterproof USB Bulkhead Connector

https://i.ebayimg.com/images/g/th0AAOSwVoFavQPd/s-l1600.jpg

G. Web site for purchase

https://www.ebay.com/itm/USB-Bulkhead-Connector-IP-67-Type-A-Females-32355-PL/223017087001?hash=item33ecdab019%3Ag%3Ath0AAOSwVoFavQPd&_sacat=0&_nkw=USB+Bulkhead+Connector%2C+IP-67%2C+Type+A+Fe&_from=R40&rt=nc&LH_TitleDesc=0

https://www.ebay.com/itm/USB-Female-Socket-Plug-Panel-Mount-Adapter-USB-3-0-Waterproof-Connector-IP67/113201047236?ssPageName=STRK%3AMEBIDX%3AIT&var=413500490069&_trksid=p2057872.m2749.l2649

H. Cost per item: $11.95 + $0 S/H for 1st site; 41 for $129.15 (USB 2.0 on 2nd site); 36 for $179.64 (USB 3.0 on 2nd site)

Appendix A2.5.4.8.3 - LeMoTech Project Box Enclosure

A. Equipment

Figure A2.5.4.8.3.1 - LeMoTech Project Box Enclosure

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.amazon.com/dp/B075DKNQSM?ref_=pe_2640190_255746880_404_E_DDE_dt_1

H. Cost per item: $15.99 + $0 S/H

I. Dimensions: 7.9"x 6.1" x 3.1" (200 mm x 155 mm x 80 mm)

Appendix A2.5.4.9 - Valves and Aeration Supplies

Appendix A2.5.4.9.1 - Electrically Actuated Shutoff Valve
A. Equipment - VELLEMAN VMA422 ARDUINO WATER VALVE

Figure A2.5.4.9.1.1 - Electrically Actuated Shutoff Valve

G. Web site for purchase

https://www.ebay.com/itm/20pcs-Aquarium-Air-Line-Tubing-Volume-Flow-White-Control-Valves/262329715658?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/VELLEMAN-VMA422-ARDUINO-WATER-VALVE/282999643063?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: 20 for $4.99 + $0 S/H; $19.97 + $0 S/H

I. Dimensions: 4 mm diameter

Appendix A2.5.4.9.2 - Manually Operated Valves Inside Aquarium

A. Equipment

Figure A2.5.4.9.2.1 - Manually operated valves

G. Web site for purchase

https://www.ebay.com/itm/20pcs-Aquarium-Air-Line-Tubing-Volume-Flow-White-Control-Valves/262329715658?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: 20 for $4.99 + $0 S/H

I. Dimensions: 4 mm diameter

Appendix A2.5.4.9.3 - Large Shutoff Valves for Aquarium

A. Item:  Fish Tank Double Tap ABS 12 mm/16 mm Water With Valve Adjustable Aquarium Supplies

Figure A2.5.4.9.3.1 - Manual Shutoff Valve: 12 mm to either 12 mm or 16 mm Hose Barb

B. Arduino shield
C. Arduino code to download or write
D. Sensor documentation
E. Wiring
F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Fish-Tank-Double-Tap-ABS-12mm-16mm-Water-With-Valve-Adjustable-Aquarium-Supplies/273410928184?ssPageName=STRK%3AMEBIDX%3AIT&var=572793328440&_trksid=p2057872.m2749.l2649

H. Cost per item: 10 for $14.90 (12 mm); 10 for $15.50 (12mm to 16mm Adapter)

I. Dimensions:

Appendix A2.5.4.9.4 - Manifolds Inside Aquarium
A. Equipment

4 mm Steel Air Flow Tube Pipe Nozzle Airpump Splitter Aquarium Control Valve (5-Way)

Figure A2.5.4.9.4.1 - Five valve manifolds for air handling

G. Web site for purchase

https://www.ebay.com/itm/4mm-Steel-Air-Flow-Tube-Pipe-Nozzle-Airpump-Splitter-Aquarium-Control-Valve/172740865611?ssPageName=STRK%3AMEBIDX%3AIT&var=471522466247&_trksid=p2057872.m2749.l2649

H. Cost per item: $3.61 + $0 S/H

Appendix A2.5.4.9.5 - Bubble Disk Aerator

A. Equipment

Figure A2.5.4.9.5.1 - Bubble Disk Aerators

G. Web site for purchase

https://www.ebay.com/itm/10X-Air-Bubble-Disk-Stone-Aquarium-Aerator-Fish-Tank-Pump-Hydroponics-Oxygen/252628206544?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: 10 for $8.99 + $0 S/H (4 purchased)

I. 1.3 cm thick x 4 cm diameter; 4 mm connector

Appendix A2.5.4.9.6 - Suction clips for mounting instruments and plumbing
A. Equipment - Suction clips

Figure A2.5.4.9.6.1 - Suction clip for 1/4" tubing

Figure A2.5.4.9.6.2 - Suction clip for 1/4" tubing

G. Web site for purchase

https://www.ebay.com/itm/10-Pcs-Aquarium-Clear-Suction-Cup-Airline-Tube-Holders-Cs-Clamps-I5R9/172979389196?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/Suction-Cup-Clip-for-Aquarium-Air-line-Tubing-1-4-Dia/251590159374?ssPageName=STRK%3AMEBIDX%3AIT&var=550490690213&_trksid=p2057872.m2749.l2649

H. Cost per item: 10 for $0.99 + $0 S/H (50 purchased); 100 pieces for $13.99 x 5

Appendix A2.5.4.10 - Assorted Hose Barb Fittings

A. Items - Assorted Hose Barb Fittings
Plastic Hose Barb Fitting Pipe Tubing Connector Reducer Union Tail Shutoff Plug

Figure A2.5.4.10.1 - 1/4 to 1/2 x 1/4 (6 mm 12 mm 6 mm) Plastic Tee 3-Way Hose Barb Fitting Reducer Adapter

Figure A2.5.4.10.2 - 1/16" to 1/8" (2 mm - 4 mm) Plastic Straight Union Hose Barb Fitting Connector Reducer

Figure A2.5.4.10.3  - 1/4 to 1/2 x 1/4 (6 mm 12 mm 6 mm) Plastic Tee 3-Way Hose Barb Fitting Reducer Adapter

Figure A2.5.4.10.4 - 1/8" to 1/2" (4 mm-12 mm) Plastic Straight Union Hose Barb Fitting Connector Reducer

Figure A2.5.4.10.5 - 1/4" to 3/8" (6 mm-10 mm) Plastic Straight Union Hose Barb Fitting Connector Reducer

Figure A2.5.4.10.6 - 1/4 to 1/8 x 1/4 (6 mm 4 mm 6 mm) Plastic Tee 3-Way Hose Barb Fitting Reducer Adapter

Figure A2.5.4.10.7 - 1/16" to 1/8" (2 mm - 4 mm) Plastic Straight Union Hose Barb Fitting Connector Reducer

Figure A2.5.4.10.8 - 5/16" to 1/2" (8 mm-12 mm) Plastic Elbow Hose Barb Fitting Pipe Reducer Connector

Figure A2.5.4.10.9 - 1/8"-1/16"-1/8" (4 mm - 2 mm - 4 mm) Tee Hose Barb Fitting Pipe Reducer Connector to Tee

Figure A2.5.4.10.10 - Hose Barb Fitting: 1/4"-1/8" Reducing Union

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Plastic-Hose-Barb-Fitting-Pipe-Tubing-Connector-Reducer-Union-Tail-Shutoff-Plug/323464983951?ssPageName=STRK%3AMEBIDX%3AIT&var=512534645563&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/1-16-to-1-8-2mm-4mm-Plastic-Straight-Union-Hose-Barb-Fitting-Connector-Reducer/323483663185?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/1-4-to-1-2-x-1-4-6mm-12mm-6mm-Plastic-Tee-3Way-Hose-Barb-Fitting-Reducer-Adapter/222654252954?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/1-8-to-1-2-4mm-12mm-Plastic-Straight-Union-Hose-Barb-Fitting-Connector-Reducer/223176421232?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/1-4-to-3-8-6mm-10mm-Plastic-Straight-Union-Hose-Barb-Fitting-Connector-Reducer/223176429008?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/1-4-to-1-8-x-1-4-6mm-4mm-6mm-Plastic-Tee-3Way-Hose-Barb-Fitting-Reducer-Adapter/322773539856?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/1-16-to-1-8-2mm-4mm-Plastic-Straight-Union-Hose-Barb-Fitting-Connector-Reducer/323483663185?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/5-16-to-1-2-8mm-12mm-Plastic-Elbow-Hose-Barb-Fitting-Pipe-Reducer-Connector/323483708337?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/1-8-1-16-1-8-4mm-2mm-4mm-Tee-Hose-Barb-Fitting-Pipe-Reducer-Connector-to-T/323483727478?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/Silicone-Tubing-Clear-Food-Pharmaceutical-Grade-Peristaltic-Pump-Hose-All-Sizes/113411717461?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $2.49 + $2.99 S/H + 1.99 + 2.99 S/H + 2.49 + 2.99 S/H + 2.29 + 2.99 S/H + 2.49 + 2.99 S/H + 1.99 + 2.99 S/H + 2.29 + 2.99 S/H + 2.49 + 2.99 S/H + 1.99

I. Dimensions:

A. Item - Assorted Stainless Steel Hose Barb Fittings
6 mm - 12 mm Hose Barb Tail 1/4" 1/2" NPT Female Thread Connector Joint Pipe Fitting

Figure A2.5.4.10.11 - 6 mm-12 mm Hose Barb Tail 1/4" 1/2" NPT Female Thread Connector Joint Pipe Fitting

1/4" Male Thread Pipe Fitting x 6 mm Barb Hose Tail Connector Stainless Steel NPT

Figure A2.5.4.10.12 - 1/4" Male Thread Pipe Fitting x 6 mm Barb Hose Tail Connector Stainless Steel NPT

NPT 1/4" Male Thread Pipe Fitting x 8 mm Barb Hose Tail Connector Stainless Steel

Figure A2.5.4.10.13 - NPT 1/4" Male Thread Pipe Fitting x 8 mm Barb Hose Tail Connector Stainless Steel

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/6mm-12mm-Hose-Barb-Tail-1-4-1-2-NPT-Female-Thread-Connector-Joint-Pipe-Fitting/162708441349?ssPageName=STRK%3AMEBIDX%3AIT&var=461725966530&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/1-4-Male-Thread-Pipe-Fitting-x-6mm-Barb-Hose-Tail-Connector-Stainless-Steel-NPT/271349426522?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/NPT-1-4-Male-Thread-Pipe-Fitting-x-8mm-Barb-Hose-Tail-Connector-Stainless-Steel/142701982986?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $2.43 (6 mm to 1/4" NPT) + 2.51 (8 mm to 1/4" NPT) + 1.39 + 1.05 + ?? S/H

I. Dimensions:

A. Item
 10Pcs 2/3/4 Way Aquarium Air Valves Connector Tube Fish Tank Clear Plastic New

Figure A2.5.4.10.14 - 2/3/4 Way Hose Barb Connectors

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/10Pcs-2-3-4-Way-Aquarium-Air-Valves-Connector-Tube-Fish-Tank-Clear-Plastic-Nw/302979944321?ssPageName=STRK%3AMEBIDX%3AIT&var=601914930460&_trksid=p2057872.m2749.l2649

H. Cost per item: 40 crosses for $8.48; 40 elbows for $3.92; 40 tees for $4.60

I. Dimensions:

A. Item

100 pcs tube connector holder inner Dia 3 mm-6 mm soft tube dosing pump air tube

Figure A2.5.4.10.15 - Hose Barb Connectors For Any Size from 3 mm to 6 mm Tubing

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/100-pcs-tube-connector-holder-inner-Dia-3mm-6mm-soft-tube-dosing-pump-air-tube/161722843677?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: 100 pieces for $9.99

I. Dimensions:

A. Item

Plastic Aquarium Fish Tank 4 to 8 mm Hose Air Tube Connectors 10 pcs White

Figure A2.5.4.10.16 - Hose Barb Connectors: 4 mm to 8 mm Hose Barb Connectors

B. Arduino shield
C. Arduino code to download or write
D. Sensor documentation
E. Wiring
F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/Plastic-Aquarium-Fish-Tank-4-to-8-MM-Hose-Air-Tube-Connectors-10pcs-White/173730482999?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $3.42

I. Dimensions:

A. Item
Solvent Resistant Tube Connector 2 mm to 4 mm Wide Format Printer INK 6x4 4x2 #22

Figure A.2.5.4.10.17 - 2 mm to 4 mm Hose Barb Connector

B. Arduino shield
C. Arduino code to download or write
D. Sensor documentation
E. Wiring
F. Testing with Arduino/computer
G. Web site for purchase

https://www.ebay.com/itm/Solvent-Resistant-Tube-Connector-2mm-to-4mm-Wide-Format-Printer-INK-6x4-4x2-22/182198690917?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: 5 for $9.95 + 2.75

I. Dimensions:

Appendix A2.5.4.11 - Tubing and O-Rings

Appendix A2.5.4.11.1 - Silicone Tubing

A. Silicone Tubing

Figure A2.5.4.11.1.1: Silicone tubing (Silicone Tubing Clear Food Pharmaceautical Grade 2019)

In Figure A2.5.4.11.1.1, the yellow arrow points to a 4.8 mm diameter hose.

B. Arduino shield: N/A

C. Arduino code to download or write: N/A

D. Item Specifications:

Table A2.5.4.11.1.1: Different silicone tubing diameter specification chart (Silicone Tubing Clear Food Pharmaceautical Grade 2019)

E. Wiring

G. Web site for purchase:

https://www.ebay.com/itm/Silicone-Tubing-Clear-Food-Pharmaceutical-Grade-Peristaltic-Pump-Hose-All-Sizes/113411717461?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $28.99 / 5 m

Appendix A2.5.4.11.2 - Tygon Tubing

A. Item
TYGON Tubing,4mm I.D.,49 ft.,Clear,Flexible, ACF1S1518

TYGON Tubing,2mm I.D.,49 ft.,Clear,Flexible, ACF1S1502

Tubing,1/16 I.D.,100 ft.,Clear,Flexible TYGON ACF02002

Tygon Tubing, 4 mm I.D.,49 ft., Clear, Flexible, ACF1S1518

Tubing - 4 mm 1/2/5/10 m Clear Aquarium Fish Tank Tube PVC Hose Pipe Air Pump Air Line

Figure A2.5.4.11.2.1: Tygon tubing (Tygon Tubing 2019)

B. Arduino shield

C. Arduino code to download or write

D. Item documentation

Table A2.5.4.11.2.1: Tygon tubing specifications

| Tygon Tubing Specifications |  |
| --- | --- |
| Inside diameter | 4.00 mm |
| Outside diameter | 6.00 mm |
| Length | 49 ft |
| Bend radius | 16 mm |
| Material | Tygon |
| Max pressure | 1.4 psi |
| Tensile strength | 1750 psi |

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/TYGON-Tubing-4mm-I-D-49-ft-Clear-Flexible-ACF1S1518/253727657982?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/TYGON-Tubing-4mm-I-D-49-ft-Clear-Flexible-ACF1S1518/253727657982?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/TYGON-Tubing-2mm-I-D-49-ft-Clear-Flexible-ACF1S1502/253729236323?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/Tubing-1-16-I-D-100-ft-Clear-Flexible-TYGON-ACF02002/222482420180?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/4mm-1-2-5-10m-Clear-Aquarium-Fish-Tank-Tube-PVC-Hose-Pipe-Air-Pump-Air-Line/162414918156?ssPageName=STRK%3AMEBIDX%3AIT&var=461439258915&_trksid=p2057872.m2749.l2649

H. Cost per item: $34.60; 22.33; 24.06; $4.29 for 10 m + $0 S/H (purchased 50 m)

I. Dimensions: 4 mm dia. x 10 m long x 5 items (last batch)

Appendix A2.5.4.11.3 - O-Rings

A. Items
US Stock QTY 100 19 mm OD 15 mm ID 2 mm Dia Food Grade Silicone Rubber Seal O-Ring

US Stock 100x 19 mm OD 15 mm ID 2 mm Dia Food Grade Silicone Rubber Seal O-Ring Red

O-Rings Nitrile Rubber 12 mm x 17 mm x 2.5 mm Seal Rings Sealing Gasket

O-Ring Seals Buna-N; 19 mm X 24mm X 2.5 mm Width; Sealing Gasket

10 PCS Black Rubber Oil Filter Seal O Rings Gaskets 17 mm x 12 mm x 2.5

US Stock 50 pcs 22 mm OD 19 mm ID 1.5 mm Dia FKM Viton Seal Fluorine Rubber O-Ring

20 pcs Black Rubber Airtight O-Ring Sealing ring Gasket (Line diameter:1.0 mm）

B. Arduino shield
C. Arduino code to download or write
D. Sensor documentation
E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/US-Stock-QTY-100-19mm-OD-15mm-ID-2mm-Dia-Food-Grade-Silicone-Rubber-Seal-O-Ring-/392098660610

https://www.ebay.com/itm/US-Stock-100x-19mm-OD-15mm-ID-2mm-Dia-Food-Grade-Silicone-Rubber-Seal-O-Ring-Red-/392098660214

https://www.ebay.com.au/p/10pcs-Black-Rubber-Oil-Filter-Seal-O-Rings-Gaskets-17mm-X-12mm-X-2-5mm/1240517578

https://www.ebay.com/i/401579596601?chn=ps

https://www.ebay.com/itm/20pcs-Black-Rubber-Airtight-O-Ring-Sealing-ring-Gasket-Line-diameter-1-0mm-/263724711549

H. Cost per item: 2 for $17.40; 2 for $16.16; 4 + 9 for $7.96 and 17.55; 4 for $21.48; 10 for $23.80; 2 for $18.64; 10 for $35.20

I. Dimensions:

## Appendix A2.7 - Piping & Instrumentation Diagrams

## Appendix A2.7.3 - Feed Distribution System

Figure A2.7.3.1: P&ID example of feed distribution.  Stream 123 flows from the feedstocks to 10-way valve V-103 for distribution to bioreactors 1-6 via streams 01-01 – 01-06. Stream 124 daisy chains to 10 way valve V-104 (not pictured) for further distribution.

## Appendix A2.7.4 - Waste Collection System

Figure A2.7.4.1: P&ID example of waste collection.  Streams 19-01 – 19-06 send waste from bioreactors 1-6 to 10-way valve V-114 for common return to the concentration analyzer A-01 via stream 234.

Figure A2.7.4.2: P&ID example of return distribution. Stream 438 flows from the output of the purification system and returns fluid for distribution. 10 way valve V-116 distributes to bioreactors 1-6 via stream 20-01 – 20-06. Stream 439 daisy chains to the following 10 way valve V-117 (not pictured) for further return distribution.

A.4 Full Tissue Engineering Test Bed

Figure A2.7.4.3: P&ID of entire tissue engineering test bed

Appendix 2.8 – Hazard and Operability Analysis (HAZOP)

Figure A2.8.1 – Example of a more thorough hazard and operability analysis

Appendix A2.9 - LabView VI's

Appendix A2.9.1 - LabView VI’s from Prior Brenner Projects

- Acquisitionloop

- When I tried to load this there were some VI’s missing

- Loop to display temperature readings of thermocouple type using a nested while loop. Also, writes the date/time of reading and the temperature to a file “file.vi”

- Includes: SinglepointTC.vi

- Antoinecoeff

- Antoineeqn

- Avgstdev

- Bas2Soln

- Bedarea

- Bedcalcs

- Bigarray

- Boiling Theatofvapor

- Buildingandbackuppower

- Catalytic Reactor test in Storage Bed 2

- Catalytic reactor test

- Changeinenthalpysubi

- Changeinmolarenthalpysubi

- Channel string to name array2

- Channelstringtonamearray

- Close all fuelcell Valves

- Close all Storagebed Valves

- Closeallredormervalves

- CloselBA

- Clustertoarray

- Complete Shutdown

- Configure Acquicistion2

- Configureacquisition3

- Continuoustcmeasurement

- Continuousthermocouplemeasurement

- Controlregion

- Conv TC range to volt range

- Conv TC tange to volt range 2

- Conv thermo temp range to volt range 2

- Cpcalc

- Cpcalc2009

- Create demo setpoint 4

- Daqfreq

- Dateandtime

- Defaultranges

- defaultTCrange

- Deleteoldestvalue2

- Desorp Bed 1 Vent

- Desorp Bed 2 Vent

- Evac of H2 Storage System

- Evac of Storage Bed 1

- Evac of Storage Bed 2

- Evacuation of H2 Storage System

- First order system 4

- Fuel Cell N2 Purge

- Fuel cell Test

- Fuelcellcurrent

- Fumehoodflowrate

- H2 to SotorageBed1 Cylinder

- H2 to Storage Bed2 Clinder

- H2 to Storage Bed2 Cylinder

- H2 to StorageBed1 Cylinder

- H2frontpanel--

- Heatcapacitycoefficients

- Heatcapacityofcoefficient2009

- Heaterpower

- Heatervoltageoff

- Heatofformation

- Ibaheat

- IBAtubing

- Mainprogram

- Massandenergyfuelcell

- Mmddyyyy

- Normalized PID output 4

- Numderiv

- Numericalderivative

- Numericalderivative2

- Numericaldr

- Numericaldr2

- Numericaldr3

- Pascaltriangle2

- Pid demo integrator 4

- PIDcontrol

- Rangeinitialize

- Runlognotes

- Runlognotes2

- Scope (library)

- Second order system 4

- Sethialarms

- Sethihialarms

- Setlowalarms

- Setlowalarms2

- Setlowlowalarms

- Setlowlowalarms2

- Simple PID 4

- Simple PID with normalized output 4

- Singlepointcurrent

- SinglepointTC

- Singlepointvoltage

- Standard Deviation2

- Storage 1 to Storage 2

- Storage 2 to Storage 1

- Storage Bed 1 to Storage bed 2

- Storage Bed 2 to Storage Bed 1

- System type 4

- Tcpremote

- Trunc

- Usernamepasswordsplitter

- Valve Controljrb

- Valve Controljrb2

- Voltacquire

- Voltscale

- Voltscale07312000

- Write Temperature to File

- Write Temperature to File2

Appendix 2.9.2 -  VIs for Tissue Engineering Test Bed

Acquisitionloop (Main)

·         gets the date and time, selects thermocouple type, set of temperatures and writes 	them to a file

Antoinecoeff

·         template for inputting Antoine coefficients

Antoineeqn

·         calls antionecoeff program and calculates vapor pressures for a given 	temperature

Avgstdev

·         calculates averages and standard deviations

Bas2Soln

·         has set of answers for LabView Basics 2 course

·         contains File Reader.VI

Bedarea

·         dimensions for penetrations into an H2 purifier bed to go into heat transfer 	calculations

Bedcalcs

·         an accumulator of information for calculating heat transfer losses

Bigarray

·         top: settings for all sensors’ alarms for high alarms, high high alarms, low alarms, low low alarms

·         bottom: various sensor readings

·         for safety and troubleshooting

BoilingTheatofvapor

·         allows you to call boiling points and heat of vaporization data for different 	molecules

Buildingandbackuppower

·         designed to be called if building power gets shut off

·         uninterruptable power supply (UPS)

Catalytic Reactor test in Storage Bed 2

·         changes position of valves and sequences through in a particular order

Catalytic Reactor test

·         conceptually the same as Catalytic Reactor Test in Storage Bed 2

Changeinenthalpysubi

·         energy balance calculations

Changeinmolarenthalpysubi

·         gets called by changeinenthalpysubi

Channel string to name array 2

·         lists various unit ops

clustertoarray

·         bundling alarms into one mega-array

Complete Shutdown

·         everything that is necessary to get to a safe condition; shut-down program

Configure Acquisition2

·         takes alarm settings and bundles them all together

Configureaqusition3

·         takes alarm settings and bundles them all together

Controlregion

·         writing high alarms to a file; part of master shut down program

Cpcalc

·         heat capacity calculations

Daqfreq

·         determine how often data should be recorded

Desorp Bed 1 Vent

·         valve sequences

Desorp Bed 2 Vent

·         valve sequences

Fuelcellcurrent

·         valve sequences

H2frontpanel

·         master program; calls other programs

Sethialarms

·         sensor readings

Sethihialarms

·         sensor readings

Setlowalarms1

·         sensor readings

Setlowalarms2

·         sensor readings

Setlowlowalarms

·         sensor readings

Setlowlowalarms2

·         sensor readings

Appendix 2.11 - Equipment List & Budget

INSERT link to Component Cost datasheet.xlsx

Appendix A3.1.1 - Instrumentation for 10-Way Valve Automation

A. Item:  Lot of 4 Valco VICI EMHMA-CE Microelectric Valve Actuator with Cheminert 	10 port

Figure A3.1.1.1: VICI: 10-way valves (Valco VICI Microelectric Valve Actuator 2019)

B. Arduino code to download or write: N/A

C. Arduino code to download or write: NA

D. Item Specifications:

Table A3.1.1.1: Item Specifications for VICI 10-Way Valve

| VICI 10-Way Specifications |  |
| --- | --- |
| Pressure rating | 5000 psi |
| Fittings | - 1/16" Valco ZDV fittings - 6-40 fittings for 1/16" tubing - 1/4-28 fittings - For 1/16" tubing - For 1/8" tubing - 1/2-20 fittings |
| Power supply | 24 V |
| Manufacturer Part Number (MPN) | EMHMA-CE 04V-0206H |

E. Wiring:

G. Website for Purchase:

https://www.ebay.com/itm/Lot-of-4-Valco-VICI-EMHMA-CE-Microelectric-Valve-Actuator-W-CHEMINERT-10-Port/273778250061?hash=item3fbe74914d:g:zYsAAOSwImRYdsCJ&frcectupt=true

H. Cost per item: 18 for $450

## Appendix A3.1.2 - National Instruments SC-2054

A. NI SC-2054

Figure A3.1.2.1: NI SC-2054 (National Instruments SC-2054 Module 2019)

B. Arduino code to download or write: N/A

C. Arduino code to download or write: N/A

D. Item Specifications:

Figure A3.1.2.2: SC-2054 Parts Locator Diagram (DAQ SC-205X Series User Manual 1998)

Figure A3.1.2.3: Pin Assignments for SC-2054 I/O Connectors J1 and J2

(DAQ SC-205X Series User Manual 1998)

Figure A3.1.2.4: Pin assignments for SC-2054 I/O connectors J4 and J8

(DAQ SC-205X Series User Manual 1998)

Figure A3.1.2.5: Pin assignments for SC-2054 I/O connectors J6 and J10

(DAQ SC-205X Series User Manual 1998)

Table A3.1.2.1: Item specifications of NI SC-2054

| NI SC-2054 Specifications |  |
| --- | --- |
| Weight | 2 lbs |
| Ports | 4 x 50 pin digital I/O connection ports + 6 x 26 pin digital I/O connection ports |
| UNSPSC code | 23260000 |

E. Wiring: See Figures A3.1.2.2 - A3.1.2.5 just above.

G. Website for Purchase:

https://www.ebay.com/itm/National-Instruments-SC-2054-Module-Used/332695822243?epid=1955618879&hash=item4d76375fa3:g:JbEAAOSwlHJbLa~T

H. Cost per item: $24.95

Appendix A3.1.3 - National Instruments R1005050 Ribbon Cable 182762-02

A. National Instruments R1005050 Ribbon Cable 182762-02, 2 m

National Instruments R1005050 Ribbon Cable 182762-0R5

National Instruments R1005050 Ribbon Cable 182762B-01 100 Pin 1 m NI DAQ

Figure A3.1.3.1: 100 pin ribbon cable (National Instruments R1005050 Ribbon Cable 2018)

B. Arduino shield: N/A

C. Arduino code to download or write: N/A

D. Item Specifications:

Table A3.1.3.1: 100 pin ribbon cable specifications

| 100 Pin Ribbon Cable Specifications |  |
| --- | --- |
| Manufacturer Part Number (MPN) | R1005050 |
| Connection capabilities | Y-shaped ribbon cable that connects to the 100-pin connector on PC-DIO-96 digital I/O device and is terminated with two 50-pin IDC connectors |
| Length | 2 meters |

E. Wiring

G. Web site for purchase

https://www.ebay.com/itm/National-Instruments-R1005050-Ribbon-Cable-182762-02-2M/253685193690?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/National-Instruments-R1005050-Ribbon-Cable-182762-0R5/163582274009?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/National-Instruments-R1005050-Ribbon-Cable-182762B-01-100-Pin-1m-NI-DAQ/123573828338?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $65.99 + 49.00 + 34.50

I. Dimensions:

Appendix A3.1.4 - 26 Wire Rainbow Ribbon Cable

A. Item: 4 ft 26 wire F/F connector IDC rainbow ribbon cable

Figure A3.1.4.1 - 26 Wire Rainbow Ribbon Cable

(26 Pin 26 Way F/F Connector IDC Flat Rainbow Ribbon Cable 2019)

B. Arduino shield: N/A

C. Arduino code to download or write: N/A

D. Item Specifications:

Table A3.1.4.1: Item specifications for 26 pin ribbon cable

| 26 Pin Ribbon Cable Specifications |  |
| --- | --- |
| Connection capabilities | Female/female connection / 26 pin 26 way |
| Length | 4 feet |
| Width | 1.3 inches |
| Weight | 34 grams |

In Figure A3.1.4.2, the wires are numbered 1-20. Starting at the left most wire, brown is wire 1, to the right most wire, black is wire 20, the colors labeled correspond to wire numbers 1-20 depending on their chronological position. This ribbon cable pictured in Figure A3.1.4.2 is a 20-pin ribbon cable. Though the 26-pin ribbon cable is used for all diagnostic testing, the wires used are wires 10-18 (Table A3.1.4.1), and the 6 wires not depicted in Figure A3.1.4.2 are not used for automation of the VICI 10-way valve.

Figure A3.1.4.2: Schematic of 20-pin ribbon cable

(Multiposition Electric Actuator Instruction Manual 2000)

E. Wiring: See above Section D

G. Website for Purchase:

https://www.ebay.com/itm/1-4ft-26-Pin-26-Way-F-F-Connector-IDC-Flat-Rainbow-Ribbon-Cable/312535453305?epid=1567752965&hash=item48c4908e79:g:YVQAAOSwqctcloAx

H. Cost per item: $4.38

Appendix A3.1.5 - National Instruments PCI-DIO-96 96 signal digital input/output card

A. Item: National Instruments PCI-DIO-96 96 signal digital input/output card

Figure A3.1.5.1: PCI-DIO-96 (Full Tested PCI-DIO-96 2019)

B. Arduino shield: N/A

C. Arduino code to download or write: N/A

D. Item Specifications:

Table A3.1.5.1: Item specifications for PCI-DIO-96

| PCI-DIO-96 Specifications |  |
| --- | --- |
| Connection capabilities | Connects to R1005050100 pin ribbon cable |
| Manufacturer Part Number (MPN) | HFKP053B |
| PCI card voltage | - V / 5 V |

E. Wiring:

F. Website for Purchase

https://www.ebay.com/itm/Full-Tested-PCI-DIO-96-60-Day-Warranty-vpu/323653381446?hash=item4b5b3ead46:g:NyQAAOSw8PJcQF~J

G. Cost per item: $67.07

Appendix A3.2.1 - Valco VICI HPLC 8 Position Valve Controller

A. Items 
VICI Valco Instruments Co E4 Model 120VAC Air Actuator with Digital Controller

VICI Valco Instruments Co. Inc. Model E4

VICI Valco Instruments Cheminert HPLC 8 Pos Valve System EMHMA-CE 090-0331L

VICI VALCO DIGITAL VALVE SEQUENCE PROGRAMMER DVSP4 & MODEL E-16 VALVE ASSEMBLY

Figure A3.2.1.1 - Valco VICI HPLC Multiposition Valve Controller

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/VICI-Valco-Instruments-Co-E4-Model-120VAC-Air-Actuator-with-Digital-Controller/263815621896?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/VICI-Valco-Instruments-Co-inc-Model-E4/162938038236?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/VICI-Valco-Instruments-Cheminert-HPLC-8-Pos-Valve-System-EMHMA-CE-090-0331L/282233896924?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/VICI-VALCO-DIGITAL-VALVE-SEQUENCE-PROGRAMMER-DVSP4-MODEL-E-16-VALVE-ASSEMBLY/401732167948?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $75.18 + 64.95 + 179 + 18 S/H

I. Dimensions:

Appendix A3.2.2 - Stainless Steel Ferrules for Multiposition Valves

A. Items
VICI/Valco 1/16" Ferrule 303 Stainless Steel ZF1-10 (Pack of 10)

VICI/Valco 1/16" Ferrule 303 Stainless Steel ZN1-10 (Pack of 10)

B. Arduino shield

C. Arduino code to download or write

D. Sensor documentation

E. Wiring

F. Testing with Arduino/computer

G. Web site for purchase

https://www.ebay.com/itm/VICI-Valco-1-16-Ferrule-303-Stainless-Steel-ZF1-10-Pack-of-10/323736589759?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

https://www.ebay.com/itm/VICI-Valco-1-16-Internal-Nut-303-Stainless-Steel-ZN1-10-pack-of-10/323736591511?ssPageName=STRK%3AMEBIDX%3AIT&_trksid=p2057872.m2749.l2649

H. Cost per item: $24.45 + 29.75

I. Dimensions:

Appendix A4.1 - Absorbance vs. Concentration

Appendix A4.1.1 - Sample Calculations

Molar absorptivity

|  | ε= αdyeL | (A4.1.1.1) |
| --- | --- | --- |

For uncoated dye trial 2, from linear regression data in Figure A4.1.2.1 in Appendix A4.1.2, αdye = 0.9952

path length of cuvette L = 1 cm

|  | ε= 0.99521 = 0.9952 Lmoldye * cm |  |
| --- | --- | --- |

Concentration

|  | C=Aε*L | (A4.1.1.2) |
| --- | --- | --- |

In uncoated dye trial 2, at time t = 7 s, absorbance A = 0.154

molar absorptivity ε = 0.9952 Lmoldye*s

path length of cuvette L  = 1 cm

|  | C=0.1540.9952 *1 = 0.155 moldyeL |  |
| --- | --- | --- |

Concentration Function

The concentration curve function Ct is developed from fitting two curves to the data collected. Two separate polynomial functions are generated to fit the upward sloping curve from start to the maximum and, likewise, the downward sloping curve to finish. For uncoated dye trial 2, regression yields the two explicit equations below. C1 is from bounds 0 s to 10 s, C2 is on bounds between 10 s to completion at 300 s. The plot is shown in Figure A4.1.2.2 in Appendix A4.1.2.

Ct  = C1+C2

C1=-(1.22*10-3)*t2 + (1.12*10-2)*t + (7.64*10-3)

C2=-8.13*10-9*t3 + 6.96*10-6*t2- 2.12*10-3*t-0.230

Mass in Effluent

|  | m= 0∞Q*Ctdt | (A4.1.1.3) |
| --- | --- | --- |

volumetric flowrate

Q=0.0106 Ls

Since Ct=0 s at t=300 s, integration will be bounded from 0 to 300 s.

integration yields:

m= 0.01060330Ctdt

where integrating  Ctdt from 0 to 300 yields 18.47 moldyeLs

Solving yields

m= 0.01060330Ctdt=0.0106*18.47=0.1958 moldye

Percent Error in Mass in Effluent

|  | % Error= m0-mm0*100% | (A4.1.1.4) |
| --- | --- | --- |

m0 = mass initial

m= mass final

Error= 0.200 mol- 0.1954 mol0.200 mol*100%=2.05%

Partial Derivative Error Analysis

Equation 4 can be rewritten as equation A4.1.1.5 below

|  | m= Q*Ct | (A4.1.1.5) |
| --- | --- | --- |

where mass of the tracer in effluent m and volumetric flowrate Q

integration of the concentration per time function, area under Ct=Area= Ct

the error is mass is given in equation A4.1.1.6:

|  | dm=∂m∂Q*dQ+∂m∂Ct*dCt | (A4.1.1.6) |
| --- | --- | --- |

which equates to equation A4.1.1.7:

|  | dm=C*dQ+¦Q*dCt¦ | (A4.1.1.7) |
| --- | --- | --- |

Dividing equation A4.1.1.7 by N yields the fractional error in mass can be defined as the following:

|  | ∂m∂m=C*dQQ*Ct+C*dQQ*Ct | (A4.1.1.8) |
| --- | --- | --- |

which reduces to equation A4.1.1.9:

|  | dmm=dQQ+dCtCt | (A4.1.1.9) |
| --- | --- | --- |

where error in flowrate dQ=2.83*10-4 Ls

flowrate Q=0.0106 Ls

area under Ct=Area=18.47 moldyeL

error in the concentration function, dCt is calculated as follows:

The concentration function Ct for any given data point can be written in terms of the trapezoidal rule equation A4.1.1.10 below, where a trapezoid can be calculated between any two pairs of data points. The error in each trapezoid can be calculated and the total error in all the trapezoids summed will equal the total area of the trapezoids, which equals that total error of the area under the Ct curve.

|  | C=(t1-t0)*C1+C02 | (A4.1.1.10) |
| --- | --- | --- |

t0 = value of time measured at some data point

t1= value of time measured the next data point sequentially following t0

C0 = the value of concentration measured at some data point

C1= value of time measured the next data point sequentially following C0

The error in the integration of concentration function dC is defined in equation A4.1.1.11:

|  | dCt=∂Ct∂t1*dt1+∂Ct∂t0*dt0+∂Ct∂C1*dC1+∂Ct∂C0*dC0 | (A4.1.1.11) |
| --- | --- | --- |

Equation A4.1.1.11 simplifies to

|  | dCt==C1+C02*(dt1+dt0)+dt1+dt02*(dC1+dC0) | (A4.1.1.12) |
| --- | --- | --- |

where the error in concentration measurement is defined in equation A4.1.1.13:

|  | dC0=dC1=dAα | (A4.1.1.13) |
| --- | --- | --- |

dA = the error in absorbance measurement = 0.003

αdye = 0.9952

dC0=dC1=0.0030.9952=3.0*10-3moldyeL

Error in time measurement: dt= dt1= dt0= 0.5 s

For uncoated dye trial 2, the values below are used to solve for the error in the first trapezoid:

t0 = 0 s

t1= 3 s

C0 = 0 moldyeL

C1= 0.020 moldyeL

Solving equation A.9 yields

dCt=0.020+02*(0.5+0.5)+0.5+0.52*(0.020+0)=0.020 moldye*sL

Summing the combined dC for trapezoid in a trial yields the total dC of that trial shown in equation A4.1.1.14 below.

|  | i=0300dCti=dCt | (A4.1.1.14) |
| --- | --- | --- |

Equating for each trapezoid from 0 to 300 seconds yields

i=0300dCti=3.434 moldye*sL= dCt

Plugging in the value for dCt into equation A4.1.1.6, the fractional error in mass can be defined as the following:

|  | dmm=dQQ+dCtCt | (A4.1.1.15) |
| --- | --- | --- |

where error in flowrate dQ=2.83*10-4 Ls

flowrate Q=0.0106 Ls

area under Ct=Area= C=18.47 moldye*sL

and error in concentration function, dCt =3.434 moldye*sL

Solving yields:

dmm=2.83*10-40.0106+3.43418.47 =0.213

Multiplying by 100% yields a percent error in the closure of the mass balance:

dmm*100%=21.3%

Residence Time Distribution

|  | Et=C(t)0∞Ctdt | (4.1.1.16) |
| --- | --- | --- |

where Ct  for uncoated dye trial 2 is described in equation A.6

and 0∞Ctdt = 18.43 moldye*sL

Solving yields:

|  | Et=C(t)18.43 |  |
| --- | --- | --- |

The residence time distribution curve for uncoated dye trial 2 is shown in Figure A4.1.2.4 in Appendix A4.1.2.

Mean residence time

|  | tm=0∞t*Etdt | (4.1.1.17) |
| --- | --- | --- |

Since Ct=0 s at t=300 s, integration will be bounded from 0 to 300 s.

Solving yields:

|  | tm=0300t* C(t)18.47 = 76.25 seconds |  |
| --- | --- | --- |

Variance

|  | σ2=0∞(t-tm)2*Etdt | (4.1.1.18) |
| --- | --- | --- |

Given the mean residence time of tm = 76.25 seconds:

|  | σ2=0∞(t-76.25)2*C(t)18.47dt = 3484.06 seconds2 |  |
| --- | --- | --- |

Cumulative Distribution Curve

|  | 0tEtdt=F(t) | (4.1.1.19) |
| --- | --- | --- |

The residence time distribution function, E(t) is given as:

Et=C(t)18.47

0300C(t)18.47dt=F(t)

The cumulative distribution function curve for uncoated dye trial 2 is shown in Figure A4.1.2.6 in Appendix A4.1.2.

Peclet Number

|  | σ2tm2= 2Pe - 2Pe2*(1-e-Pe) | (4.1.1.20) |
| --- | --- | --- |

For uncoated dye trial 2,

mean residence time, tm = 76.25 s

variance, σ2 =  3884.06 s2

rearranging and solving yields

|  | 0 = 2Pe - 2Pe2*1-e-Pe- 3884.0676.25 2 | Pe=1.777 |
| --- | --- | --- |

Dispersion

|  | 1Pe=dispersion | (4.1.1.21) |
| --- | --- | --- |

For uncoated dye trial 2,

where Pe = 1.777

Appendix A4.1.2 - Example of Reactor Analysis Study Results

Table A4.1.2.1: Calibration curve data for trial 2 uncoated dye

| Concentration moldyeL | Absorbance |
| --- | --- |
| 0 | 0.001 |
| 0 | 0.000 |
| 0 | 0.000 |
| 0.15 | 0.130 |
| 0.15 | 0.140 |
| 0.15 | 0.147 |
| 0.20 | 0.193 |
| 0.20 | 0.199 |
| 0.20 | 0.203 |
| 0.25 | 0.240 |
| 0.25 | 0.242 |
| 0.25 | 0.248 |
| 0.33 | 0.326 |
| 0.33 | 0.334 |
| 0.33 | 0.330 |
| 0.50 | 0.508 |
| 0.50 | 0.497 |
| 0.50 | 0.503 |

Figure A4.1.2.1: Calibration curve for uncoated dye trials 1 and 2 at 380 nm

Table A4.1.2.2: Concentration data for trial 2 uncoated dye

| Time (s) | Concentration (moldyeL) | Absorbance |
| --- | --- | --- |
| 0 | 0.000 | 0.000 |
| 3 | 0.042 | 0.049 |
| 5 | 0.092 | 0.109 |
| 7 | 0.145 | 0.171 |
| 9 | 0.211 | 0.249 |
| 15 | 0.221 | 0.260 |
| 20 | 0.211 | 0.249 |
| 25 | 0.207 | 0.244 |
| 30 | 0.195 | 0.230 |
| 35 | 0.193 | 0.228 |
| 40 | 0.170 | 0.200 |
| 45 | 0.165 | 0.194 |
| 50 | 0.159 | 0.188 |
| 55 | 0.138 | 0.163 |
| 60 | 0.129 | 0.152 |
| 70 | 0.127 | 0.150 |
| 80 | 0.104 | 0.123 |
| 90 | 0.098 | 0.115 |
| 120 | 0.081 | 0.096 |
| 150 | 0.065 | 0.077 |
| 180 | 0.040 | 0.047 |
| 210 | 0.024 | 0.028 |
| 240 | 0.015 | 0.018 |
| 270 | 0.006 | 0.007 |
| 300 | 0.000 | 0.000 |

Figure A4.1.2.2: Ct for uncoated dye trial 2

Figure A4.1.2.3: Polymath report for uncoated dye trial 2 solving for Area

Figure A4.1.2.4: Residence time distribution, Et, for uncoated dye trial 2

Figure A4.1.2.5: Polymath report for uncoated dye trial 2, solving full reactor analysis

Figure A4.1.2.6: Cumulative distribution curve, Ft, for uncoated dye trial 2

Appendix A4.1.3 - Reactor Analysis Data

Appendix A4.1.3.1 - Uncoated Tracer Dye

Table A4.1.3.1.1: Results for uncoated tracer dye studies, trials 1-5

| Trial # | m0 | m | % random error | % error from partial derivative calculus | tm | σ2 | Dispersion |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.200 mol | 0.2070 mol | 3.50% | 20.29% | 81.85 s | 3818 s2 | 0.504 |
| 2 | 0.200 mol | 0.1959 mol | 2.05% | 21.30% | 76.25 s | 3484 s2 | 0.563 |
| 3 | 0.200 mol | 0.2141 mol | 7.05% | 20.30% | 78.95 s | 4194 s2 | 0.754 |
| 4 | 0.200 mol | 0.2159 mol | 10.70% | 20.07% | 78.91 s | 3984 s2 | 0.659 |
| 5 | 0.200 mol | 0.2306 mol | 15.30% | 18.95% | 80.50 s | 3895 s2 | 0.567 |
| Avg. | 0.200 mol | 0.2127 mol | 7.72% | 20.18% | 79.29s | 3875 s2 | 0.609 |

Figure A4.1.3.1.1: Polymath report for uncoated dye trial 1, solving full reactor analysis

Figure A4.1.3.1.2: Polymath report for uncoated dye trial 2, solving full reactor analysis

Figure A4.1.3.1.3: Polymath report for uncoated dye trial 3, solving full reactor analysis

Figure A4.1.3.1.4: Polymath report for uncoated dye trial 4, solving full reactor analysis

Figure A4.1.3.1.5: Polymath report for uncoated dye trial 5, solving full reactor analysis

Appendix A4.1.3.2 - Uncoated Glucose Tracer Results

Table A4.1.3.2.1: Results for uncoated tracer glucose studies, trials 1-5

| Trial # | m0 | m | % random error | % error from partial derivative calculus | tm | σ2 | Dispersion |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2000 mg/dl | 2190 mg/dl | 9.50% | 21.60% | 72.01 s | 3767 s2 | 0.957 |
| 2 | 2000 mg/dl | 1896 mg/dl | 5.20% | 21.28% | 75.97 s | 3599 s2 | 0.618 |
| 3 | 2000 mg/dl | 1903 mg/dl | 4.85% | 21.70% | 74.75 s | 3742 s2 | 0.745 |
| 4 | 2000 mg/dl | 2055 mg/dl | 2.75% | 19.34% | 78.65 s | 3963 s2 | 0.666 |
| 5 | 2000 mg/dl | 1831 mg/dl | 8.45% | 20.94% | 67.97 s | 2981 s2 | 0.674 |
| Avg. | 2000 mg/dl | 1975 mg/dl | 6.15% | 20.97% | 73.87 s | 3611 s2 | 0.732 |

Figure A4.1.3.2.1: Polymath report for uncoated glucose trial 1, solving full reactor analysis

Figure A4.1.3.2.2: Polymath report for uncoated glucose trial 2, solving full reactor analysis

Figure A4.1.3.2.3: Polymath report for uncoated glucose trial 3, solving full reactor analysis

Figure A4.1.3.2.4: Polymath report for uncoated glucose trial 4, solving full reactor analysis

Figure A4.1.3.2.5: Polymath report for uncoated glucose trial 5, solving full reactor analysis

Appendix A4.1.3.3 - Reactor Analysis: Coated Tracer Dye and Glucose

Table A4.1.3.3.1: Results for coated tracer dye studies, trials 1-5

| Trial # | m0 | m | % random error | % error from partial derivative calculus | tm | σ2 | Dispersion |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.200 mol | 0.2202 mol | 10.10% | 20.21% | 80.38 s | 3644 s2 | 0.493 |
| 2 | 0.200 mol | 0.2112 mol | 5.60% | 20.46% | 82.91 s | 4454 s2 | 0.680 |
| 3 | 0.200 mol | 0.2076 mol | 3.80% | 20.51% | 73.20 s | 2914 s2 | 0.458 |
| 4 | 0.200 mol | 0.1998 mol | 0.10% | 20.24% | 77.55 s | 3465 s2 | 0.515 |
| 5 | 0.200 mol | 0.2246 mol | 12.30% | 18.95% | 86.76 s | 4269 s2 | 0.499 |
| Avg. | 0.200 mol | 0.2127 mol | 6.38% | 20.07% | 80.16 s | 3749 s2 | 0.529 |

Figure A4.1.3.3.1: Polymath report for coated tracer dye trial 1, solving full reactor analysis

Figure A4.1.3.3.2: Polymath report for coated tracer dye trial 2, solving full reactor analysis

Figure A4.1.3.3.3: Polymath report for coated tracer dye trial 3, solving full reactor analysis

Figure A4.1.3.3.4: Polymath report for coated tracer dye trial 4, solving full reactor analysis

Figure A4.1.3.3.5: Polymath report for coated tracer dye trial 5, solving full reactor analysis

Appendix A4.1.3.4 - Coated Glucose

Table A4.1.3.4.1: Results for coated tracer glucose studies, trials 1-5

| Trial # | m0 | m | % random error | % error from partial derivative calculus | tm | σ2 | Dispersion |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2000 mg/dl | 2126 mg/dl | 6.30% | 20.79% | 78.17 s | 3768 s2 | 0.601 |
| 2 | 2000 mg/dl | 2106 mg/dl | 5.30% | 20.04% | 80.65 s | 3690 s2 | 0.499 |
| 3 | 2000 mg/dl | 1956 mg/dl | 2.20% | 20.90% | 82.57 s | 4449 s2 | 0.693 |
| 4 | 2000 mg/dl | 2035 mg/dl | 1.75% | 21.45% | 71.90 s | 3336 s2 | 0.673 |
| 5 | 2000 mg/dl | 2153 mg/dl | 7.65% | 20.86% | 74.77 s | 3539 s2 | 0.641 |
| Avg. | 2000 mg/dl | 2075 mg/dl | 4.64% | 20.81% | 77.66 s | 3758 s2 | 0.621 |

Figure A4.1.3.4.1: Polymath report for coated tracer glucose trial 1, solving full reactor analysis

Figure A4.1.3.4.2: Polymath report for coated tracer glucose trial 2, solving full reactor analysis

Figure A4.1.3.4.3: Polymath report for coated tracer glucose trial 3, solving full reactor analysis

Figure A4.1.3.4.4: Polymath report for coated tracer glucose trial 4, solving full reactor analysis

Figure A4.1.3.4.5: Polymath report for coated tracer glucose trial 5, solving full reactor analysis

http://journals.plos.org/plosone/article?id=10.1371/journal.pone.0174068#pone.0174068.ref004